3GPP TSG-RAN WG1 #89	R1-1708693

Hangzhou, China, 15th – 19th May, 2017

Source:	Ericsson

Title:	On scalable codebook for up to 32 ports

Agenda Item:	7.1.2.3.6

Document for:	Discussion and Decision

# Introduction

In RAN1#87, the following agreement and working assumptions were made:

Agreements:

- In NR, a UE can be configured with a CSI-RS resource configuration with X ports

  - Supported values of X are up to at least 32

- NR supports up to at least 32 port codebook

  - FFS: Codebook design

- Study the potential benefits of 64 ports

Working assumptions:

- For Type I,  CSI feedback using a PMI codebook for X CSI-RS ports is supported

- Supported values of X are at least 1,2,4,8,12,16,[24],32

  - Note: For X=1, Type I does not have PMI feedback

- Support for other values of X is not precluded

- Note the number of ports in CSI-RS resource configuration may not be the same with the number of ports in the PMI codebook

In this contribution, we discuss the need for supporting 24 ports in Type I CSI feedback.

# Discussion

## Performance comparison of different number of antenna ports

System simulations were done to compare the performance with 16, 24, and 32 ports under 3D UMa.  For a given number of antenna ports, different port layouts were also investigated. 16 ports results were used as the baseline and the performance gains over the baseline are shown in Figure 1, where NxM means N row and M column of antenna ports. The same shape of port layout is compared for different number of ports, e.g. 1D 1x8 port layout for 16 ports is used as the baseline for 1x12 for 24 ports and 1x16 for 32 ports.  Other simulation assumptions can be found in the Appendix. It can be seen that the average mean and cell edge throughput gain for 32 ports are 24% and 66% over 16 ports, while for 24 ports, the average mean and cell edge throughput gain are 16% and 40% respectively.

24 ports provide average 16% mean and 40% cell edge throughput gains over 16 ports.

Whether 24 or 32 ports antenna is deployed is a deployment choise based on factors such as size, cost, and performance, but clearly 24 ports provides good performance gains over 16 ports and can be a good compromise between 16 and 32 ports.  It is desirable to be supported in the codebook designs for 24 ports.

Figure 1: Comparison of SU-MIMO performance with 16, 24, and 32 ports.

## Performance of 24 ports with and without a 24 ports codebook

We next compare the performance of 24 ports deployed with and without a 24-port codebook. In the case of without a 24 codebook, two approaches summarized below would be used:

- Approach 1:  Non-precoded CSI-RS (i.e., Class A) using a mismatched codebook where the number of ports in the codebook is larger than the number of ports that are actually deployed.  With this type of mismatched codebook, the gNB transmits CSI-RS on the used ports (i.e., the number of ports actually deployed) and does not transmit CSI-RS on the unused ports.  The UE measure the channel on all ports (including used and unused ports).  As shown in Figure 2(a), for a 4×6 array with 2x1 virtualization, the UE is configured with a mismatched codebook corresponding to a 32-port 2×8 layout.  For a 12×2 array with 2×1 virtualization, the UE is configured with a mismatched codebook corresponding to a 32-port 8x2 layout as shown in Figure 2(b).

- Approach 2:  Applying precoded CSI-RSs such that each precoded CSI-RS is beamformed in different directions in the horizontal or vertical domain (i.e., Class B).  As shown in Figure 3(a), for a 4×6 array with 2×1 virtualization, we place K = 6 beams horizontally with 4 CSI-RS ports per beam.  For a 12×2 array with 2×1 virtualization, we place K = 6 beams vertically with 4 CSI-RS ports per beam as shown in Figure 3(b).

It should be noted that Approach 2 has the same amount of CSI-RS overhead when compared to the 24-port codebook (i.e., 24 ports will be required in both cases).  However, with Approach 1, the UE measures the channel on all CSI-RS ports including the used and unused ports.  Hence, Approach 1 will have a higher CSI-RS overhead when compared to the 24-port codebook.  The overhead associated with Approach 1 corresponds to the overhead associated with 32 port CSI-RS.

In Tables 1-2, the performance of the two approaches that do not rely on the 24-port codebook is compared to the performance of the 24-port codebook.  From the results in these tables, it can be clearly seen that Approach 2 of using precoded CSI-RS (Class B) suffers a performance loss due to the reduced quality of the channel estimate resulting from the constraint that the channel estimate must lie within one of the subspaces spanned by the set of precoded CSI-RSs.  Furthermore, Approach 1 of using a mismatched 32-port codebook also suffers performance losses due to unnecessary interference/noise leakage in the unused CSI-RS ports.  This is because the UE measures interference/noise at the unused ports which can affect the PMI/CQI estimation depending on the severity of interference/noise leakage.

Figure 2: Simulated cases for Approach 1 with 24-port layouts. For Case (a) a 2×6 port layout is mismatched with a 32-port 2×8 layout codebook.  For Case (b) a 6×2 port layout is mismatched with a 32-port 8×2 layout codebook.

| (a) | (b) |
| --- | --- |

Figure 3: Simulated cases for Approach 2 with 24-port layouts. For Class B operation there are (a) 6 DFT beams in the horizontal dimension and (b) 6 DFT beams in the vertical dimension, both with 4 ports/beam.

The approach of utilizing precoded CSI-RS for 24 ports has a performance loss compared to the non-precoded CSI-RS for 24 ports with a 24 port codebook for all the simulated cases.

The approach of utilizing a 32 ports codebook  for an actually deployed antenna array of 24 ports also suffers performance losses.

Table 1: Comparison with Approaches without 24 Port Codebook for 3D UMa 2D Wide Array

| Baseline RU | 50% |  |  | 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline (Class A – 24-port 2×6 Codebook) | Approach 1 (Class A – Figure 1a) | Approach 2 (Class B – Figure 2a) | Baseline (Class A – 24-port 2×6 Codebook) | Approach 1 (Class A – Figure 1a) | Approach 2 (Class B – Figure 2a) |
| Mean User Throughput Gain | 0% | -5% | -11% | 0% | -13% | -26% |
| Cell-Edge User Throughput Gain | 0% | -8% | -22% | 0% | -22% | -38% |

Table 2: Comparison with Approaches without 24 Port Codebook for 3D UMa 2D Tall Array

| Baseline RU | 50% |  |  | 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline (Class A – 24-port 6×2 Codebook) | Approach 1 (Class A – Figure 1b) | Approach 2 (Class B – Figure 2b) | Baseline (Class A – 24-port 6×2 Codebook) | Approach 1 (Class A – Figure 1b) | Approach 2 (Class B – Figure 2b) |
| Mean User Throughput Gain | 0% | -5% | -3% | 0% | -13% | -6% |
| Cell-Edge User Throughput Gain | 0% | -8% | -6% | 0% | -23% | -9% |

Based on the observations in this section, we make the following proposals:

Confirm the working assumption on the supported number of CSI-RS ports for Type I CSI feedback and remove the brackets around the value 24.

The number of CSI-RS ports supported for Type I CSI feedback includes at least 1, 2, 4, 8, 12, 16, 24, 32.

# Conclusions

In this contribution we made the following observations:

Observation 1	24 ports provide average 16% mean and 40% cell edge throughput gains over 16 ports.

Observation 2	The approach of utilizing precoded CSI-RS for 24 ports has a performance loss compared to the non-precoded CSI-RS for 24 ports with a 24 port codebook for all the simulated cases.

Observation 3	The approach of utilizing a 32 ports codebook  for an actually deployed antenna array of 24 ports also suffers performance losses.

Based on the discussion in this contribution we propose the following:

Proposal 1	Confirm the working assumption on the supported number of CSI-RS ports for Type I CSI feedback and remove the brackets around the value 24.

Proposal 2	The number of CSI-RS ports supported for Type I CSI feedback includes at least 1, 2, 4, 8, 12, 16, 24, 32.

# References

Chairman’s note, RAN1#87.

# Appendix: Simulation Assumptions

| Carrier frequency | 2 GHz |
| --- | --- |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMa |
| Antenna Configurations | (M, N): 16 ports: 4x4, 8x2, 2x8, 16x1, 24 ports: 4x6, 12x2, 2x12, 6x4, 24x1 32 ports: 4x8, 16x2, 2x16, 8x4, 32x1 2x1 virtualization, 3D UMa (122° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE Rx antenna | Two cross polarized isotropic antennas |
| UE noise figure | 9 dB |
| eNB Tx power | 46dBm (3D UMa) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for Channel estimation error modeled. |
| Class A Codebook | LTE Rel-14 class A, config 1, O1=O2=4 |
| Class B | 6 DFT beams in one dimension and Rel-12 4 port codebook in the other dimension |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |