3GPP TSG RAN WG1 Meeting #89                                            R1-1708751

Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:	6.1.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on modulation scheme selection for 256QAM

Document for:	Discussion/Decision

# Introduction

In Rel-13, 256QAM was specified for PDSCH. As a result of its specification, a new MCS table was introduced in 36.213. In this contribution, we present some observations on the performance of different modulation schemes under different scenarios, from which we conclude that the current mechanism to select the modulation scheme (static for a given MCS value) may not lead to optimum performance in a variety of scenarios. We also find that this problem exists not only for the cases of 256QAM, but also for some legacy MCS/TBS values.

# Observations on performance with different modulation schemes

In Figure 1 we show the SNR vs BLER for 64QAM and 256QAM in the following scenario:

- TM4

- 2T2R

- EPA5L

- TBS 63776 (MCS 20 for 256QAM and MCS 27 for 64QAM)

- 2 CRS ports

And the following differences:

- Left Figure, CFI=1

- Right Figure: CFI=3

Essentially, the main difference between these two scenarios is that the first one operates at a higher coding rate regime than the second due to the smaller number of resource elements.

We observe that, under these settings, for the CFI=1 case 64QAM outperforms 256QAM, but the opposite behaviour is observed in the CFI=3.

Figure 1 BLER for CFI=1 (left) and CFI=3 (right)

Similar observations can be made on the legacy TBS table depending on the number of available REs. In Figure 2 we calculate the code rate for a CFI=3 case with 32CSI-RS tones per RB, and we can observe that, in many cases, it leads to data rates beyond 0.931.

Figure 2 Code rate calculation for 64-QAM TBS table. The entries in red are above 0.931

In general, the problem is given by the fact that the association between MCS/TBS and modulation scheme is static, i.e., for a given MCS, the UE uses the modulation scheme in the MCS table. This selection is made regardless of the number of available resource elements for data rate matching.

Observation 1: The static association between MCS/TBS and modulation scheme leads to suboptimal modulation selection in some cases.

# Potential solutions

From the evidence presented in the previous section, it can be seen that the modulation scheme selection has to selected depending the number of available resources for optimum performance. Thus, depending on a variety of configurations, such as number of control symbols, number of REs used by reference signals, CP length, etc., the optimum modulation scheme may be different.

In this section, we provide two possible solutions to this problem.

Solution 1: Modulation determination based on rate matching

Since the optimum modulation scheme depends on the TBS size and number of available REs, one possible solution would be to determine the modulation scheme based on these two values. The number of available REs should be calculated according to the actual rate matching (which is known by eNB and UE). A pseudocode of the procedure would be as follows:

- From the DCI, obtain the resource allocation (#RBs) and MCS

- From the #RBs and MCS, determine the TBS using the current table

- Calculate the number of available REs for data transmission

  - This is the number of symbols that the UE uses for rate matching.

- Calculate the spectral efficiency as SE=TBS/(#REs)

- From the SE and a set of thresholds (T16QAM, T64QAM, T256QAM) obtain the modulation scheme

One example of the last step would be:

if SE<1.1768 bits/RE

modulation is QPSK

else if SE<2.5684 bits/RE

modulation is 16QAM

else if SE<5.3349 bits/RE

modulation is 64QAM

else, modulation is 256QAM

Solution 2: Modulation determination based on explicit signalling

A similar problem regarding modulation scheme selection was found in Rel-14 FeMTC and Rel-14 Voice/video enhancements. In these cases, the background was that for the case of repetitions, the optimum modulation scheme may be different than the one for single transmission. For PDSCH in FeMTC, the solution was to always use QPSK for the case of R>1 (which is similar to our Solution 1). For PUSCH in FeMTC and voice/video, a 1-bit indication in DCI was introduced to “override” the modulation scheme. The agreement in voice/video enhancements is as follows:

- When operating in the new PUSCH enhancement mode, include an additional 1-bit field in the uplink DCI in USS to override the modulation scheme as follows:

  - If the modulation order is Qm=2 , then the field is reserved

  - If the modulation is greater than 2 and the bit is set to be true, then override the modulation order with Qm-2(i.e., 16-QAM QPSK, 64-QAM16-QAM, 256-QAM64-QAM)

In our case the solution is a bit more complicated, since for some scenarios the optimum modulation scheme would be higher than the “default” modulation scheme (e.g. move QPSK to 16QAM), and in some other cases it may be lower (e.g. move 256QAM to 64QAM). Thus, we propose a slight modification of this method that allows to increase or decrease the modulation scheme depending on the MCS as follows:

- In the MCS table, add one additional column that indicates whether the “modulation overriding” indicates “increase” or “decrease”.

- The bit in the DCI is interpreted as follows:

  - If bit =0, keep the modulation scheme in the MCS table

  - If bit=1, increase or decrease the modulation scheme according to the entry in the MCS table.

An example of step 1) is shown in Table 1, where the 256QAM table is modified to include this new indicator. Note that the adjustment is set to “increase” right before a “default modulation order” increase, and to “decrease” right after.

This second solution has the disadvantage of higher overhead (1 additional bit in DCI), but offers the advantage of network flexibility, e.g. the eNB can pick the modulation scheme depending on other factors such as observed delay spread (which may also affect the optimum modulation scheme) or presence of signals that puncture the PDSCH (of which the UE may not be aware of).

Table 1: Example of MCS/TBS table with modulation adjustment

| MCS Index | Modulation Order | Modulation Order adjustment | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | Increase | 0 |
| 1 | 2 | Increase | 2 |
| 2 | 2 | Increase | 4 |
| 3 | 2 | Increase | 6 |
| 4 | 2 | Increase | 8 |
| 5 | 4 | Decrease | 10 |
| 6 | 4 | Decrease | 11 |
| 7 | 4 | Decrease | 12 |
| 8 | 4 | Increase | 13 |
| 9 | 4 | Increase | 14 |
| 10 | 4 | Increase | 15 |
| 11 | 6 | Decrease | 16 |
| 12 | 6 | Decrease | 17 |
| 13 | 6 | Decrease | 18 |
| 14 | 6 | Decrease | 19 |
| 15 | 6 | Increase | 20 |
| 16 | 6 | Increase | 21 |
| 17 | 6 | Increase | 22 |
| 18 | 6 | Increase | 23 |
| 19 | 6 | Increase | 24 |
| 20 | 8 | Decrease | 25 |
| 21 | 8 | Decrease | 27 |
| 22 | 8 | Decrease | 28 |
| 23 | 8 | Decrease | 29 |
| 24 | 8 | Decrease | 30 |
| 25 | 8 | Decrease | 31 |
| 26 | 8 | Decrease | 32 |
| 27 | 8 | Decrease | 33/33A |
| 28 | 2 | N/A | reserved |
| 29 | 4 | N/A |  |
| 30 | 6 | N/A |  |
| 31 | 8 | N/A |  |

Observation 2: The following two solutions can be considered for modulation scheme selection:

- Solution 1: Select modulation scheme based on TBS and number of REs used for rate matching.

- Solution 2: Introduce “Modulation overriding” field in DCI to change the modulation scheme.

Proposal: Adopt Solution 2 (Modulation overriding” field in DCI) for modulation scheme selection.

# Summary of proposals

Observation 1: The static association between MCS/TBS and modulation scheme leads to suboptimal modulation selection in some cases.

Observation 2: The following two solutions can be considered for modulation scheme selection:

- Solution 1: Select modulation scheme based on TBS and number of REs used for rate matching

- Solution 2: Introduce “Modulation overriding” field in DCI to change the modulation scheme.

Proposal: Adopt Solution 2 (Modulation overriding” field in DCI) for modulation scheme selection.