3GPP TSG RAN WG1 Meeting #89                                     R1-1708831

Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:		7.1.4.2.1.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Performance evaluation of polar code constructions

Document for:		Discussion and Decision

# 1 	Introduction

In RAN1 #NR Ad-Hoc meeting, following agreement was made to provide further evaluations of control channel coding proposals.

Agreement:

- To compare CRC-related aspects of polar code design,

  - The same FAR performance (the same as LTE) should be considered for a fair comparison

  - List size Lmax 8 is the baseline (evaluations of other values are not precluded)

- Performance metrics (may be based on analytic derivation)

  - BLER

  - FAR (with AWGN as input to the decoder)

- Polar codes for control channels support one of the following alternatives:

  - Alt. 1: CRC + “basic polar” (i.e. as per above agreed description) codes

    - 1a: Longer CRC

      - e.g.	(J + J’) bits CRC + basic polar

    - 1b: J bit CRC

      - The J bits can be distributed

    - The CRC can be used for both error detection and error correction

  - Alt. 2: J bits CRC + concatenated polar codes

    - e.g.	 	 J bits CRC + J’ bits CRC + basic polar;

J bits CRC + J’ bits distributed CRC + basic polar;

J bits CRC + PC bits + basic polar; (i.e. PC-Polar)

J bits CRC + Hash sequence + basic polar;

…

    - J bits CRC is only used for error detection

The main purpose of the evaluations was to see BLER and FAR of different proposals. There were different proposals and evaluations for polar code constructions, and Ran1 #88bis made the following conclusion.

Agreement:

- J CRC bits are provided (which may be used for error detection and may also be used to assist decoding and potentially for early termination)

  - J may be different in DL and UL

  - J may depend on the payload size in the UL (0 not precluded)

- In addition, J’ assistance bits are provided in reliable locations (which may be used to assist decoding and potentially for early termination)

- J + J’ <= the number of bits required to satisfy the FAR target (nFAR) + 6

  - Working assumption:

    - For DL, nFAR = 16 (at least for eMBB-related DCI)

    - For UL, nFAR = 8 or 16 (at least for eMBB-related UCI; note that this applies for UL cases with CRC)

- J’>0

- Working assumption: J”<=2 additional assistance bits are provided in unreliable locations (which may be used to assist decoding and potentially for early termination)

  - Can be revisited in RAN1#89 if significant benefit is shown from a larger value of J” without undue complexity – companies are encouraged to additionally evaluate J”=8

- The J’ (and J” if any) bits may be CRC and/or PC and/or hash bits (downscope if possible)

- Placement of the J, J’ (and J” if any) assistance bits is FFS after the study of early termination techniques

  - Appended?

  - Distributed?

    - evenly?

    - unevenly?

In this contribution, we compare three different code constructions, which are CA-Polar where CRC attached at the end of the info block, distribute-CRC Polar code where CRC bits are distributed according to the pattern mentioned in [1], and PC-Polar described in [2].

# 2 	Performance

## 2.1 	Simulation parameters

Simulation parameters are provided in Table I. These parameters were first agreed during Ran1 #86 to evaluate control channel coding candidates.

Table I: Simulation parameters

| Channel | AWGN |  |  |
| --- | --- | --- | --- |
| Modulation | QPSK |  |  |
| Coding Scheme | PC-Polar [2] | CA-Polar | Distributed-CRC Polar [1] |
| Code rate | 1/12, 1/6, 1/3, 1/2, 2/3 |  |  |
| Decoding algorithm | Parity check aided List-8 | CRC aided List-8 (CRC used after decoding the full info block to select the path) | X CRC bits are used for tree pruning, X = 4. |
| Info. block length (bits w/o CRC) | 16, 32, 48, 64, 80, 120, 200 |  |  |
| CRC bits (detection + correction) | 16 | 19 | 19 |

Please note that when X = 0, which means CRC bits are not used for tree pruning. CRC bits can be used after full info is decoded. The performance will be similar to CA-Polar scenario. Distributed CRC polar provides the flexibility of using CRC bits in tree pruning.

Code construction procedure (reliability ordering, shortening) is assumed as in [2]. Coded block (M) is calculated such a way that Coded block size = info block size/code rate, where CRC bits cause an increase in the effective code rate. Selected parameters are further explained in Annex I. CRC polynomial used for 19 CRC case is [1 1 0 1 0 0 0 1 1 1 0 1 0 1 1 1 1 0 0 1]. LTE 16 bits CRC polynomial is used for PC-Polar simulations.

## 2.2 	BLER performance comparison

In this section, we provide the simulation results of BLER for the agreed block size and rate combinations.

Case 1: 16 info. bits

Figure 1: BLER vs. Es/No for 16 info. bits

Case 2: 32 info. bits

Figure 2: BLER vs. Es/No for 32 info. bits

Case 3: 48 info. bits

Figure 3: BLER vs. Es/No for 48 info. bits

Case 4: 64 info. bits

Figure 4: BLER vs. Es/No for 64 info. bits

Case 5: 80 info. bits

Figure 5: BLER vs. Es/No for 80 info. bits

Case 6: 120 info. bits

Figure 6: BLER vs. Es/No for 120 info. bits

Case 7: 200 info. bits

Figure 7: BLER vs. Es/No for 200 info. bits

Observation 1: Distributed CRC-polar outperforms CA-Polar and PC-polar for all block sizes and code rates.

Observation 2: Gains are significant for payload sizes less than 100 bits. In practice, almost all the time DCI and UCI payload sizes will be less than 100 bits.

It is understandable that FAR of distributed CRC will be much lower than CA-Polar and PC-Polar when they are operating in this operating mode. Distributed CRC polar will have different FAR depending on the number of CRC bits used for pruning. However, as we mentioned above it will provide the same BLER and FAR when we set X = 0. Therefore, it is quite flexible to work on different modes. For example, when UE only uses aggregation level 1 and a given DCI format for decoding, FAR equivalent to 16 bit CRC may not be needed. 16 bit CRC is decided on control channels assuming the worst case of all blind decodes. Having the flexibility of using CRC bits provide good BLER performance for the system.

Observation 3: Distributed CRC polar has the capability of adjusting the FAR according to the requirement.

Based on these evaluations and the benefits of early termination we highlighted in [3], it is evident that distributed CRC polar proposed in [1] is the best polar code construction scheme and should be adopted in eMBB control channels.

Proposal 1: Distributed CRC polar code construction should be used in the control channels.

# 3	Conclusion

In this contribution, we compare the performance of CA-Polar, PC-Polar, and Distributed CRC polar, and we have the following observations and proposal.

Observation 1: Distributed CRC-polar outperforms CA-Polar and PC-polar for all block sizes and code rates.

Observation 2: Gains are significant for payload sizes less than 100 bits. In practice, almost all the time DCI and UCI payload sizes will be less than 100 bits.

Observation 3: Distributed CRC polar has the capability of adjusting the FAR according to the requirement.

Proposal 1: Distributed CRC polar code construction should be used in the control channels.

# References

- R1-1708833, “Design details of distributed CRC”, Nokia, Alcatel-Lucent Shanghai Bell

- R1-1700088, “Summary of polar code design for control channels,”, Huawei, HiSilicon

- R1-1708832, “Early termination benefits of CRC distribution”, Nokia, Alcatel-Lucent Shanghai Bell

# Annex I

Table II: Number of encoded bits for different info bits (without CRC) and code rates.

|  | 16 | 32 | 48 | 64 | 80 | 120 | 200 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1/12 | 192 | 384 | 576 | 768 | 960 | 1440 | 2400 |
| 1/6 | 96 | 192 | 288 | 384 | 480 | 720 | 1200 |
| 1/3 | 48 | 96 | 144 | 192 | 240 | 360 | 600 |
| 1/2 | 32 | 64 | 96 | 128 | 160 | 240 | 400 |
| 2/3 | 24 | 48 | 72 | 96 | 120 | 180 | 300 |

Table III: Mother polar code sizes

|  | 16 | 32 | 48 | 64 | 80 | 120 | 200 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1/12 | 256 | 512 | 1024 | 1024 | 1024 | 2048 | 2048 |
| 1/6 | 128 | 256 | 512 | 512 | 512 | 1024 | 2048 |
| 1/3 | 64 | 128 | 256 | 256 | 256 | 512 | 1024 |
| 1/2 | 64 | 64 | 128 | 128 | 256 | 256 | 512 |
| 2/3 | 32 | 64 | 128 | 128 | 128 | 256 | 512 |

Red color points exceed code rate one, and does not include in the simulation results.