3GPP TSG RAN WG1 Meeting #89		R1-1708976

Hangzhou, P.R. China 15th – 19th May 2017

Source:	WILUS Inc.

Title:	On short PUCCH structure for NR

Agenda item:	7.1.3.2.1

Document for:	Discussion/Decision

- Introduction

At the previous RAN1#88bis meeting, there have been several agreements on PUCCH with short duration as follows [1]:

Agreements at RAN1 #88bis: - At least for 1 symbol short-PUCCH with more than 2 bits, the following is supported. - RS and UCI are multiplexed in FDM manner in the OFDM symbol where RS and UCI are mapped on different subcarriers and coherent demodulation are supported. - FFS: Details on RS - FFS: whether to support option 6 (pre-DFT) - FFS: for 1 and 2 bits Agreements at RAN1 #88bis: - For 1-symbol PUCCH without SR with 1 or 2 bit(s) UCI payload size, RAN1 will select one from the following options. - Option 1: RS and UCI are multiplexed by FDM manner in the OFDM symbol - UCI can be sequence - FFS: low PAPR design is applied - Option 4: Sequence selection with low PAPR - FFS following cases: - If SR only - If with SR + other UCI; - This does not imply the necessity of special SR design - FFS whether the design may or may not depend on the frequency range Agreements at RAN1 #88bis: - For 1-symbol short PUCCH with > 2 UCI bits, the following is supported for the agreed Option 1: - QPSK for UCI - X1 to X2 PRBs can be configured to support various UCI payload sizes - Both localized (contiguous) and distributed (non-contiguous) allocations are supported - FFS: detailed PRB allocations and signaling of the configuration - FFS: values of X1, X2 - DMRS overhead: down-select among the following options: - Option 1: one value (e.g., 1/2, 1/3, 1/4, 1/5, …) - Option 2: multiple values depending on, e.g. UCI payload size etc. Agreements at RAN1 #88bis: - For 2-symbol NR-PUCCH, following options are considered (including possible down-selection) - Option 1: 2-symbol NR-PUCCH is composed of two 1-symbol NR-PUCCHs conveying the same UCI. - 1-1: Same UCI is repeated across the symbols using repetition of a 1-symbol NR-PUCCH. - 1-2: UCI is encoded and the encoded UCI bits are distributed across the symbols. - Option 2: 2-symbol NR-PUCCH is composed of two symbols conveying different UCIs. - E.g., time-sensitive UCI (e.g., HARQ-ACK) is in the second symbol. Agreements at RAN1 #88bis: - Two NR-PUCCHs can be transmitted from one UE on the same slot in TDM manner. - The two NR-PUCCHs can be short-PUCCH. - The two NR-PUCCHs can be long-PUCCH + short-PUCCH. - FFS whether or not to have the two NR-PUCCHs as long-PUCCH + long-PUCCH - FFS: other multiplexing scheme(s) between the two NR-PUCCHs - FFS the case of more than 2 NR-PUCCHs in one slot from a UE (if more than 2, only short-PUCCHs)

In this contribution, we discuss PUCCH structure in a short duration and provide our view regarding PUCCH with short duration.

Discussion on PUCCH structure with short duration

For PUCCH in 1-symbol duration with more than 2 bits, we agreed FDM between RS and UCI at the previous RAN1 #88bis meeting:

Agreements at RAN1 #88bis:

- At least for 1 symbol short-PUCCH with more than 2 bits, the following is supported.

  - RS and UCI are multiplexed in FDM manner in the OFDM symbol where RS and UCI are mapped on different subcarriers and coherent demodulation are supported.

    - FFS: Details on RS

    - FFS: whether to support option 6 (pre-DFT)

    - FFS: for 1 and 2 bits

However, we have not concluded yet which option would be supported in RAN1 between option-1 (FDM based) and option-4 (sequence based w/o RS) for 1-symbol PUCCH with 1 or 2 bit(s) UCI payload size. In this section, we compare PUCCH options in terms of some aspects such as PAPR/CM, UE multiplexing capability for HARQ-ACK/SR, frequency diversity, PUCCH DMRS flexibility, commonality between 1 and 2-symbol PUCCH and applicability with payload size. In Table 1, the comparison among PUCCH options for 1-symbol PUCCH and 2-symbol PUCCH structure is addressed as follows:

Table 1. Comparison among options for 1-or 2-symbol PUCCH structures

From the perspective of the PUCCH PAPR/CM aspects, UE multiplexing capability on each UCI type, and PUCCH cell coverage, it would be proper to consider Option 4 for accommodating 1 or 2 bits UCI payload size. In addition to FDM between RS and UCI for the large payload size, therefore, we prefer to consider sequence-based UCI transmission without RS (option 4) for small payload size in terms of cell coverage, UE multiplexing capacity for HARQ-ACK/SR and commonality between 1 and 2-symbol PUCCH structure.

- Proposal: we prefer to consider sequence-based UCI transmission without RS (option 4) for small payload size in terms of some aspects such as cell coverage, UE multiplexing capacity for HARQ-ACK/SR and commonality between 1 and 2-symbol PUCCH structure.

- Conclusion

In this contribution, we have compared several options for PUCCH structure with 1-symbol and 2-symbol in a short duration and provide our view as follows:

- Proposal: we prefer to consider sequence-based UCI transmission without RS (option 4) for small payload size in terms of some aspects such as cell coverage, UE multiplexing capacity for HARQ-ACK/SR and commonality between 1 and 2-symbol PUCCH structure.

References

[1] Draft Report of 3GPP TSG RAN WG1 #88bis v0.1.0