3GPP TSG RAN WG1 Meeting #89	       	R1-1708982

Hangzhou, China, 15 - 19 May 2017

Source: 	Ericsson

Title:	On resource reselection for P2X UEs

Agenda Item:	6.1.3

Document for:	Discussion and Decision

# Introduction

An LS with the following information has been sent by RAN2 to RAN1 [1]:

During RAN2#97bis meeting, RAN WG2 has discussed the following RAN1 agreement which has RAN2 impacts: For P-UEs performing random resource selection, - Resource is reselected for every TB if only random selection is allowed in the TX pool. - Resource is reselected following the semi-persistent resource reservation procedure if partial sensing is allowed in the TX pool.. RAN2 has some concerns about the above technical solution which allows P-UEs using random selection to perform semi-persistent resource reservation, which is deterministic rather than random. Particularly, this solution may cause a problem that the resources selected by a P-UE collide with those selected by another P-UE in the same pool persistently, regardless of whether partial sensing or random selection is used by another P-UE. In this respect, RAN2 would like to ask RAN1 to answer the following question: - What is the potential motivation or benefit of having a new resource selection mode for P-UEs performing random selection when sharing the same transmission pool with P-UEs performing partial sensing?

In this contribution, we present our views on this issue. A draft LS reply is included in [2].

# Discussion

RAN2 is concerned that the second behavior may lead to persistent collisions between P-UEs and other P-UEs. We agree with the observation made by RAN2 and believe that the agreed behavior is not satisfactory:

- If the pool for P-UE transmissions with random selection is allocated exclusive resources, then resources should be reselected for every TB. In this case, collisions between P-UEs are the only problem and the use of semi-persistent reservation only degrades performance.

- If the pool for P-UE transmissions with random selection overlaps with other pools, then it may be beneficial to use semi-persistent reservation so that at least other UEs can avoid collision by means of sensing.

Note, however, that the P-UE may not be aware of the configuration of other pools. Thus, to implement the desired behavior it is necessary to introduce new signaling.

Proposal:

- The existing behavior for P-UEs is changed to:

  - (Pre)configuration indicates whether resource reservation shall be used or not by P-UEs if only random selection is allowed in the TX pool.

  - (Pre)configuration indicates whether resource reservation shall be used or not by UEs if partial sensing is allowed in the TX pool.

We have included a draft LS reply in [2].

# Conclusion

In this contribution, we have discussed an issue related to resource reselection for P-UEs. We have proposed the following:

Proposal:

- The existing behavior for P-UEs is changed to:

  - (Pre)configuration indicates whether resource reservation shall be used or not by P-UEs if only random selection is allowed in the TX pool.

  - (Pre)configuration indicates whether resource reservation shall be used or not by UEs if partial sensing is allowed in the TX pool.

# Reference

- R1-1706866 (R2-1703942), “LS on resource reselection for P2X UEs”, RAN WG2

- R1-1708990 “[Draft] Response LS on resource reselection for P2X UEs”