3GPP TSG RAN WG1 Meeting #89                                     R1-1709189 Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:		7.1.4.1.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Performance of LDPC design for eMBB

Document for:		Discussion and Decision

# 1	Introduction

In RAN1 #88bis meeting, new agreements on LDPC parameters were made and BLER performance is identified as the main criteria for code selection by the June Ran1 #Ad-Hoc meeting. In Ran1 #NR ad-hoc meeting, Ran1 agreed evaluations are required according to the following agreement,

Conclusion:

- Evaluations at BLER of a single code block = 1e-2 (for performance comparison between codes) and 1e-4 (for the purpose of comparing the error floor performance of the codes)

  - (Note that this does not preclude other comparison criteria)

More specific evaluation parameters were agreed in the RAN1 #88 meeting as below.

Conclusion:

- Minimum set of information block sizes granularity for evaluation at BLER 1e-2 and 1e-4:

| Kmin≤K≤512Kmin<=K<=512 | 528<=K<=1024528≤K≤1024 | 1056<=K<=20481056≤K≤2048 | 2048<=K<=6144 | 6144<=K<=8192 |
| --- | --- | --- | --- | --- |
| 8 | 16 | 32 | 64 | 128 |

- Some off-grid values of K shall also be evaluated.

- Minimum information block size for evaluation = 40

In this contribution, we provide simulation results of the LDPC codes that are proposed in [1].

# 2	Performance

2.1			Simulation assumptions

We use simulation parameters showed in Table 1 to provide results for BLER versus SNR for the proposed codes in [1].

Table 1: Simulation parameters

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Coding Scheme | LDPC |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |
| Decoding algorithm | Sum-Product, flooding floating point with 50 iteration |
| Info. block length (bits w/o CRC) | 40~8448 |

2.2		 	Performance at short block sizes

Figure 1&2 shows the required SNR for BLER = 10-2 and BLER = 10-4.The modulation method is QPSK, and the channel is AWGN. A similar performance is to be expected for 64 QAM as it does not change the properties of codes. Modulo lifting is used to get different code blocks with required padding/puncturing and repetition when required. These parameters used for simulation are provided in [2]. For smaller block sizes, the highest code rate of 2/3 is considered.

Figure 1: Required SNR for different code rates with short block size for BLER = 10-2

Figure 2: Required SNR for different code rates with short block size for BLER = 10-4

Observation 1: The proposed short block PCM provide good performance over a wider range of code rates, and IR HARQ is also supported.

2.3		 	Performance at larger block sizes

Figure 3&4 shows the required SNR for BLER = 10-2 and BLER = 10-4. Modulo lifting is used for different block sizes while puncturing happens on systematic bits and from the end of the parity bits whenever required.

Figure 3: Required SNR for different code rates with large block size for BLER = 10-2

Figure 4: Required SNR for different code rates with large block size for BLER = 10-4

Observation 2: The proposed large block PCM provide good performance over a wider range of code rates and provides good implementation benefits.

# 3	Conclusion

In this contribution, we provide simulation results of the LDPC codes that are proposed in [1]. Observations are,

Observation 1: The proposed short block PCM provide good performance over a wider range of code rates, and IR HARQ is also supported.

Observation 2: The proposed large block PCM provide good performance over a wider range of code rates and provides good implementation benefits.

# References

- R1-1708829, “LDPC design for eMBB”, Nokia, Alcatel-Lucent Shanghai Bell

- R1-1708828, “Rate matching for eMBB” Nokia, Alcatel-Lucent Shanghai Bell