3GPP TSG RAN WG1 Meeting#89	R1-1709199

Hangzhou, China, 15th - 19th May 2017

Agenda item:	7.1.2.5

Source:	LG Electronics

Title:	Phase 2 and Phase 3 SLS Calibration Results for NR MIMO

Document for:	Discussion and Information

Introduction

Through e-mail discussion [NRAH1-02-Ph2] and [NRAH1-02-Ph3], it was further discussed clarification on simulation assumptions for phase 2 and 3 calibration campaign. In this contribution, we provide our phase 2 and 3 system level calibration results for NR MIMO.

Phase 2 Calibration

We herein provide our initial simulation results based on simulation assumptions summarized in Table A-1. In the evaluation, we consider Dense Urban Macro (4GHz), Indoor hotspot with 12 sites (30GHz), Urban Macro (30GHz) and Dense Urban Micro (30GHz). Also, we select the analog beam pair maximizing RSRP. For SVD precoding, we employ averaged instantaneous wideband channel and we assume the round-robin scheduler. Also, we consider outdoor-to-incar penetration loss which is 9 dB (LN, σ = 5 dB).

Figure 1. CDF of UE spectral efficiency

In Figures 1, the CDF of UE spectral efficiency (bps/Hz/UE) with SU-MIMO and rank-1 transmission is presented. Also, we summarize the average SE per TRP, 5% and 50% UE SE in Table 2 with various scenarios.  In Table 2, the outage probability can be expressed as

Outage probability= Number of user with zero throughputTotal number of user .

Table 2: Spectral efficiency with various calibration scenarios

|  | Mean SE (bps/Hz/TRP) | 5% UE SE (bps/Hz/UE) | 50% UE SE (bps/Hz/UE) | Outage Probability |
| --- | --- | --- | --- | --- |
| Dense Urban Macro (4GHz) | 2.804 | 0.147 | 0.268 | 0% |
| Urban Macro (30GHz) | 1.4528 | 0 | 0.132 | 19% |
| Indoor Hotspot 12Site (30GHz) | 3.078 | 0.18 | 0.287 | 0% |
| Dense Urban Micro (30GHz) | 2.357 | 0.155 | 0.288 | 0% |

Phase 3 Calibration

In this section, we provide phase 3 calibration results for channel blockage. In this simulation, we consider Urban Macro with 30GHz and blockage model-A K=5 Landscape mode in TR38.900.

Figure 2. CDF of coupling loss at 0ms with channel blockage

Figure 3. CDF of wideband SINR at 0ms with channel blockage

Figure 4. CDF of ASA at 0ms with channel blockage

Figure 5. CDF of with channel blockage

In Figure 5, the outage can be calculated as 25%.

Conclusion

In this contribution, we provided our simulation results for phase 2 and phase 3 NR-MIMO calibration according to the agreed scenarios and simulation assumptions.

References

R1-168547, Offline discussion further updated summary of evaluation assumptions for NR, NTT DOCOMO.

- Annex

Table A-1. Simulation assumptions of large scale calibration

| Parameter | Values |
| --- | --- |
| BS Tx power | Refer to [1] |
| Noise figure for BS | Below 6GHz: 5 dB; Above 6GHz: 7 dB |
| Noise figure for UE | Below 6GHz: 9 dB; Above 6GHz: 10dB |
| BS antenna configurations | Refer to the baselines in [1] and Option 1 of Indoor Hotspot agreed in [86-27]. |
| BS antenna pattern | Refer to [1] |
| UE antenna configurations | 30GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 4GHz: (M,N,P,Mg,Ng) = (1,1,2,1,1) The panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels. 4GHz: (M,N,P,Mg,Ng) = (1,1,2,1,1) |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| UE antenna pattern | Refer to [1] |
| Inter-panel calibration | Ideal |
| UE receiver type | MMSE-IRC |
| Number of UEs | Refer to [1] |
| UE distribution | Refer to [1] |
| Transmission scheme | SU-MIMO with SVD digital beamfomring |
| Rank restriction | Rank 1 |
| Digital precoding | SVD based beamforming |
| Scheduler | Round Robin scheduler |
| Feedback | CQI, PMI and RI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB |
| Hybrid ARQ | Maximum 4 transmissions |
| Traffic model | Full buffer |