3GPP TSG RAN WG1 Meeting #89	R1-17xxxxx

Hangzhou, China, 15-19 May, 2017

Agenda Item:	6.2.3.3.3

Source:	Huawei, HiSilicon

Title:	Evaluation results of short TTI solutions for Rel-15 sidelink

Document for:	Discussion and decision

# Introduction

At the RAN1 #88bis meeting, the agreements below were reached [1]:

- For study of PC5 operation with short TTI

  - Evaluation of sTTI performance is done by means of analysis, link level and system level simulation

  - Maximum latency between packet arrival at Layer 1 and resource selected for transmission

  - Other latency improvements can be evaluated

  - Improvement reliability can be considered including retransmission if used

  - Impact on Rel-14 UEs is evaluated

  - For system level evaluations, the target for maximum latency between packet arrival at Layer 1 and resource selected for transmission is [20] ms at least for Rel-15 UEs

    - Discuss further the [20] ms value

In this contribution, evaluation results and analysis of candidate sTTI solutions are provided. The corresponding solution and some assumptions can be found in our companion contributions [2] and [3] respectively.

# Simulation results

PRR performance and AGC impact results are provided in this section. The results corresponding to the shared AGC symbol option are shown in Subsection 2.1. AGC impacts are analyzed in Subsection 2.2. Both freeway 140km/h and 70km/h scenarios as agreed in [4] are simulated. For mixed scenarios, the proportions of UEs are: (Rel-14 UE, Rel-15 UE) = {(50, 50), (20, 80. In all cases, it is assumed that (T1, T2) = (1ms, 20ms) for Rel-14 UEs and (T1, T2) = (1ms, 10ms) for Rel-15 UEs. Detailed system level simulation assumptions and parameters are listed in Table 1 in Appendix A. The corresponding sTTI link-level assumptions and results are provided in Appendix B and C, respectively.

## PRR performance

With the shared AGC symbol option, a UE transmitting data in the 2nd sTTI of one subframe also transmits in the 1st symbol of the subframe in order to facilitate AGC settling and avoid clipping effects. Simulation results are shown in Figure 1 and Figure 2, respectively. The 50/50 and 20/80 mixture cases are marked in red and blue, respectively. Legacy Rel-14 and pure R15 performance curves are plotted for reference as well.

Figure 1.  PRR Performance (freeway, 140km/h)

Figure 2.  PRR Performance (freeway, 70km/h)

Based on the evaluation results, we draw the following observations:

Observation 1:

- Rel-15 sTTI UEs co-existing in the same resource pool as Rel-14 legacy UEs have no impact on Rel-14 PRR performance. In particular, Rel-14 UEs achieve significantly better PRR performance than legacy deployment scenario.

- As Rel-15 sTTI UE penetration increases, both Rel-14 and Rel-15 achieves increasingly better PRR performance, since sTTI transmission has lower system impacts than normal TTI transmissions.

- Pure Rel-15 achieves the best PRR performance in the 140km/h scenario. The performance degrades in the 70km/h case due to increased PSSCH frequency occupation and vehicle density.

## AGC impact

Figure 3.  CDF distribution of receiving power difference between two sTTIs in one subframe

The CDF distribution of the receiving power difference between two sTTIs in one subframe is depicted in Figure 2. Based on the evaluation results, we draw the following observations:

Observation 2: 95th percentile power difference between two Rel-15 sTTI transmissions in one subframe is 20 dB.

Observation 3: ADC implementations with 20dB anti-clipping capability are sufficient to avoid clipping effects with 95% probability.

# Conclusion

In this contribution, the simulation results of sTTI are provided and the following observations are made:

Observation 1:

- Rel-15 sTTI UEs co-existing in the same resource pool as Rel-14 legacy UEs has no impact on Rel-14 PRR performance. In particular, Rel-14 UEs achieve significantly better PRR performance than legacy deployment scenario.

- As Rel-15 sTTI UE penetration increases, both Rel-14 and Rel-15 achieves increasingly better PRR performance, since sTTI transmission has lower system impacts than normal TTI transmissions.

- Pure Rel-15 achieves the best PRR performance in the 140km/h scenario. The performance degrades in the 70km/h case due to increased PSSCH frequency occupation and vehicle density.

Observation 2: 95th percentile power difference between two Rel-15 sTTI transmissions in one subframe is 20 dB.

Observation 3: ADC implementations with 20dB anti-clipping capability are sufficient to avoid clipping effects with 95% probability.

# References

“RAN1 Chairman’s Notes”, RAN1 #88bis, Spokane, USA, April. 3-7, 2017.

R1-1707006, “Short TTI solutions for Rel-15 sidelink”, Huawei, Hisilicon, RAN1 #89, Hangzhou, China, 15-19 May, 2017.

R1-1707005, “Remaining details of evaluation methodology for short TTI in Rel-15 sidelink”, Huawei, Hisilicon, RAN1 #89, Hangzhou, China, 15-19 May, 2017.

TR 36.885, “Study on LTE-based V2X Services”, v14.0.0, 2016.

# Appendix A: System-level simulation setting of sTTI

Table 1	System level simulation setting of sTTI

| Parameter | Value |
| --- | --- |
| Deployment scenario | Same as Rel-14 deployment scenario |
| Proportion of Rel-14 and Rel-15 UEs | (Rel-14 UE, Rel-15 UE) = {(50, 50), (20, 80)} - Case 1: Rel-15 UEs use 1ms TTI (SA and data) - Case 2: Rel-15 UEs use 0.5ms TTI (SA and data) |
| Traffic model | Periodic broadcast traffic: - Rel-14: 4 x 190 byte + 1 x 300 byte; 100 ms period; 100 ms latency - Rel-15: 4 x 190 byte + 1 x 300 byte; 100 ms period; 20 ms latency |
| Resource (re-)selection for Rel-15 | Shared AGC symbol option: Rel-14 resource (re-)selection with the following modification: PSSCH-RSRP and S-RSSI measurements based on slot granularity. |
| T1/T2 assumptions | - Rel-14: T1=1, T2=20 - Rel-15: T1=1, T2=10 |
| Number of transmission(s) per packet | 2 |
| TTI Structure | - Subframe TTI granularity (LTE Rel-14 legacy TTI structure) - Slot TTI granularity (Table 3 and Table 4 below) |
| AGC settling time | Same as Rel-14 |
| Time for Tx/Rx switching | Same as Rel-14 |
| Frequency allocation | - Subframe TTI granularity: - 2 PRB for SCI - 12 PRB for 190 bytes 16 PRB for 300 bytes - Slot TTI granularity: - 4 PRB for SCI - 45 PRB for both 190 bytes and 300 bytes |
| Performance metric used for comparison | - The PRR performance of V2V communication among Rel-15 UEs - The PRR performance of V2V communication among Rel-14 UEs - The PRR performance of V2V communication from Rel-14 UEs to Rel-15 UEs |

# Appendix B: Link-level simulation setting of sTTI

Table 2	Link level simulation setting of sTTI

| Parameter | Value |
| --- | --- |
| General parameters: |  |
| Carrier frequency | 6G Hz |
| System bandwidth | 10 MHz |
| Channel Model | AWGN |
| Tx-Rx antenna configuration | 1x2 |
| Receiver type | MMSE |
| TTI length | 7symbols |
| PSSCH parameters: |  |
| TBS | 300Bytes, 190Bytes |
| Modulation | QPSK |
| PRB | 45 |
| PSCCH parameters: |  |
| TBS | 48bits (including 16bits CRC) |
| Modulation | QPSK |
| PRB | 4 |

# Appendix C: Link-level simulation results of sTTI

Link-level simulation results of sTTI are shown in Figure 4 for PSCCH and Figure 5 for PSSCH, where the curves corresponding to 190 bytes and 300 bytes are drawn in Figure 5(a) and 5(b), respectively. The red curves represent the link-level performance corresponding to the 1st TTI (1 AGC, 2 DMRS, 3 Data, 1 Gap).  The blue and black curves represent the link-level performances corresponding to the 2nd TTI with different overhead: Blue (1 AGC, 2 DMRS, 3 Data, 1 Gap) and black (2 DMRS, 4 Data, 1 Gap). As can be observed, the red and blue curves almost overlap due to similar overhead. The black curve has approximately 1dB gain for PSCCH and 2-3 dB gain for PSSCH.

The evaluated sTTI structures for the shared AGC option are illustrated in Table 3. The assumptions are

- Rel-15 UEs transmitting in either the 1st or 2nd sTTI will transmit in the 1st symbol of the subframe.

- For UE transmitting in the 1st sTTI, it can receive in the 2nd sTTI (i.e., Structure (a)).

- For UE transmitting in the 2nd sTTI, it cannot receive in the 1st sTTI due to large overhead.

- For UE not transmitting in this subframe, it can receive in both sTTIs (i.e., Structure (b)).

Table 3	Evaluated TTI structures for shared AGC symbol option

| Rel-15 UE transmitting in the 1st sTTI | Rel-15 UE not transmitting in the subframe |
| --- | --- |
| Structure (a) | Structure (b) |
| 2nd slot: Blue curve in Figure 2/3 | 1st slot: Red curve in Figure 2/3 2nd slot: Black curve in Figure 2/3 |

Figure 4.  Link-level PSCCH performance of sTTI with slot granularity

(a) 190 bytes								   (b) 300 bytes

Figure 5.  Link-level PSSCH performance of sTTI with slot granularity