3GPP TSG RAN WG1 Meeting #89	       	R1-1709300

Hangzhou, China, 15 - 19 May 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | [924] | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [DRAFT] Corrections to Mode-3 scheduling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Core |  |  |  |  |  | Date: |  |  | 2017-05-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The same subframe indexing is used for PC5 and Uu transmissions even though they may take place on different carriers with different indices. The timing of the PC5 transmission is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mode 3 transmission takes place on the first valid PC5 subframe after the start of subframe n+4+m on the Uu carrier based on transmitter timing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC5 subframe to use for Mode-3 transmission may be ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 14.2.1	UE procedure for transmitting the PSCCH

For sidelink transmission mode 1and PSCCH period i,

- the UE shall determine the subframes and resource blocks for transmitting SCI format 0 as follows.

-	SCI format 0 is transmitted in two subframes in the subframe pool and one physical resource block per slot in each of the two subframes, wherein the physical resource blocks belong to the resource block pool, where the subframe pool and the resource block pool are indicated by the PSCCH resource configuration (as defined in Subclause 14.2.3)

-	the two subframes and the resource blocks are determined using “Resource for PSCCH” field () in the configured sidelink grant (described in [8]) as described in Subclause 14.2.1.1.

- the UE shall set the contents of the SCI format 0 as follows:

-	the UE shall set the Modulation and coding scheme field according to the Modulation and coding scheme indicated by the higher layer parameter mcs-r12 if the parameter is configured by higher layers.

-	the UE shall set the Frequency hopping flag according to the “Frequency hopping flag” field in the configured sidelink grant.

-	the UE shall set the Resource block assignment and hopping resource allocation according to the “Resource block assignment and hopping resource allocation” field in the configured sidelink grant.

-	the UE shall set the Time resource pattern according to the “Time resource pattern” field in the configured sidelink grant .

-	the UE shall set the eleven-bit Timing advance indication to  to indicate sidelink reception timing adjustment value using the NTA (defined in [3]) value for the UE in the subframe that is no earlier than subframe  (described in Subclause 14.2.1.1).

For sidelink transmission mode 2,

-	SCI format 0 is transmitted in two subframes in the subframe pool and one physical resource block per slot in each of the two subframes, wherein the physical resource blocks belongs to the resource block pool, where the subframe pool and the resource block pool are indicated by the PSCCH resource configuration (as defined in Subclause 14.2.3)

-	the two subframes and the resource blocks are determined using the procedure described in Subclause 14.2.1.2

-	the UE shall set the eleven-bit Timing advance indication  in the SCI format 0 to zero.

For sidelink transmission mode 3,

-	The UE shall determine the subframes and resource blocks for transmitting SCI format 1 as follows:

-	SCI format 1 is transmitted in two physical resource blocks per slot in each subframe where the corresponding PSSCH is transmitted.

-	If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-RNTI, one transmission of PSCCH is in the PSCCH resource  (described in Subclause 14.2.4) in the first subframe that is included in  and that starts not earlier than TDL-NTA2×TS+4+m×10-3.  is the value indicated by "Lowest index of the sub-channel allocated to the initial transmission" associated with the configured sidelink grant (described in [8]),  is determined by Subclause 14.1.5, the value m is indicated by ‘Subframe offset’ field in the corresponding DCI format 5A if this field is present and m=0 otherwise, TDL is the start of the downlink subframe carrying the DCI, and NTA and TS are described in [3].

-	If “Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe  where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant.  corresponds to the value  determined by the procedure in Subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-SPS-RNTI, the UE shall consider the received DCI information as a valid sidelink semi-persistent activation or release only for the SPS configuration indicated by the SL SPS configuration index field. If the received DCI activates an SL SPS configuration, one transmission of PSCCH is in the PSCCH resource  (described in Subclause 14.2.4) in the first subframe that is included in  and that starts not earlier than TDL-NTA2×TS+4+m×10-3. is the value indicated by "Lowest index of the sub-channel allocated to the initial transmission" associated with the configured sidelink grant (described in [8]),  is determined by Subclause 14.1.5, the value m is indicated by ‘Subframe offset’ field in the corresponding DCI format 5A if this field is present and m=0 otherwise, TDL is the start of the downlink subframe carrying the DCI, and NTA and TS are described in [3].

-	If “Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe  where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant.  corresponds to the value  determined by the procedure in Subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	The UE shall set the contents of the SCI format 1 as follows:

-	the UE shall set the Modulation and coding scheme as indicated by higher layers.

-	the UE shall set the “Priority” field according to the highest priority among those priority(s) indicated by higher layers corresponding to the transport block.

-	the UE shall set the Time gap between initial transmission and retransmission field, the Frequency resource location of the initial transmission and retransmission field, and the Retransmission index field such that the set of time and frequency resources determined for PSSCH according to Subclause 14.1.1.4C is in accordance with the PSSCH resource allocation indicated by the configured sidelink grant.

-	the UE shall set the Resource reservation to zero.

-	Each transmission of SCI format 1 is transmitted in one subframe and two physical resource blocks per slot of the subframe.

-	The UE shall randomly select the cyclic shift  among {0, 3, 6, 9} in each PSCCH transmission.

For sidelink transmission mode 4,

-	The UE shall determine the subframes and resource blocks for transmitting SCI format 1 as follows:

-	SCI format 1 is transmitted in two physical resource blocks per slot in each subframe where the corresponding PSSCH is transmitted.

-	If the configured sidelink grant from higher layer indicates the PSCCH resource in subframe , one transmission of PSCCH is in the indicated PSCCH resource m (described in Subclause 14.2.4) in subframe .

-	If "Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe  where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant,  corresponds to the value  determined by the procedure in Subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	the UE shall set the contents of the SCI format 1 as follows:

-	the UE shall set the Modulation and coding scheme as indicated by higher layers.

-	the UE shall set the “Priority” field according to the highest priority among those priority(s) indicated by higher layers corresponding to the transport block.

-	the UE shall set the Time gap between initial transmission and retransmission field, the Frequency resource location of the initial transmission and retransmission field, and the Retransmission index field such that the set of time and frequency resources determined for PSSCH according to Subclause 14.1.1.4C is in accordance with the PSSCH resource allocation indicated by the configured sidelink grant.

-	the UE shall set the Resource reservation field according to table 14.2.1-1 based on indicated value X, where X is equal to the Resource reservation interval provided by higher layers divided by 100.

-	Each transmission of SCI format 1 is transmitted in one subframe and two physical resource blocks per slot of the subframe.

-	The UE shall randomly select the cyclic shift  among {0, 3, 6, 9} in each PSCCH transmission.

Table 14.2.1-1: Determination of the Resource reservation field in SCI format 1

| Resource reservation field in SCI format 1 | Indicated value X | Condition |
| --- | --- | --- |
| ‘0001’, ‘0010’, …, ‘1010’ | Decimal equivalent of the field | The higher layer decides to keep the resource for the transmission of the next transport block and the value X meets . |
| ‘1011’ | 0.5 | The higher layer decides to keep the resource for the transmission of the next transport block and the value X is 0.5. |
| ‘1100’ | 0.2 | The higher layer decides to keep the resource for the transmission of the next transport block and the value X is 0.2. |
| ‘0000’ | 0 | The higher layer decides not to keep the resource for the transmission of the next transport block. |
| ‘1101’, ‘1110’, ‘1111’ | Reserved |  |

Table 14.2.1-1: Mapping of SCI format 1 offset field to indicated value m

| Subframe offset field in SCI format 1 | Indicated value m |
| --- | --- |
| ‘00’ | 0 |
| ‘01’ | 1 |
| ‘10’ | 2 |
| ‘11’ | 3 |

#### < Unchanged parts are omitted >