3GPP TSG RAN WG1 89			R1- 1709501

Hangzhou, China, May 15th-19th

Agenda item:	7.1.2.1.3

Source: 	Intel

Title: 	Summary of Issues and WFs on Codebook based transmission for UL

Document for:	Discussion

# Introduction

In RAN1#88bis, the following agreements were made [1]:

Agreements: - Codebook based transmission for UL is supported at least by following signaling in UL grant: - SRI+TPMI+TRI, where - The TPMI is used to indicate preferred precoder over the SRS ports in the selected SRS resource by the SRI. - No SRI when a single SRS resource is configured - The TPMI is used to indicate preferred precoder over the SRS ports in the configured single SRS resource. - Support indication on selection of multiple SRS resources - FFS details Agreements: - For codebook based transmission for CP-OFDM based UL, when a UE is configured with UL frequency selective precoding and if subband TPMI signaling is supported, support one of the following alternatives: - Alt 1:Subband TPMIs are signaled via DCI to the UE only for allocated PRBs for a given PUSCH transmission - Alt 2:Subband TPMIs are signaled via DCI to the UE for all PRBs in UL, regardless of the actual RA for a given PUSCH transmission - Other alternatives are not precluded - Note: Subband TPMI may correspond to W2 if dual-stage codebook is supported - FFS whether or not wideband TPMI is always signaled along with subband TPMI

Based on the submitted contributions in RAN1#89 for this agenda item, the following issues are identified and summarized in the following sections.

- UL codebook structure

- DL control signaling

# UL codebook structure

There two options for the UL codebook structure:

- Single-stage codebook: Qualcomm, Ericsson, Nokia(wideband transmission), Intel

- Two-stage codebook: LG, Samsung, Nokia(frequency selective transmission)

# DL Control Signaling

In terms of the DL subband TPMI indication, there are the following options:

- Couple TPMI & RA: CATT, Ericsson

- Decouple TPMI & RA: Intel

- Wide-band and mid-band indication: IDC

# List of Current WF

The following related WFs are listed below:

WF on UL codebook design principle  		LG

- UL codebook is designed by assuming a single panel.

  - Codebook details are FFS

  - Existing LTE codebooks should be considered as baseline

WF on codebook based UL transmission		Huawei

- For UL frequency selective precoding, NR supports subband TPMI indication to the UE via DCI for the allocated PRBs only

  - Subband TPMIs are signaled in the second stage DCI if two-stage-DCI is supported

- Support at least two-stage codebook in NR for codebook based UL-MIMO

- On top of indication of multiple SRS resources selection, support signaling to indicate TPMI across the selected multiple SRS resources in the UL grant for PUSCH precoder determination

# Possible Agreement

The following may be the possible agreement:

- UL codebook is designed by assuming a single panel

- Re-use LTE DL codebook for Rel-8 to be the UL codebook at least for 2 transmission antenna port for CP-OFDM.

- Define the uplink codebook first and then determine the number of transmission ports for frequency selective precoding.

# References

- 3GPP, RAN1#88bis, Chairman’s Notes