3GPP TSG RAN WG1 Meeting #89			R1-1709660

# Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:	7.1.9

Source: 	ZTE

Title: 	Summary of Power Control Issues

Document for:	Discussion

Introduction

In RAN1#88-bis, the following agreements were made [1]:

Agreements: - For beam specific power control, NR defines beam specific open & closed loop parameters. - FFS: details on beam common parameter(s) - Note: Agreed on RAN1 #88 FFS details on “beam specific”, especially regarding handling layer/layer-group/panel specific/beam group specific/beam pair link specific power control - gNB is aware of the power headroom differences for different waveforms, if the UE can be configured for both waveforms. - FFS: offset configured/specified, reported, - FFS on the details of power control parameters for example, P_c, Max or other open/closed loop parameter

Based on the submitted contributions in RAN1#89 for this agenda item ([2] - [40]), at least the following issues are identified and summarized in the following sections. Due to the many issues remaining for power control, only a few are identified here following the topics in the submitted contributions.

Issues in NR for PC

Observation:

Compared with LTE, NR needs to consider more complicated cases, e.g. multi-numerology, multi-beam (multi-panel), two waveforms, multi-service/traffic (eMBB, URLLC), multi-access schemes (NOMA), all of which will affect the UL PC design.

XX-specific Power control:

For XX-specific Power control, beside beam-specific power control (agreed):

- Waveform specific power control:

  - PCMAX value should be determined per waveform: InterDigital, Qualcomm, NTT DOCOMO, Nokia, Samsung, CATT, ZTE, OPPO

- Numerology specific power control: InterDigital, Nokia, Samsung, CATT, Huawei

- Separate power control for each waveform (reuse some parameters) :InterDigital, Qualcomm, Nokia, Intel, OPPO

- Traffic specific power control: Huawei

- Panel specific UL power control is not required: Ericsson

- Different UL multiple access schemes, OMA/NOMA: CATT

- Different reliability requirements, such as eMBB and URLLC, the UE specific component should be set to different value: CATT, LG

XX-specific PHR (power headroom）

- Beam specific PHR: Nokia, Huawei

- Multiple PHR for multiple potential beam candidates: Nokia

- Separate PHR per waveform or offset: InterDigital, QC, Nokia, NTT DOCOMO

- Separate power headroom reports based on the factors of service, waveform, numerology, and beam of PUSCH transmission: Samsung

- Study power headroom reports for FDM of PUSCH and long PUCCH case, TDM of PUSCH and short PUCCH case: Samsung

- PHR calculation for CA operation supports different scheduling durations among cells.: Samsung

RS for pathloss calculation：

- ZTE: multiple RS types, configured flexibly

- Ericsson: multiple RS types, RS type related parameter in PC formula

- Mitsubishi: NR-SS + CSI-RS;

- CMCC: PRACH using NR-SS, and it is preferred to explicitly indicate beam;

- Intel：CSI-RS for beam management P-1/P-2/P-3 used for PL measurement;

- Nokia: DL RS for beam management；CSI-RS in P1/P2/P3 and mobility specific RS can be used for pathloss estimate in open-loop power control

- HW: Both RS for mobility and UE-specific RS for beam management based pathloss estimations should be supported.

•	L3 RSRP based on RS for mobility should be supported

•	L1 RSRP based on UE-specific reference signal should be supported.

- LG：for OL-PC, proper DL RS for pathloss compensation should be defined or configured per SRS resource;

- ASUSTEK：Association between different types of UL transmission and DL RS

Others:

- Interference aware PC: Huawei, Samsung, Nokia, ZTE

- Layer specific UL power control is not required: Ericsson

- NR does not support FDM of simultaneous PUSCH and S-PUCCH transmissions from a same UE: Samsung

Power sharing between NR and LTE

Note that RAN4 requested RAN1 to study the feasibility of the power sharing mechanisms between LTE and NR as soon as possible.

Some companies as follows consider the power sharing between NR and LTE, but it seems no consensus currently.

- Samsung

- NTT DOCOMO

- InterDigital

- Nokia

WFs

In addition to the above topics, the following WFs are identified for this topic to the best of our knowledge:

- [R1-1709660]  WF on UL power control ZTE

- [R1-17xxxxx]  WF on UL power sharing between LTE and NR NTT DOCOMO, Ericsson, InterDigital, Nokia, Huawei

- References

- 3GPP, RAN1#88, Chairman’s Notes

- R1-1706904	Power control for CA and DC	Huawei, HiSilicon

- R1-1706930	Detailed considerations on UL power control design for NR	Huawei, HiSilicon

- R1-1707057	UL transmission power control	Mitsubishi Electric Co.

- R1-1707137	UL MIMO power control	ZTE

- R1-1707422	Discussion On UL Power Control for data channel	Intel Corporation

- R1-1707529	PUSCH Power Control in Multi-beam configuration	CATT

- R1-1707530	PUCCH Power Control and Priority in Power Scaling	CATT

- R1-1707531	SRS Power Control in Multi-beam configuration	CATT

- R1-1707532	NR Power Headroom Calculation	CATT

- R1-1707681	Discussion on beam-specific power control for UL	LG Electronics

- R1-1707682	Discussion on UL power control for NR	LG Electronics

- R1-1707683	Discussion on power sharing for LTE-NR DC	LG Electronics

- R1-1707728	Discussion on uplink power control	Guangdong OPPO Mobile Telecom.

- R1-1707808	Dual Connectivity between E-UTRA and NR	NEC

- R1-1708063	UL Power Control Aspects in NR	Samsung

- R1-1708064	Feasibility for Power Sharing between LTE an NR	Samsung

- R1-1708065	Power Control for Transmissions with Different Numerologies	Samsung

- R1-1708066	On PHR aspects for NR	Samsung

- R1-1708067	Power Control for UL CA	Samsung

- R1-1708068	Power Control for Multi-Beam Operation	Samsung

- R1-1708069	Power Control for PUCCH	Samsung

- R1-1708220	UL Power Control for MIMO	Nokia, Alcatel-Lucent Shanghai Bell

- R1-1708221	On power sharing between LTE and NR in DC	Nokia, Alcatel-Lucent Shanghai Bell

- R1-1708361	Power Control for Multi Beam Transmission in NR	InterDigital, Inc.

- R1-1708404	Beam specific UL power control procedure	CMCC

- R1-1708496	Uplink power control considering waveform switching	NTT DOCOMO, INC.

- R1-1708497	Power sharing between LTE and NR in NSA	NTT DOCOMO, INC.

- R1-1708656	Power control and PHR for different waveforms	Qualcomm Incorporated

- R1-1708657	Multi-beam PDCCH design	Qualcomm Incorporated

- R1-1708658	Performance comparison between CA-polar and PC-polar for very small block length	Qualcomm Incorporated

- R1-1708659	PDSCH structure	Qualcomm Incorporated

- R1-1708660	PUSCH structure	Qualcomm Incorporated

- R1-1708714	On reference signals for UL power control	Ericsson

- R1-1708715	On UL power control for NR	Ericsson

- R1-1709004	Power Sharing in NR	InterDigital Inc.

- R1-1709011	Power control with multiple waveforms and numerologies	InterDigital Inc.

- R1-1709047	UL power control in multi-beam based approaches	ASUSTEK COMPUTER (SHANGHAI)

- R1-1709118	Impact of Multiple Waveforms on UL Power Control for NR	Ericsson

- R1-1709119	On Power Control Options for LTE-NR NSA Operation	Ericsson