3GPP TSG RAN WG1 Meeting #90			R1-1712552

Prague, P.R. Czechia 21th – 25th August 2017

Source: 	Intel Corporation

Title:	Discussion for Mechanism to Recover from Beam Failure

Agenda item:	    6.1.2.2.7

Document for:	Discussion and Decision

# Introduction

In RAN1 NR AH #2 meeting, the following agreements have been achieved. [1]

Agreements:

- RAN1 agrees that the certain number of beam failure recovery request  transmissions is NW configurable by using some parameters

  - Parameters used by the NW could be:

    - Number of transmissions

    - Solely based on timer

    - Combination of above

- FFS: whether beam failure recovery procedure is influenced by the RLF event

In RAN1 #89 meeting, the following working assumptions and agreements on beam recovery have been achieved. [2]

Working assumption:

- Support at least the following triggering condition(s) for beam failure recovery request transmission:

  - Condition 1: when beam failure is detected and candidate beam is identified at least for the case when only CSI-RS is used for new candidate beam identification

  - FFS Condition 2: Beam failure is detected alone at least for the case of no reciprocity

    - FFS how the recovery request is transmitted without knowledge of candidate beam

  - Note: if both conditions are supported, which triggering condition to use by UE also depends on both gNB configuration and UE capability

Agreements:

- Support the following channel(s) for beam failure recovery request transmission:

  - Non-contention based channel based on PRACH, which uses a resource orthogonal to resources of other PRACH transmissions, at least for the FDM case

    - FFS other ways of achieving orthogonality, e.g., CDM/TDM with other PRACH resources

    - FFS whether or not have different sequence and/or format than those of PRACH for other purposes

    - Note: this does not prevent PRACH design optimization attempt for beam failure recovery request transmission from other agenda item

    - FFS: Retransmission behavior on this PRACH  resource is similar to regular RACH procedure

  - Support using PUCCH for beam failure recovery request transmission

    - FFS whether PUCCH is with beam sweeping or not

    - Note: this may or may not impact PUCCH design

  - FFS Contention-based PRACH resources as supplement to contention-free beam failure recovery resources

    - From traditional RACH resource pool

    - 4-step RACH procedure is used

    - Note: contention-based PRACH resources is used e.g., if a new candidate beam does not have resources for contention-free PRACH-like transmission

  - FFS whether a UE is semi-statically configured to use one of them or both, if both, whether or not support dynamic selection of one of the channel(s) by a UE if the UE is configured with both

Agreements:

- To receive gNB response for beam failure recovery request, a UE monitors NR PDCCH with the assumption that the corresponding PDCCH DM-RS is spatial QCL’ed with RS of the UE-identified candidate beam(s)

  - FFS whether the candidate beam(s) is identified from a preconfigured set or not

  - Detection of a gNB’s response for beam failure recovery request during a time window is supported

    - FFS the time window is configured or pre-determined

    - FFS the number of monitoring occasions within the time window

    - FFS the size/location of the time window

    - If there is no response detected within the window, the UE may perform re-tx of the request

      - FFS details

  - If not detected after a certain number of transmission(s), UE notifies higher layer entities

    - FFS the number of transmission(s) or possibly further in combination with or solely determined by a timer

In RAN1 #88bis meeting, the following agreements on beam recovery have been achieved. [3]

- UE Beam failure recovery mechanism includes the following aspects

  - Beam failure detection

  - New candidate beam identification

  - Beam failure recovery request transmission

  - UE monitors gNB response for beam failure recovery request

- Beam failure detection

  - UE monitors beam failure detection RS to assess if a beam failure trigger condition has been met

  - Beam failure detection RS at least includes periodic CSI-RS for beam management

    - SS-block within the serving cell can be considered, if SS-block is also used in beam management as well

  - FFS: Trigger condition for declaring beam failure

- New candidate beam identification

  - UE monitors beam identification RS to find a new candidate beam

  - Beam identification RS includes

    - Periodic CSI-RS for beam management, if it is configured by NW

    - Periodic CSI-RS and SS-blocks within the serving cell, if SS-block is also used in beam management as well

- Beam failure recovery request transmission

  - Information carried by beam failure recovery request includes at least one followings

    - Explicit/implicit information about identifying UE and new gNB TX beam information

    - Explicit/implicit information about identifying UE and whether or not new candidate beam exists

    - FFS:

      - Information indicating UE beam failure

      - Additional information, e.g., new beam quality

  - Down-selection between the following options for beam failure recovery request transmission

    - PRACH

    - PUCCH

    - PRACH-like (e.g.,different parameter for preamble sequence from PRACH)

  - Beam failure recovery request resource/signal may be additionally used for scheduling request

- UE monitors a control channel search space to receive gNB response for beam failure recovery request

  - FFS: the control channel search space can be same or different from the current control channel search space associated with serving BPLs

  - FFS: UE further reaction if gNB does not receive beam failure recovery request transmission

In RAN1 #88 meeting, the following agreements on UE initiated beam recovery have been achieved [4].

- The following mechanisms should be supported in NR:

  - The UL transmission to report beam failure can be located in the same time instance as PRACH:

    - Resources orthogonal to PRACH resources

      - FFS orthogonal in frequency and/or sequences (not intended to impact PRACH design)

      - FFS channels/signals

  - The UL transmission to report beam failure can be located at a time instance (configurable for a UE) different from PRACH

    - Consider the impact of RACH periodicity in configuring the UL signal to report beam failure located in slots outside PRACH

    - FFS the signal/channel for the UL transmission

  - Additional mechanisms using other channels/signals are not precluded (e.g., SR, UL grant free PUSCH, UL control)

In this contribution, we will provide some discussions on the remaining issues of UE initiated beam recovery including content of the triggering event, physical channel details to transmit the beam recovery request and DL signal for measurement.

# Discussion

## 2.1 Condition of Beam Recovery Request

For the multi-beam operation, both the gNB and UE may maintain multiple beams. A best gNB-UE beam pair can be helpful to increase the link budget. The UE may be able to measure the beam quality of each gNB beam with corresponding UE beam, and then report this beam quality to gNB. As a result of UE movement, rotation and blockage, the beam quality may change. There could be the following beam change cases:

- Case 1: gNB beam changed, UE beam unchanged

- Case 2: gNB beam unchanged, UE beam changed

- Case 3: gNB beam changed, UE beam changed

The UE could distinguish above cases based on the downlink measurement of CSI-RS. To find out whether a new gNB Tx beam exists, the UE could measure different beam management (BM) CSI-RS resources. To find out whether a new Rx beam exists, one possible way is to use the omni-directional antenna to measure the beams. If current UE beam is not good, the RSRP for current gNB beam measured by omni-directional antenna could be better than that measured by directional antenna.

The UE could use a wider Rx beam or omni-receiving to measure the CSI-RS in P-1. Then it could find out whether a better gNB beam exists. So it can distinguish the case 2 from case 1 and 3. Then it can compare the RSRP between current Rx beam and the wider Rx beam or omni-receiving to distinguish whether a new UE beam exists. If the UE detects the gNB beam should change and the new gNB beam should not be in the maintained gNB-UE beam pair list, it could trigger the beam state reporting. If the UE also detects the UE beam should change, it could trigger the CSI-RS for P-3 to find out a better UE beam. Hence we would like to support at least two types: one for new gNB beam discovery, which can be based on the beam state reporting request; the other for new UE beam discovery, which can be based on the request of CSI-RS for P-3. In addition, there may be another type of UE that only has omni-directional antenna. Then those UE does not need to request the CSI-RS for P-3. Instead, only the new gNB beam identification is enough.

Proposal 1: Apart from identifying gNB TX beam information, the information carried by beam failure recovery request could also indicate whether Rx beam refinement is needed.

## 2.2 Transmission of beam recovery signal

It has been agreed that non-contention based PRACH and PUCCH can be used to transmit the beam recovery signal. The on/off of PRACH could be used to carry 1-bit information. The PUCCH could carry N bits information. To save overhead, it is better that the payload size of PUCCH is small. As discussed before, for UEs with the capability of Rx beamforming, there can be two types of requests: one is the request of beam reporting; the other is the request of CSI-RS for P-3. For the UE with omni-directional antenna only, only the request of beam reporting is necessary. Therefore the PUCCH can be used for the UE with the capability of Rx beamforming and PRACH can be used for the UE with omni-directional antenna only or when there is no configured resource of PUCCH for UE with Rx beamforming.

When the PUCCH for beam recovery and PRACH are multiplexed at the same time instance, one possible way is to use FDM based mapping approach. Then the signal structure for PRACH and PUCCH should have the same boundary for one time-unit, which may include multiple sub-time units, if they are received by the same Rx antenna port(s). As shown in Figure 1, the gNB could use different Rx beams in different repetition(s) and the each repetition of PUCCH and PRACH should have the same boundary. Note that the PRACH and PUCCH can be mapped continuously or discontinuously in frequency domain. If the numerologies for PRACH and PUCCH are different, the time unit for the channel with small subcarrier spacing should be the same as that with large subcarrier spacing.

Figure 1: Example of FDMed PRACH and PUCCH structure

When the PUCCH and PRACH are multiplexed in different time instance, to save the overhead of PUCCH, one possible way is to use larger subcarrier spacing. Then it could create more PUCCH resources for the UE to transmit its beam recovery request. Hence the numerology of PUCCH can be configurable.

Proposal 2: PUCCH can be used for the UE with the capability of Rx beamforming and PRACH can be used for the UE with omni-directional antenna only or when there is no configured resource of PUCCH for UE with Rx beamforming.

Proposal 3: When resources of PUCCH for beam recovery are configured in the same time instance of PRACH, those should have aligned time-unit and each PUCCH time-unit can be divided into several sub-time units by using different numerologies between PUCCH and PRACH.

Proposal 4: NR supports configurable numerology for PUCCH/PRACH for beam recovery.

## 2.3 Beam recovery without beam correspondence

Without beam correspondence, the best beam pair link (BPL) for both UL and DL could be different. As shown in Figure 2, the blockage may happen so that one BPL may be broken. For UL beam recovery, the gNB could trigger PRACH or SRS to find out a workable BPL. For DL beam recovery, the beam recovery message should help the gNB to find out a DL Tx beam to transmit the response to the beam recovery request. As both non-contention based PRACH and PUCCH can be used to carry the information, it should be studied how to define the behaviour for each channel.

Figure 2: one scenario for beam recovery without beam correspondence

For non-contention based PRACH, one possible way is to carry the DL Tx beam information by the time/frequency resource of PRACH. Then the time/frequency resource of PRACH can be one-to-one mapped to the resources of CSI-RS for beam management. Figure 3 illustrates the structure of PRACH resource, where the UE should transmit the non-contention based PRACH at PRACH resource 4. Then the gNB would use DL Tx beam 4 to transmit the beam recovery response. However the UL Rx beam to receive PRACH resource 4 may not happen to be the UL Rx beam in current UL BPL. Then how to determine the transmission power for the non-contention based PRACH can be one issue. Since current pathloss is not easy to be known by the UE, one possible way is to use the pathloss averaged from the multiple beams. Another possible way is to determine the transmission power only by the maximum transmission power since the beam recovery request may be a one-time request. Further if the UE is configured with PUCCH resource for beam recovery, it can explicitly report the targeting CRI by the PUCCH. Then the gNB can find out a beam to transmit the beam recovery request response.

Figure 3: Non-contention based PRACH for beam recovery without beam correspondence

Proposal 5: Without beam correspondence, if the UE is configured with resource of PUCCH for beam recovery, it should be supported to use the PUCCH to explicitly transmit the CRI.

Proposal 6: Without beam correspondence, if the UE is only configured with resource of non-contention based PRACH for beam recovery, the DL Tx beam information of beam recovery response can be carried by the time/frequency resource index of the PRACH, and its transmission power can be determined at least by the maximum transmission power.

## 2.4 Interference randomization

Further, the intra-cell and inter-cell interference randomization schemes can help to improve the performance of PUCCH. For example, some hopping schemes can be used for one UE to transmit multiple PUCCH repetitions. In each repetition, different inter-cell interference can be observed. The hopping scheme can include time, frequency, sequence and cyclic shift hopping. The hopping pattern can be determined by the cell ID, so that the inter-cell interference can be randomized. Figure 4 illustrates one example for the hopping of PUCCH/PRACH resources for one particular UE, where different NW beams can be used to receive different repetitions of the signal.

Figure 4: one example for hopping of PUCCH/PRACH for beam recovery

Proposal 7: NR supports interference randomization technique for PUCCH/PRACH for beam recovery, and time/frequency/sequence/cyclic shift hopping can be considered.

## 2.5 Receiving of beam recovery signal

If the reciprocity is ensured at the TRP side, the Rx beam in PUCCH resources can be one-to-one mapped to the beam in beam management reference signal for P-1. Then UE only needs to transmit one PUCCH at the resource corresponding to the selected downlink beam, which can be current TRP beam if only UE beam change happens. If the reciprocity is not guaranteed, the UE could transmit the PUCCH over multiple resources. To save the overhead, the gNB could use a sub-set of beams to receive multiple PUCCH resources. Figure 5 illustrates one example for the beam sub-set pattern. The beams within one sub-set can be highly un-correlated and cover the full span of AoA and ZoA. Then the UE could transmit the PUCCH to a group of PUCCH resource if gNB does not have the capability of beam correspondence. The overhead for the PUCCH could be reduced. Hence the beam sub-set based receiving should be supported.

Figure 5: beam sub-set pattern

Proposal 8: with regard to the overhead reduction, it should be supported that the gNB uses a sub-set of beams to receive the PUCCH for one UE when no beam correspondence in gNB side can be confirmed.

## 2.6 Control Signalling of beam recovery request

It has been agreed that the gNB can provide some parameters for the UE to determine the beam recovery request transmission. There are some candidate parameters as follows:

- Option 1. Number of beam recovery request

- Option 2. Timer for beam recovery request

Both option 1 and option 2 can be used to define when the UE behaviour of transmitting the beam recovery request. Since the resource of beam recovery for each UE should be configured by the gNB, then either option is enough to provide enough information to define this UE behaviour. However, to define a timer is much easier for a gNB to control the beam and link status within a given time. Since a UE can be configured with both non-contention based and PUCCH for beam recovery, to define the number of beam recovery request seems to be confusing if both resources are configured. Hence to option 2 looks to be a clean solution.

Proposal 9: it should be supported that a timer for beam recovery request is configured by the gNB.

# Conclusion

In this contribution we have provided our views on mechanisms to recovery from beam failure. From the discussion, we have the following proposals.

Proposal 1: Apart from identifying gNB TX beam information, the information carried by beam failure recovery request could also indicate whether Rx beam refinement is needed.

Proposal 2: PUCCH can be used for the UE with the capability of Rx beamforming and PRACH can be used for the UE with omni-directional antenna only or when there is no configured resource of PUCCH for UE with Rx beamforming.

Proposal 3: When resources of PUCCH for beam recovery are configured in the same time instance of PRACH, those should have aligned time-unit and each PUCCH time-unit can be divided into several sub-time units by using different numerologies between PUCCH and PRACH.

Proposal 4: NR supports configurable numerology for PUCCH/PRACH for beam recovery.

Proposal 5: Without beam correspondence, if the UE is configured with resource of PUCCH for beam recovery, it should be supported to use the PUCCH to explicitly transmit the CRI.

Proposal 6: Without beam correspondence, if the UE is only configured with resource of non-contention based PRACH for beam recovery, the DL Tx beam information of beam recovery response can be carried by the time/frequency resource index of the PRACH, and its transmission power can be determined at least by the maximum transmission power.

Proposal 7: NR supports PUCCH randomization technique. For example, time/frequency/sequence/cyclic shift hopping can be considered.

Proposal 8: with regard to the overhead reduction, it should be supported that the gNB uses a sub-set of beams to receive the PUCCH for one UE when no beam correspondence in gNB side can be confirmed.

Proposal 9: it should be supported that a timer for beam recovery request is configured by the gNB.

# References

- 3GPP Chairman's Notes RAN1 NR AH #2

- 3GPP Chairman's Notes RAN1 #89

- 3GPP Chairman's Notes RAN1 #88bis

- 3GPP Chairman's Notes RAN1 #88