3GPP TSG RAN WG1 Meeting #90                                            	R1-1712802

Prague, Czechia, 21th – 25th August 2017

Agenda item:       5.2.6.4

Source: 	Qualcomm Incorporated

Title: 	Increased PDSCH spectral efficiency

Document for:     Discussion/Decision

# Introduction

In RAN#75 a new work item RP-170732 “New WID on Even further enhanced MTC for LTE” was approved. One of the objectives of the work item is to define support for 64QAM in PDSCH. In RAN1#88b and RAN1 #89 the following agreements were reached:

Agreements:

- Support for 64QAM for non-repeated unicast PDSCH in connected mode in CE Mode A is introduced.

  - The feature is enabled/disabled by eNB via UE-specific signaling.

    - Unless enabled, UE shall assume no use of 64QAM.

  - The MCS field in DCI format 6-1A is [FFS: 4 or 5 bits] in UE-specific search space.

  - The max TBS for each UE category and max PDSCH channel bandwidth support is the same as Rel-14.

    - This implies that N_soft is also the same as Rel-14.

  - FFS details of MCS, TBS and CQI tables for the support of 64QAM.

Agreement:

- A capability is introduced for Rel-15 BL/CE UEs for the support of 64QAM.

- For Rel-15 BL/CE UEs, when 64QAM is enabled via higher layer configuration,

  - When the DCI indicates no PDSCH repetition,

    - The MCS field in DCI format 6-1A UE-specific search space is extended to 5 bits by reinterpreting the frequency hopping flag as the MSB

      - The size of DCI format 6-1A is not changed.

    - The modulation and TBS index table is reused from Table 7.1.7.1-1 in TS 36.213.

    - The TBS is determined using Table 7.1.7.2.1-1 in TS 36.213 with the following restriction:

      - The max TBS in Rel-14 is used wherever the TBS value in Table 7.1.7.2.1-1 exceeds the max TBS in Rel-14.

Working assumption:

- For Rel-15 BL/CE UEs, when 64QAM is enabled via higher layer configuration,

  - The CQI table is as below.

| CQI index | modulation | code rate x 1024 x | efficiency x |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | 64QAM | 466 | 2.7305 |
| 12 | 64QAM | 567 | 3.3223 |
| 13 | 64QAM | 666 | 3.9023 |
| 14 | 64QAM | 772 | 4.5234 |
| 15 | 64QAM | 873 | 5.1152 |

In this contribution we present our views on further changes required to introduce 64QAM for eMTC.

# Remaining issues for 64-QAM support

In Rel-13, the parameter RCSI was introduced to determine the number of subframes in the CSI reference resource for eMTC. For instance, if RCSI=1, the feedback is determined based on a single repetition.

This parameter has also impact on several other procedures. In the following, we list two agreement/specification paragraphs and devise possible conflicts between them:

- Rel-15 Agreement: Support for 64QAM for non-repeated unicast PDSCH in connected mode in CE Mode A is introduced.

  - This also implies that the UE cannot consider 64-QAM for CSI feedback if RCSI=1

- TS 36.213: If the BL/CE UE is configured in CEModeA, and if the PDSCH is assigned by or semi-statically scheduled by a MPDCCH with DCI format 6-1A, the UE shall assume no PDSCH repetition if the higher layer parameter csi-NumRepetitionCE-r13 indicates one subframe.

These two behaviors implicitly define that the UE cannot be configured simultaneously with 64-QAM and repetitions.

Observation 1: Under current specification, the UE cannot be configured simultaneously with 64-QAM and repetitions.

This behaviour is undesirable due to the fact that the eNB would need to reconfigure the UE when radio conditions deteriorate. We present the following options to determine the UE behaviour:

- Option 1: The UE is not expected to be configured with 64-QAM and repetitions simultaneously

- Option 2: When configured in 64-QAM mode and with repetitions, the UE uses a modified CQI table:

  - Option 2.1: Use the agreed 64-QAM table, but disregard the entries specific to 64-QAM

  - Option 2.2: Use the legacy 16-QAM table

  - Option 2.3: Use the agreed 64-QAM table, but set RCSI=1 for the entries corresponding to 64-QAM

Option 2.1 and 2.2 have a smaller dynamic range, since the peak spectral efficiency would be that of the maximum  16-QAM MCS. Option 2.3 offer a better dynamic range, but they have a discontinuity in the spectral efficiency due to changing RCSI. An example implementation of option 2.3 is shown in Table 1.

Table 1: Proposal of modified table for RCSI>1

| CQI index | modulation | code rate x 1024 | Assumed RCSI | efficiency x RCSI |
| --- | --- | --- | --- | --- |
| 0 | out of range |  |  |  |
| 1 | QPSK | 40 | RCSI | 0.0781 |
| 2 | QPSK | 78 | RCSI | 0.1523 |
| 3 | QPSK | 120 | RCSI | 0.2344 |
| 4 | QPSK | 193 | RCSI | 0.3770 |
| 5 | QPSK | 308 | RCSI | 0.6016 |
| 6 | QPSK | 449 | RCSI | 0.8770 |
| 7 | QPSK | 602 | RCSI | 1.1758 |
| 8 | 16QAM | 378 | RCSI | 1.4766 |
| 9 | 16QAM | 490 | RCSI | 1.9141 |
| 10 | 16QAM | 616 | RCSI | 2.4063 |
| 11 | 64QAM | 466 | 1 | 2.7305 |
| 12 | 64QAM | 567 | 1 | 3.3223 |
| 13 | 64QAM | 666 | 1 | 3.9023 |
| 14 | 64QAM | 772 | 1 | 4.5234 |
| 15 | 64QAM | 873 | 1 | 5.1152 |

Proposal 1: When the UE is configured with 64-QAM and RCSI>1, and for CQI calculations, the UE use the agreed 64-QAM CQI table, but assumes RCSI =1 for the entries corresponding to 64-QAM.

# Summary of proposals

Observation 1: Under current specification, the UE cannot be configured simultaneously with 64-QAM and repetitions.

Proposal 1: When the UE is configured with 64-QAM and RCSI>1, and for CQI calculations, the UE use the agreed 64-QAM CQI table, but assumes RCSI =1 for the entries corresponding to 64-QAM.