3GPP TSG RAN WG1 Meeting #90		R1-1713352

Prague, Czechia, 21th – 25th August 2017

Source:	Panasonic

Title: 	Discussion on UCI multiplexing with different use cases

Agenda Item:		6.1.3.2.7

Document for:	Discussion

Introduction

Following agreements had been made in RAN2 on SR design [1].

Agreements

1.	Multiple SR configurations can be configured to the UE and which SR configuration is used depends on the LCH that triggers the SR.  The granularity of SR configuration for a logical channel is FFS.

2. 	From RAN2 point of view a single bit SR with multiple SR configuration is sufficient to distinguish the “numerology/TTI length” of the logical channel that trigger the SR.  RAN2 has not identified other use cases for which multibit SR is need with sufficient support.

3.	RAN2 does not see the need to convey buffer status information.

4. 	Send LS to RAN1 to indicate to RAN1 that RAN2 doesn’t see the need to support multi-bit SR.

This contribution discusses SR for URLLC and multiplexing of SR and HARQ-ACK considering different use cases such as URLLC and eMBB.

Discussion

SR for URLLC

From URLLC perspective, SR resource (periodicity) should be at least around once per 0.125ms in order to satisfy the latency requirement agreed in [2] i.e. “For URLLC the target for user plane latency should be 0.5ms for UL, and 0.5ms for DL” and “a general URLLC reliability requirement for one transmission of a packet is 1-10-5 for 32 bytes with a user plane latency of 1ms”. There could be a situation when HARQ-ACK and SR for both URLLC are simultaneously required to be sent. For the case when both HARQ-ACK and SR transmission are occurred at the same time, to drop either HARQ-ACK or SR would possible but it means increase latency. Short latency is the one of merits of short PUCCH operation, and, therefore, HARQ-ACK and SR simultaneous operation is essential at least for URLLC. We think for short-PUCCH, SR should be designed to allow the multiplexing with 1 or 2 bits UCI payload size in the same symbol.  The design on multiplexing of 1 or 2 bits UCI and SR for short-PUCCH is discussed in [3].

Proposal 1: SR resource (periodicity) of one of configurations should be at least around once per 0.125ms in order to satisfy the latency requirement.

Proposal 2: SR should be designed to allow the multiplexing with 1 or 2 bits UCI payload size in the same symbol.

Multiplexing of UCIs for different use cases within one UE

There could be a situation when UCIs for different use cases to be sent simultaneously, for example, HARQ-ACK for eMBB and SR for URLLC are simultaneously required to be sent. In this case, how to support multiplexing of UCIs for different use cases should be considered.

eMBB HARQ-ACK might have different duration from URLLC SR assuming to have different latency requirement. In LTE, for HARQ-ACK and SR overlapping when HARQ-ACK is repeated, dropping SR is specified. This is because in LTE, HARQ-ACK is prioritized than SR as DL resource is scarcer and to increase DL throughput is more important. On the other hand, in NR, to support different use cases which have different requirement is necessary. Especially, URLLC SR should not be dropped or de-prioritized than eMBB HARQ-ACK.

Considering above, it should be possible to transmit both UCIs for URLLC and UCIs for eMBB simultaneously with different PUCCH resources. In this case, to transmit both up to available power should be no issue. On the other hand, if it reaches the situation only one is possible to be sent or transmit power should be scaled below maximum transmit power, the priority would be given to UCIs for URLLC than UCIs for eMBB. Dropping rule or transmit power control considering such prioritization should be considered further.

Proposal 3: A rule for sending multiple UCIs simultaneously is necessary, especially considering different type of use cases such as URLLC.

Proposal 4: Dropping rule or transmit power control considering prioritization among different use cases should be considered.

Conclusion

In this contribution, we discussed simultaneous transmission of different UCIs considering different use cases. We have following proposals:

Proposal 1: SR resource (periodicity) of one of configurations should be at least around once per 0.125ms in order to satisfy the latency requirement.

Proposal 2: SR should be designed to allow the multiplexing with 1 or 2 bits UCI payload size in the same symbol.

Proposal 3: A rule for sending multiple UCIs simultaneously is necessary, especially considering different type of use cases such as URLLC.

Proposal 4: Dropping rule or transmit power control considering prioritization among different use cases should be considered.

Reference

[1] R1-1710211 (R2-1705867), “LS on SR design principle in NR,” RAN2, RAN1 NR Ad-Hoc#2

[2] TR 38.913

[3] R1-1713344, “Discussion on 1-symbol NR-PUCCH for UCI of up to 2 bits,” Panasonic, RAN1#90