3GPP TSG RAN WG1 Meeting #90			R1-1713432

Prague, Czech Republic, 21th – 25th August 2017

Agenda item:	6.1.3.2.2.1

Source: 	Qualcomm Incorporated

Title: 	Long PUCCH design with 1 or 2 bits UCI payload

Document for:	Discussion/Decision

# Introduction

In RAN1 89 [1], the following are agreed for long PUCCH.

Agreements:

- Long duration NR-PUCCH for up to 2 bits in a given slot is composed as the followings:

  - HARQ ACK by BPSK or QPSK modulation is repeated in time domain and multiplied with sequence(s)

    - FFS: pi/2 BPSK usage

  - Two states SR is based on on-off-keying

  - Time domain OCC can be applied over multiple UCI/DMRS symbols per frequency hop

Agreements:

- NR supports following long-PUCCH:

  - One PUCCH format for UCI with up to 2 bits with high multiplexing capacity

  - One PUCCH format for UCI with large payload with no multiplexing capacity

- FFS: One PUCCH format for UCI with moderate payload with some multiplexing capacity

  - Note: this could be a variation of one of the former PUCCH formats.

In RAN1 NR Ad-Hoc#2 [2], a few further agreements are agreed for long PUCCH with 1 or 2 bits UCI payload.

Agreements:

- For a long PUCCH with up to 2 bits UCI:

  - Working assumption:

    - DMRS always occurs in every other symbols in the long PUCCH

      - FFS: even or odd symbols

    - The working assumption can be revisited if an issue is found in terms of transient period.

Agreements:

  - The DMRS symbol(s) are formed as follows:

    - DMRS for a PUCCH is a sequence (e.g. a cyclic shift of a CAZAC or computer generated sequence) in frequency domain with Orthogonal Cover Code (OCC) in time.

  - Note: The data symbols are formed as the following:

    - The modulated UCI bit(s) is multiplied with a sequence (e.g. a cyclic shift of a CAZAC or computer generated sequence) in frequency domain with Orthogonal Cover Code (OCC) in time.

      - BPSK and QPSK modulations for 1 and 2 UCI bits, respectively.

  - Note: The usage of OCC for DMRS and UCI symbols is effectively disabled for differentiation of the users that are assigned with the same OCC.

Agreements:

- For a long PUCCH in a slot,

  - At most one hop for the long PUCCH is supported.

    - FFS: details

With the above agreements, the structure of long PUCCH with 1 or 2 bits UCI is basically in place, except that one important piece is missing. The missing piece is the frequency hopping details. In this contribution, we discuss two aspects of frequency hopping. First, we share our view on whether frequency hopping can be disabled. If it can be disabled, it should be disabled on what conditions. Second, if frequency hopping is enabled, what is the symbol position where hopping occurs.

# Enable frequency hopping or not

Frequency hopping is an important aspect for long PUCCH design. Frequency hopping can bring diversity gain in frequency selective fading channel and also bring robustness in case of frequency jammer. However, there is a loss in coherent combining gain in channel estimation comes with frequency hopping. The loss is roughly 3dB if two hops are applied in one slot. This is a trade-off between channel combining gain and frequency diversity gain. Which gain has more impact to PUCCH performance depends on many factors such as hopping distance, channel statistics, SNR region, and PUCCH duration. Theoretical analysis is hard to conduct in this case. We study the topic with a set of simulations.

The scenario we pick to run simulations is a long PUCCH with 4 symbols, where two of them are DMRS symbols and two of them are UCI symbols, as shown in Figure 1. The detailed simulation assumptions are listed in Appendix. The simulation results are shown in Figure 2. The first simulation is with AWGN channel. This is not a channel in reality. But it helps to show the tread of trade-off between channel estimation combing gain and frequency diversity. With AWGN channel, because there is no frequency diversity, it is expected that enable frequency hopping will lead to performance loss, in all SNR region. With EPA channel which has smaller frequency diversity, there is a cross point between disabling and enabling frequency hopping. At low SNR region, disabling frequency hopping has better performance, which is because channel combining gain is more important than frequency diversity in low SNR region. At high SNR region, since channel estimation is relatively good, the extra gain of further channel combing is saturated. Therefore, frequency hopping with diversity starts to outperform channel combining without frequency hopping in high SNR region. With ETU channel which has larger frequency diversity, frequency hopping outperforms disabling hopping in all SNR region.

Figure 1: Structure for 4-symbol long PUCCH with up to 2 bits UCI payload

Figure 2: Performance of 1 bit ACK carried by 4-symbol long PUCCH with and without frequency hopping

Based on the above observations from the simulations results, we have the following proposal

Proposal 1: gNB should be able to enable and disable frequency hopping for long PUCCH.

- FFS semi-static signalling or dynamic signalling to enable and disable hopping.

# Hopping position

Once hopping is enabled, another question is on which symbol the hopping occurs. Intuitively, for best PUCCH performance, one principle for deciding the hopping position should be allow floating hopping position depends on the starting and ending symbol location, such that the hopping can occur roughly at the middle of the long PUCCH duration. However, this principle of hopping in the middle of long PUCCH duration may not lead to best PUCCH performance in certain cases. Such cases could be when long PUCCH has N=4M+2 symbols where M = 1, 2, 3, especially when M=1 which leads to long PUCCH with 6 symbols. With a 6-symbols long PUCCH, there are two reasonable choices of hopping position, as shown in Figure 3. With alternative 1, hopping occurs on symbol 3, which leads to equal length of two parts of PUCCH. However, the first part has two DMRS symbols and 1 data symbol. The second part has 1 DMRS symbol with 2 data symbols. Therefore, in each part, the pilot to data ratio is very different from 50%. As we know, for 1 or 2 bits UCI, the optimal pilot ratio should be 50%. Therefore, alternative 1 does not achieve optimal pilot ratio in both parts, which should lead to some performance degradation for PUCCH. On the other hand, with alternative 2, hopping occurs on symbol 2. Both parts can achieve the optimal pilot ratio. Therefore, we expect alternative 2 yields better performance than alternative 1, which is confirmed by simulation results as shown in Figure 4. The details of simulation assumptions are given in Appendix.

For long PUCCH with other durations where N=4M, N=4M+1, N=4M+3, the hopping in the middle of long PUCCH duration should work fine.

Figure 3: Two alternative hopping positions for a 6-symbol long PUCCH

Figure 4: Performance comparison of two alternative hopping positions for a 6-symbol long PUCCH

Proposal 2: For a long PUCCH with N symbols to carry 1 or 2 bits UCI, if hopping is enabled, the length for the two parts after hopping is determined as following

- If N=4M, the length of part 1 is 2M, while the length of part 2 is 2M

- If N=4M+1, the length of part 1 is 2M, while the length of part 2 is 2M+1

- If N=4M+2, the length of part 1 is 2M, while the length of part 2 is 2M+2

- If N=4M+3, the length of part 1 is 2M+1, while the length of part 2 is 2M+2

# Conclusions

To summarize, we discussed frequency hopping for long PUCCH with up to 2 bits UCI payload.  We have following proposals.

Proposal 1: gNB should be able to enable and disable frequency hopping for long PUCCH.

- FFS semi-static signalling or dynamic signalling to enable and disable hopping.

Proposal 2: For a long PUCCH with N symbols to carry 1 or 2 bits UCI, if hopping is enabled, the length for the two parts after hopping is determined as following

- If N=4M, the length of part 1 is 2M, while the length of part 2 is 2M

- If N=4M+1, the length of part 1 is 2M, while the length of part 2 is 2M+1

- If N=4M+2, the length of part 1 is 2M, while the length of part 2 is 2M+2

- If N=4M+3, the length of part 1 is 2M+1, while the length of part 2 is 2M+2

# Appendix

| Simulation assumptions for Section 2 |  |
| --- | --- |
| Numerology | 30Khz SCS |
| Hopping distance | 168 RBs |
| Long PUCCH duration | 4 symbols (2 DMRS + 2 UCI symbols) |
| # RBs for long PUCCH | 1 RB |
| Payload size | 1 bit |
| # eNB antenna | 2 |
| Channel and noise estimation | MMSE channel estimation and genie noise estimation |

| Simulation assumptions for Section 3 |  |
| --- | --- |
| Numerology | 30Khz SCS |
| Hopping distance | 168 RBs |
| Long PUCCH duration | 6 symbols (3 DMRS + 3 UCI symbols) |
| # RBs for long PUCCH | 1 RB |
| Payload size | 1 bit |
| # eNB antenna | 2 |
| Channel and noise estimation | MMSE channel estimation and genie noise estimation |

# References

- 3GPP TSG RAN WG1 Meeting #89, “RAN1 Chairman’s Notes,” Hangzhou, P.R. China 15th – 19th May 2017.

- 3GPP TSG RAN WG1 NR Ad-Hoc#2, “RAN1 Chairman’s Notes,” Qingdao, P.R. China, 27th – 30th June 2017.