3GPP TSG RAN WG1 Meeting #90							R1-1713943

Prague, Czechia, 21th – 25th August 2017

Source:	NTT DOCOMO, INC.

Title:	Long-PUCCH over multiple slots

Agenda Item:	6.1.3.2.2.3

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss the design for long-PUCCH over multiple slots.

- Long-PUCCH over multiple slots

Long-PUCCH for more than one slot is useful for improving reliability/coverage of a UCI delivery, increasing the UCI payload for a given PUCCH transmission, etc. The use of long-PUCCH with more than a slot should be UE-specific; depending on reliability/coverage requirement or UCI payload necessary to be delivered, the number of slot for long-PUCCH transmission would be different among UEs.

Support of long-PUCCH over multiple slots should be based on the long-PUCCH for one slot. More specifically, following commonalities should be made between the long-PUCCH within one slot and the long-PUCCH over multiple slots:

- PUCCH timing determination (which slot to transmit)

  - For a long-PUCCH within a slot, transmission slot of a PUCCH carrying HARQ-ACK can be indicated by the scheduling DCI or be configured by RRC.

  - For a long-PUCCH over multiple slots, starting slot of a PUCCH carrying HARQ-ACK can be indicated by the scheduling DCI or be configured by RRC.

- PUCCH duration (how many slot(s) to span)

  - For a long-PUCCH within a slot, one slot to be used for long- PUCCH carrying HARQ-ACK is semi-statically configured by RRC.

  - For a long-PUCCH over multiple slots, number of slots to be used for long-PUCCH carrying HARQ-ACK is semi-statically configured by RRC.

Proposal 1:

- Timing between DL data reception and staring slot of the long-PUCCH for its acknowledgement can be indicated by a field in the DCI from a set of values configured by higher-layer.

- The number of slot(s) a long-PUCCH carrying the acknowledgement spans is configured by higher-layer.

The slot structure or usage could be different slot-by-slot. For example, one slot could be UL-centric, while next slot could be UL-only. Besides, depending on whether short-PUCCH/SRS is multiplexed or not, the ending position of a long-PUCCH in each slot should be different. Therefore, the exact structure of long-PUCCH for each given slot within the aggregated slots for a long-PUCCH transmission over multiple slots should be determined per slot basis.

Fig. 1.	Long-PUCCH for more than one slot.

Proposal 2:

- When a long-PUCCH spans more than one slot, the long-PUCCH structure within a slot of the multiple slots should be able to be determined flexibly.

Details of frequency-hopping for a long-PUCCH within a slot is not yet clear. How to perform frequency-hopping for a long-PUCCH over multiple slots should be clarified after the frequency-hopping for a long-PUCCH within a slot is further clear.

Proposal 3:

- How to perform frequency-hopping for a long-PUCCH over multiple slots should be clarified after the frequency-hopping for a long-PUCCH within a slot is further clear.

- Conclusion

Proposal 1:

- Timing between DL data reception and staring slot of the long-PUCCH for its acknowledgement can be indicated by a field in the DCI from a set of values configured by higher-layer.

- The number of slot(s) a long-PUCCH carrying the acknowledgement spans is configured by higher-layer.

Proposal 2:

- When a long-PUCCH spans more than one slot, the long-PUCCH structure within a slot of the multiple slots should be able to be determined flexibly.

Proposal 3:

- How to perform frequency-hopping for a long-PUCCH over multiple slots should be clarified after the frequency-hopping for a long-PUCCH within a slot is further clear.

References

- 3GPP RAN1#88bis Chairman’s note.

- 3GPP RAN1#89 Chairman’s note.