3GPP TSG RAN WG1 #90	 R1-1714217

# Prague, Czech Republic, 21-25 Aug 2017

Source:	Motorola Mobility, Lenovo

Title:	Discussion on higher rank Type II codebook and feedback overhead reduction

Agenda Item:	6.1.2.2.3

Document for:	Discussion and Decision

1 Introduction

In the RAN1 #89 meeting in Hangzhou, Type II channel state information (CSI) codebook was agreed for supporting full-dimension (FD) multiple-input multiple-output (MIMO) as follows [1,2].

Agreements: - Slides 4 to 24 in R1-1709232 are agreed - For slide 20, FFS whether or not support frequency-dependent parameterization and if so, the details - FFS whether or not to further enhance analog beamforming related operations especially for >1 layers

In this contribution, we discuss Type 2 CSI feedback in a system with 4 Rx UEs.  Simulation results of such a system are presented both the current rank 1-2 codebook and a rank 3-4 extended version of this codebook.  A framework for a high resolution CSI codebook which is suitable for higher rank transmission but which uses beam space compression to reduce total overhead is also presented.

2 Type II Codebook – Review

In the current Type II CSI agreement in [1,2], a channel matrix consisting of channel vectors for W total subcarriers

is compressed in wideband (WB) such that

by exploiting the set of L selected discrete Fourier transform (DFT) beams, i.e., B∶=b1,⋯,bL∈CNtx×L.  Note that Ntx is the number of transmit antenna ports and Nrx is the number of receive antenna ports for each polarization. For the rest of this contribution, we call the compressed channel matrix as a beam space channel matrix. The channel compression procedure in WB is effective for quantizing high resolution CSI because the realistic MIMO channels are spatially correlated and the channel matrix consists of only a limited number of dominant (meaningful) propagation paths.

After constructing the beam space channel matrix in WB, the 2L-dimensional right dominant singular vector of beam space channel matrix in the s-th subband (SB),

is quantized in each SB to generate a precoding vector for each rank, where S denotes the number of SBs. Quantizing the 2L-dimensional vector is a layer and SB specific process so that total feedback overhead required for SB CSI reporting would increase proportionally to the number of maximum transmission layers and the number of SBs.

3 Type II Codebook for Higher Rank CSI

In order to fully exploit the benefits of multiuser (MU) FD-MIMO systems, it is necessary to support rank > 2 MU transmission based on high resolution CSI. One straightforward solution is to extend the Type II codebook based on the codebook design principle agreed in [1,2]. To compute rank 3 and rank 4 precoding vectors in the extended Type II codebook, we make the same quantization approach with that for rank 1 and rank 2 precoding vectors in the current Type II codebook. To be more specific, 2L-dimensional vectors should be quantized in each SB for computing rank 3 and 4 precoding vectors.

It is expected that the extended rank > 2 Type II CSI will increase system performance gain considerably [3,4]. We thus present simulation results to verify data rate performance (bits/s/Hz/user) of the higher rank Type II CSI (rank 1-4) in the 3D UMi scenario and in a 32 port (4,4,2) configuration. For system level simulations, we consider a non full buffer traffic scenario with 50% target resource utilization (RU). The rest of simulation parameters are listed in Table 2. In Fig. 1, average user packet throughputs (UPTs) are plotted for the Type II CSI (rank 1-2) and the higher rank Type II CSI (rank 1-4). Four Rx antennas were used in both cases.

Fig. 1: Performance comparison of different CSI feedback schemes H,VxN1,N2,P=4,8x(4,4,2)

These results indicate that significant performance gain, 14.8% in average UPT, and 9.5% in cell edge UPT, can be obtained from rank 3-4 multiplexing above that provided by improved spatial diversity and array gain when UEs employ 4 receive antennas. Therefore if 4 Rx UE operation is to be supported it is desirable that Type II codebooks for ranks 3 and 4 be included in the specification. While gains shown in Fig. 1 for Type II Rank 1 – 4, the associated feedback overhead of the extended Type II scheme is large as will be described below.

Observation 1: Higher rank Type II CSI (rank 1-4) will increase system performance gains considerably at the cost of huge total feedback overhead.

It is desirable therefore to investigate how feedback overhead can be reduced while retaining the increase throughput from rank 3-4 multiplexing.

4 Beam Space Channel Matrix Compressions

Before discussing CSI design framework suitable for higher rank transmission, we first summarize total feedback overhead required for the extended Type II CSI reporting. As shown in Table 1, if we simply extend the Type II codebook, the total feedback overhead for higher rank Type II (rank 1-4: T=4) SB CSI reporting would be twice that for Type II (rank 1-4: T=2) SB CSI reporting. A simple extension of the current Type II codebook would cause a huge burden on feedback links since feedback overhead would increase proportionally to the maximum transmission rank T. Therefore, Type II CSI codebook supporting higher rank transmission scenario should be designed carefully by considering limited feedback resources.

Table 1: Feedback overhead calculations

The maximum number of transmission layers T and the number of SBs S are fixed in system and therefore these system parameters cannot be controlled for the purpose of reducing CSI feedback overhead. To reduce feedback overhead, one possible solution is to modify the number of basis vectors L. Reducing the number of basis vectors is in line with compressing the beam space channel matrix additionally in WB such that

,

where A∈C2L×2L  denotes the channel compression matrix and L<L  denotes the channel compression level.

Proposal 1: Study channel compressing patterns A and channel compression levels L for developing rank 3 and rank 4 Type II CSI codebook.

5 Conclusions

CSI design framework suitable for higher rank transmission is discussed for new radio (NR). The proposals and observations discussed in this contribution are summarized as follows.

Observation 1: Higher rank Type II CSI (rank 1-4) will increase system performance gains considerably at the cost of huge total feedback overhead.

Proposal 1: Study channel compressing patterns A and channel compression levels L for developing rank 3 and rank 4 Type II CSI codebook.

References

- R1-1709232, WF on Type I and II CSI codebooks, Samsung

- 3GPP, RAN1#89, Chairman’s Notes

- R1-1710674, On higher rank (rank 3 and 4) Type II CSI, Samsung

- R1-1710454, The remaining issues for Type II codebook, Huawei, HiSilicon

Appendix A: Simulation Assumptions

Table 2: Simulation Parameters

| Channel model | 3D UMi 200m ISD |
| --- | --- |
| eNB height | 10 m |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Oversampling (O1,O2) | (4,4) |
| eNB antenna configurations H,Vx(N1,N2,P) | 32 ports: 4,8x(4,4,2) with 1x2 virtualization, 108° tilt |
| UE antenna configurations | 4 UE antennas: (2,1,2), dv=0.8 λ |
| Antenna polarizations | eNB (+45°,-45°) UE (0°, 90°) |
| eNB antenna spacings (dh,dv) | (0.5,0.8)λ |
| eNB power | 41 dBm (UMi) |
| Cell layout | 57 cells |
| Traffic model | FTP Traffic Model 1 Non full buffer (50% Target RU, 500kB packet size) |
| Scheduling | MU, Proportional fair, Max 8 MU layers |
| HARQ | Max 4 retransmission |
| CSI periodicity and CSI delay | 5 ms |
| UE speed | 3 km/h |
| Target BLER | 0.1 |
| Receiver | MMSE – IRC |
| Transmission rank | 1 – 4 |