3GPP TSG-RAN WG1 #90	R1-1714278

Prague, Czechia, 21th – 25th August, 2017

Source:	Ericsson

Title:	On transmission parameter sets

Agenda Item:	6.1.2.1.8

Document for:	Discussion and Decision

# Introduction

In RAN1#87 meeting, the following agreement was made

- Define at least two sets of transmission parameters, where

  - Transmission parameter set 1: parameters configured (FFS: L1 or L2 or L3)

    - For default transmission scheme, specify default values of parameters in the Transmission parameter set 1

      - FFS: Whether default value can be derived implicitly

    - Note that depending on parameter settings in transmission parameter set 1, the size of transmission parameter set 2, i.e. DCI size, may vary.

  - Transmission parameter set 2: parameters indicated by physical layer (e.g. NR PDCCH channel)

    - FFS whether multiple subsets is supported and how to simplify DCI format

  - Note: some transmission parameter may belong to both set-1 and set-2

  - Detail parameters and usage condition are TBD

In this contribution, we discuss the aspects related to transmission parameter sets.

# Parameter sets

A PDSCH or PUSCH transmission is configured by transmission parameters which may be RRC configured or may be indicated dynamically, by scheduling DCI. In our view, these correspond to set 1 and 2 respectively, according to the previous agreement.

A PDSCH or PUSCH transmission is defined by transmission parameters divided into two sets where transmission parameter set 1 can be obtained by RRC configuration and transmission parameter set 2 can be obtained by DCI signalling.

- Before higher layer configuration has taken place, the UE can still transmit/receive PUSCH/PDSCH by using the default parameter setting.

Examples of parameters in set 1 that are obtained by higher layer configuration are the following. Note that these are preliminary examples and additional fields or merging of fields may also be possible.

| RRC configured parameter | Values | Default value | Notes |
| --- | --- | --- | --- |
| DMRS type for DL | Type 1 or 2 | 1 |  |
| DMRS type for UL | Type 1 or 2 | 1 |  |
| DMRS scrambling for DL | integer(0…TBD) |  | Four different seed values to choose from in DCI |
| DMRS scrambling for UL | integer(0…TBD) |  | Four different seed values to choose from in DCI |
| DMRS symbol group size for DL | 1 or 2 | 2 | Number of front loaded DMRS symbols |
| DMRS symbol group size for UL | 1 or 2 | 2 | Number of front loaded DMRS symbols |
| DMRS port grouping | ON/OFF | OFF | Whether DMRS port grouping is used or not (to enable multi-TRP transmission) |
| DMRS additional | ON/OFF | ON | Whether additional DMRS is used |
| DMRS high speed | ON/OFF |  | Whether HS-DMRS pattern is used |
| PTRS presence in DL | ON/OFF |  | To configure whether PTRS is present or not |
| PTRS presence in UL | ON/OFF |  | To configure whether PTRS is present or not |
| Resource allocation type | 0 or 1 |  |  |
| PRB bundling sizes |  |  | To configure two PRB bundling sizes that can be dynamically selected by DCI |
| TPMI presence | ON/OFF |  | Whether TPMI is indicated in DCI or not |
| SRI presence (for UL) | ON/OFF |  | Whether SRI is indicated in DCI or not |
| BPL indicator presence | ON/OFF |  | Whether BPL Indicator is present in DCI or not |
| QCL parameter set configuration |  |  | QCL related configuration to support multi-TRP operation |
| TRS periodicity | {20,40,80,160} ms |  |  |
| TRS slot offset |  |  | Note that the UE can be configured with multiple TRSs |
| TRS resource configuration | 1,2,3,.. |  | We may need several different TRS burst structures |
| TRS comb |  |  | Frequency shift (comb selection) |
| TRS bandwidth |  |  |  |
| TRS scrambling identity |  |  |  |

Parameters in set 2 for PDSCH and PUSCH transmissions obtained by bit fields in DCI are the following (note that this table does not represent all fields in the DCI but only those associated with the data transmission). Note that these are preliminary examples and additional fields or merging of fields may also be possible.

| RRC configured parameter | Values | Default value | Notes |
| --- | --- | --- | --- |
| TRI | 1-8 (DL) or 1-4 (UL) |  | Rank |
| TPMI (for UL) |  |  | Precoding matrix |
| SRI |  |  | SRS indicator for UL scheduling |
| BPL | 1,2,3,4 |  | Indicates the beam pair link(s) (i.e. spatial QCL) used for data scheduling |
| Resource block assignment including PRBs and starting and ending positions |  |  | Selects a pre-configured time resource allocation (including number of slots) and selects PRBs |
| MCS |  |  |  |
| Redundancy version |  |  | For LDPC code |
| DMRS FDM with data | ON/OFF |  | Whether data should be mapped to non-DMRS REs in OFDM symbols containing DMRS |
| DMRS antenna ports |  |  | From the RRC configured DMRS set |
| Carrier Indicator | 1,2,3,.. |  | For CA |
| DMRS sequence initialization | 1,2,3 or 4 | 1 | For e.g. non-orthogonal MU-MIMO |
| Antenna ports |  |  | From the set of RRC configured DMRS ports |
| QCL parameter set selection | 1,2,3.. | 1 | To support dynamic TRP switching, selection among higher layer configured sets |

# Conclusion

Discuss further the RRC parameters of parameter sets 1 and 2 and their corresponding default values (when applicable). Note that the discussion of DCI content will take place in Scheduling agenda but from MIMO discussion perspective we need to have a clear view on what parameters are necessary for DCI as the input to the other agenda.