3GPP TSG RAN WG1 Meeting #90	R1-1714568

Prague, Czech, 21th - 25th August 2017

Agenda item:	6.2

Source:	Samsung

Title:  	Impact of channel estimation error on the performance of high spectral efficiency MIMO schemes

Document for:	Discussion

# Introduction

In RAN1#88 meeting, the following observations are related to enhanced modulation schemes [1].

Observations:

- RAN1 has studied following modulation schemes, but has not reached conclusions so far:

  - Higher order modulation e.g. 1024QAM

  - Shaped Modulation

  - Spatial Modulation

  - Diversity enhancing modulation schemes

  - Coded modulation and bits-to-symbol(s) mappings

  - APSK

  - Interpolated QPSK modulation

Spectral efficiency is an important key performance indicator (KPI) for NR. To improve spectral efficiency, the support of up to 256 QAM and spatial multiplexing for 8 layers has been proposed [2]. This enhancement approach of spectral efficiency is in fact highly related to receiver implementation. In LTE, conventional multiple-input multiple-output (MIMO) linear receivers, such as zero-forcing (ZF) or minimum mean-square error (MMSE) receivers, are assumed. For NR, we may consider the support of non-linear receivers, such as maximum likelihood (ML) and sphere decoding receivers, to provide enhanced performance, but their implementation may not be practical due to their high complexity especially when the number of layers is larger and/or the modulation order is high.

Even though this is an important aspect for high spectral efficiency, coded modulation schemes and bit-to-symbol mappings for high order modulation in conjunction with MIMO are not as yet sufficiently studied and compared with each other in Phase I. In this contribution, we will provide the link level performances of the proposed MIMO scheme, MMSE, and MMSE codeword-level SIC based on realistic channel estimation assumption.

# Performance comparison under practical channel estimation

In our performance comparison, the following receiver and modulation schemes are taken into account:

- Scheme 1: The proposed MIMO scheme with MLC and natural mapping

- Scheme 2: MMSE with BICM and Gray mapping

- Scheme 3: MMSE codeword-level SIC with BICM and Gray mapping

We assume a 4Tx-4Rx MIMO configuration and horizontal mapping: within one OFDM symbol, a codeword is first mapped across frequency domain (sub-carriers) before it is mapped across MIMO layers. Each Tx antenna sends independently encoded code blocks. Hence, for the MMSE-SIC case, SIC across encoded code blocks is assumed to be performed three times. In addition, as a practical channel estimator, the receiver is assumed to employ a least square (LS) channel estimation and linear interpolation. Other detailed evaluation assumptions and parameters are shown in Annex.

Figure 1 compares the BLER performance of schemes 1, 2, and 3 in the TDL-A channel model with short delay spread at 64-QAM and code rate=0.75 under ideal and practical channel estimation assumptions. In Figure 1, it is shown that the proposed scheme achieves SNR gains of 9 dB and 1 dB over MMSE and MMSE-SIC at BLER of 10-1 under the ideal channel estimation, respectively. As for the practical channel estimation, the proposed scheme provides gains of more than 10 dB and 1.5 dB over MMSE and MMSE-SIC, respectively. Hence, it is observed that the performance degradations due to imperfect channel estimation are similar for the proposed scheme and MMSE-SIC, and thus the performance gap obtained under the ideal assumption remains the same for the practical channel estimation assumption. Moreover, it is also observed that the slope of the proposed scheme is much steeper than those of MMSE and MMSE-SIC for both ideal and practical channel estimation assumptions.

Similar tendency can be observed for 256-QAM transmission. Figure 2 compares the BLER performance of schemes 1, 2, and 3 in the TDL-A channel model with short delay spread at 256-QAM and code rate=0.75 under ideal and practical channel estimation assumptions. In Figure 2, it is shown that the proposed scheme achieves SNR gains of 10 dB and 1.8 dB over MMSE and MMSE-SIC at BLER of 10-1 under the ideal channel estimation, respectively. As for the practical channel estimation, the proposed scheme provides gains of more than 10 dB and 4 dB over MMSE and MMSE-SIC, respectively.

We remark that the amount of SNR degradation due to imperfect channel estimation may depend on the channel estimation strategy employed at the receiver, but the tendency would be similar regardless of specific channel estimation schemes.

Figure 1: Performance comparison of MMSE, MMSE-SIC, and the proposed scheme in the 4x4 MIMO TDL-A channel model with short delay spread under ideal and practical channel estimation assumptions: BLER vs. averaged SNR for 64-QAM and code rate=0.75.

Figure 2: Performance comparison of MMSE, MMSE-SIC, and the proposed scheme in the 4x4 MIMO TDL-A channel model with short delay spread under ideal and practical channel estimation assumptions: BLER vs. averaged SNR for 256-QAM and code rate=0.75.

Observation 1: The performance degradations due to imperfect channel estimation are similar for the proposed scheme and MMSE-SIC.

Observation 2: The modulation scheme consisting of MLC and natural mapping provides high performance gain compared to the modulation scheme consisting of BICM and Gray mapping for MIMO transmission for both ideal and practical channel assumptions.

# Conclusion

This contribution discussed the impact of channel estimation error on the performance of high spectral efficiency MIMO schemes, including our proposed MIMO scheme with MLC and natural mapping. Our observations are made as follows:

Observation 1: The performance degradations due to imperfect channel estimation are similar for our proposed scheme and MMSE-SIC.

Observation 2: The modulation scheme consisting of MLC and natural mapping provides high performance gain compared to the modulation scheme consisting of BICM and Gray mapping for MIMO transmission both for ideal and practical channel assumptions.

# 4 	References

- Chairman’s Notes, RAN1 #88.

- RP-160664, “New work item on uplink capacity enhancements for LTE,” Ericsson, CMCC, RAN1 #71.

- TR 38.900 V14.0.0, “Study on channel model for frequency spectrum above 6 GHz (Release 14),” Jun. 2016.

# Annex

Table 1: Evaluation assumptions and parameters for link-level simulations.

| Parameter | Assumption |
| --- | --- |
| Waveform | OFDM |
| Carrier frequency | 4 GHz |
| System bandwidth | 10 MHz |
| Allocated RBs | 5 RBs |
| Channel model | TDL-A with short delay spread [3] |
| UE mobility | 3km/h |
| Pilot pattern | LTE CRS |
| Channel estimation | Ideal/Realistic (LS estimator + interpolation) |
| Modulation | 64/256 QAM |
| Channel code | LTE Turbo code (rate = 0.75) |
| MIMO precoding | No precoding (i.e. identity matrix) |