3GPP TSG RAN WG1 #90                                   	          								R1-1715175

Prague, Czech Republic, 21st -25th August 2017

Agenda item:		6.1.4.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation of Polar code uplink FAR

Document for:		Discussion and Decision

# 1 	Introduction

In this contribution, the FAR of uplink CA-Polar is discussed.

# 2 	Discussion

The Polar code uplink FAR is evaluated with the following simulation assumptions

Table I: Simulation parameters

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Coding Scheme | CA-Polar |
| Polar code sequence | new PW |
| Rate matching | Unified rate-matching [1] |
| Decoding algorithm | CRC aided List-8 (CRC used after decoding the full info block to select the path) |
| Info. block length (bits w/o CRC) | K=23:1:200 |
| Code rate | 1/12, 1/6, 1/5, 1/4, 1/3, 1/2, 2/3, 3/4, 4/5, 5/6 |
| CRC bits | 11 bit CRC, 0x9EB* |

The uplink FAR spectrum of CA-Polar is shown in Figure 1, showing the peak FAR of different code rates for block size K. The required FAR is around 2^-8 ≈ 0.0039. However, it is observed that the FAR may increase with the code rates and some block sizes. The maximum FAR for many block sizes double the requirement. Figure 1 shows there is a trend that the FAR may increase with the block size.

Figure 1. FAR spectrum of CA-Polar

Observation 1: High FAR is a common issue of CA polar in uplink, especially for high code rates and larger block sizes.

It is not recommended to adopt CA-Polar for UL and DCRC seems to be the only way to overcome such FAR concerns.

# 3	Conclusion

In this contribution, we provided the evaluation of the uplink FAR of CA-Polar.

Observation 1: High FAR is a common issue of CA polar in uplink, especially for high code rates and larger block sizes.

# References

- R1-1710825 "Optimization and comparison of Polar code rate-matching design", MediaTek