3GPP TSG RAN WG1 NR Meeting 90bis		R1- 1717487

Prague, Czech Republic, 9th – 13th, October 2017

Source:	vivo

Title:	DCI contents and design

Agenda Item:	7.3.1.5

Document for:	Discussion and Decision

- Introduction

This is a revised contribution from R1-1715633.

Downlink control information (DCI) is used in LTE for downlink grant, uplink grant, power control, random access procedure response, etc. It is expected similar design framework shall be adopted in NR. Some of the aspects may need to be changed according to the introduction of new physical layer designs and features. An email discussion concerning some potential DCI contents has been done and summarized in [1]. This contribution addresses these issues, especially on the DCI for DL/UL grants aspects. Besides, the DCI contents and relevant formats are discussed in this contribution.

- Discussion

  - Legacy DCI fields

In LTE, multiple formats were defined for the sake of supporting common and UE-specific DCI, which would be a pretty well starting point for NR design. In LTE, the DCI fields can generally be divided into several groups:

- Resource assignment, including DL, UL or sidelink physical resource block assignment, modulation and coding scheme.

- Spatial multiplexing related information, including precoding information, antenna ports, scrambling identity and number of layers, PDSCH RE mapping and QCL configuration, cyclic shift/OCC index for DMRS.

- HARQ related information, including HARQ processing number, new data indicator (NDI), redundancy version, and acknowledgement resource.

- Power control command, including UE-specific PUCCH and PUSCH power control, or group power control addressing for a group of UE at one shot.

- Aperiodic request command for SRS transmission and CSI measurement.

- Other specific feature request, such as carrier indicator, frequency-hopping flag, and downlink assignment index, etc.

Some fields are no longer needed in NR. For example, precoding information is not necessary due to the removal of CRS in NR. On the other hand, other fields, such as resource assignment, HARQ, spatial multiplexing and power control related information, aperiodic request command, are still essential or at least beneficial for NR.

Observation 1: Many of the DCI information fields from LTE are still essential or at least beneficial for NR.

Therefore, it is proposed that,

Proposal 1: At least the DCI information fields of resource assignment, HARQ, spatial multiplexing and power control related information, aperiodic request command, should be included in NR.

  - Potential NR DCI field

Some possible candidates from our perspective that acknowledge to the agreements include,

- indication of additional DMRS

Additional DMRS, as a complement of normal DMRS pattern, is introduced in order to assist the UE demodulation in tackling the Doppler shift or frequency error in some case. It is important for the network to turn on and off the additional DMRS dynamically, so that unnecessary overhead can be avoided while demodulation performance is preserved. A dynamic indication for the additional DMRS in DCI well meets such requirement and should be adopted in NR. Such an indication may be a separate one-bit DCI field, or be jointly coded with other field; either way may serve our needs and which is better may depend on the detailed design of additional DMRS.

Proposal 2: Indication of additional DMRS for PDSCH demodulation should be included for NR DCI.

- activation/deactivation of the BWP

It is agreed in RAN1 #90 that the activation/deactivation of DL and UL bandwidth part by explicit indication at least in DCI. One or multiple bandwidth part configurations for each component carrier can be semi-statically signaled to a UE (UE can only activate one bandwidth part for a given time instant), and then the DCI signals the activation/deactivation to UE to timely switch to the target BWP. If the number of BWP is configured as N by RRC signaling, the payload of DCI will be log2(N) bits to indicate the index of BWP.

Proposal 3: The BWP activation/deactivation should be added in NR DCI.

In addition to these fields, there are some other potential information may be needed, for example, the indication of beam index for PDSCH/PUSCH, the assigned PDSCH/PUSCH numerology, hopping position information for long PUCCH, etc. Mechanism for network to dynamically manage the UE soft buffer in order to achieve lower minimum coding rates and IR improvements, which may result in additional DCI modification, can also be considered.  However, they are still under discussion on related topics, therefore, it may be too early to make a decision right now.

  - Estimated size of DCI

According to the email discussion about DCI formats, we give some reference number of bits for each DCI field. The LTE DCI could be the starting point.

- MCS field

In LTE, it supports up to 256QAM in MCS table containing 32 values. Based on channel coding discussion, the modulation order may be extend to 1024QAM. If it uses 5bits (32 values) in DCI to indicate MCS, the flexibility of NW may be limited in support of 1024QAM. On the other hand, if it uses 6bits in DCI, the range of MCS could be extended to up to 64 so that 1024QAM can be supported. Nevertheless, the overhead of MCS in DCI is increased in this case, although UE may not use all the MCSs simultaneously. As discussed in [4] multiple MCS tables for different kinds of services should be supported. Therefore, it is proposed that the MCS field can be kept to 5bits.

- Scheduling timing information for PDSCH/PUSCH

According to the agreement of RAN1 #90 meeting in [5], the type of scheduling can divide into three cases: one-slot case, multi-slot case and non-slot (i.e., mini-slot) case. The payload size for each case is calculated and presented in the following table.

| Payload(bits) |  | Starting symbol | Ending symbol | Starting slot | Ending slot | Total | comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| One-slot | Cross-slot | 2 | 2 | 2 | 0 | 6 | Assumption: crossing maximum 4 slots |
|  | Non cross-slot | 2 | 2 | 0 | 0 | 4 |  |
| Multi-slot | Cross-slot | 2 | 2 | 2 | 2 | 8 | Assumption: maximum number of aggregated slots is 4 |
|  | Non cross-slot | 2 | 2 | 0 | 2 | 6 |  |
| Non-slot | Non cross-slot | 4 | 4 | 0 | 0 | 8 |  |

Note: The maximum number of aggregated slots is selected so that four RVs can be scheduled by a single DCI (for TTI bundling, URLLC repetition, etc.).

If the starting/ending symbol and starting/ending slot are semi-statically configured for the UE, DCI indicates a pointer to the set.

For one-slot case, if the cross-slot scheduling applied, there are totally 4*4*4=64 entries, yielding a field of 6bits. If the cross-slot scheduling is not supported, there are totally 4*4=16 entries, yielding a field of 4 bits.

For multi-slot case, if the cross-slot scheduling applied, there are totally 4*4*4*4=256 entries, yielding a field of 8bits. If the cross-slot scheduling is not supported, there are totally 4*4*4=64 entries, yielding a field of 6 bits.

For non-slot case, there are totally 14*14=196 entries, yielding a field of 8bits.

Therefore, the DCI contents for time domain PDSCH/PUSCH allocation can be up to 8bits.

DL-related DCI field show below:

| Field | bits | Notes |
| --- | --- | --- |
| Carrier indicator | 3 | For cross-carrier scheduling fixed to 3 bits. |
| DL BWP indication | log2(N) | N is the number of configured DL BWPs |
| Frequency-domain PDSCH resources - PRBs | NRBDLP | Resources in the frequency domain, NRBDLis the number of RBs, and P is the RBG size. It’s DL Type 0 RA. |
| Time domain PDSCH resources | 8 | Resources in the time domain, including cross-slot scheduling |
| PDSCH/PDCCH resource sharing | 1~? | Indicates whether the PDSCH is mapped to the semi-statically configured resource sets or not. If have multiple resource sets, it needs multiple bits. |
| Bundling size | 1 | Indicates the PRB bundling size |
| MCS | 5 | Modulation and coding scheme per TB. |
| New data indicator | 1 | For one TB, multiple bits if we have multiple TBs |
| Redundancy version | 2 | For one TB, multiple bits if we have multiple TBs |
| HARQ process number. | 3~4 | HARQ process number |
| CBGFI | 0~1 | CBG flush indication. Consists of 1 bit (for one CW) if configured. |
| CBGTI | N | Indicates the CBG(s) (re)transmitted. Consists of N bits bitmap if CBG is configured. |
| PUCCH power control | 2 |  |
| PUCCH resource indicator | 8~? | Indicates the PUCCH resource allocation, including the time-domain information and possibly the resource offset. |
| Antenna port(s) | 5 | The antenna ports to receive, if the antenna ports field jointly encoded with number of layers, its field could be 7 bits. |
| number of layers | 3 | The number of layers to receive |
| QCL indication | 3 | Beam indication to indicate QCL assumption between DL RS antenna port(s) and DMRS antenna port(s) of DL data channel at least w.r.t. spatial QCL parameter |
| SRS triggering request | 1 | To trigger an SRS transmission in the uplink. |
| RNTI | 16 | Identity of the UE (or group of UEs) |

UL-related DCI field show below:

| Field | bits | Notes |
| --- | --- | --- |
| Carrier indicator | 3 | For cross-carrier scheduling, fixed at 3 bits. |
| UL BWP indication | log2(N) | N is the number of configured UL BWPs |
| Frequency-domain PUSCH resources - PRBs | NRBUL/P) | Resources in the frequency domain. It is UL type0 RA. |
| Time domain PUSCH resources | 8 | Resources in the time domain, including cross-slot scheduling |
| MCS | 5 | Modulation and coding scheme per TB. |
| New data indicator | 1 | For one TB, multiple bits if we have multiple TBs |
| Redundancy version | 2 | For one TB, multiple bits if we have multiple TBs |
| HARQ process number. | 3~4 | HARQ process number |
| CBGTI | N | Indicates the CBG(s) to (re)transmitted. Consists of N bits bitmap if CBG is configured. |
| PUSCH power control | 2 |  |
| SRS resource indicator (SRI) | 3 | To indicate a reference SRS resource(s) from a previous bundle of transmitted SRS resources. The UE should use the precoder on the indicated reference SRS resource(s) for PUSCH. FFS if subband SRI is supported. |
| wideband and/or subband TPMI | ~10 | To select a transmit precoding matrix from the UL codebook |
| Transmission Rank Indicator | 2 |  |
| Antenna port(s), scrambling identity (FFS) and number of layers | 3 |  |
| CSI report triggering request, | 3 | To trigger a CSI report |
| CSI measurement request | 3 | CSI-RS indication (including IMR) |
| SRS triggering request | 3 | To trigger an SRS transmission in the uplink. |
| RNTI | 16 | Identity of the UE (or group of UEs) |

- Conclusion

In the contribution, we have some investigations on the DCI content and design, and observe that,

Observation 1: Many of the DCI information fields from LTE are still essential or at least beneficial for NR.

Based on these observations, we propose that,

Proposal 1: At least the DCI information fields of resource assignment, HARQ, spatial multiplexing and power control related information, aperiodic request command, should be included in NR.

Proposal 2: Indication of additional DMRS for PDSCH demodulation should be included for NR DCI.

Proposal 3: The BWP activation/deactivation should be added in NR DCI.

References

- R1-1716581, “Summary of e-mail discussion on DCI contents”, Ericsson, September 2017.

- TR 38.802, “Study on new radio access technology Physical layer aspects”, V14.2.0.

- R1-1717504, “Remaining details for bandwidth part operation”, vivo, October 2017.

- R1-1717497, “On DL/UL resource allocation”, vivo, October 2017.

- “RAN1#90 Chairman Notes”, Prague, Czech Republic, 21st – 25th August 2017.