3GPP TSG-RAN WG1 #90bis	R1-1717852

Prague, Czech Republic, 9th – 13th October 2017

Source:	Ericsson

Title:	Baseline evaluation results for UMa-AV

Agenda Item:	6.2.7.1

Document for:	Discussion and Decision

# Introduction

In RAN#75, the study item on enhanced LTE support for aerial vehicles was approved [1]. The objective of the study is to investigate various RAN1 and RAN2 aspects associated with using terrestrial LTE networks to provide connectivity to aerial vehicles. In RAN1#88bis, RAN1#89, and a follow-up email discussion, most evaluation assumptions were agreed for evaluating the performance of using LTE network deployments with base station antennas targeting terrestrial coverage to serve low altitude aerial vehicles (a.k.a., drones). In this contribution, we present the baseline throughput results for UMa-AV.

# UMa-AV throughput statistics

In this section, we present evaluation results on UMa-AV packet throughput for data traffic including

- DL 5%, 50%, 95% and mean packet throughput statistics for terrestrial UEs Data traffic

- DL 5%, 50%, 95% and mean packet throughput statistics of aerial UEs Data traffic

The following agreed simulation cases are evaluated.

- Case 1: 0 aerial UE per sector for reference

- Case 3: 1 aerial UE per sector

- Case 4: 3 aerial UEs per sector

- Case 5: 5 aerial UEs per sector

In all cases, there are 15 UEs per sector including aerial UEs.

Two baseline scenarios are considered: Case 1 with 20% resource utilization (RU), Case 1 with 50% RU.

## UMa-AV downlink

Table 1 presents evaluation results on UMa-AV downlink packet throughput for data traffic.  In Table 1, throughput statistics are separately presented for Terrestrial UEs and Aerial UEs.  Note that in Terrestrial UEs results, case 1 is chosen as the baseline for comparison. In Aerial UEs results, case 3 is chosen as the baseline for comparison.

Table 1: Throughput statistics for UMa-AV downlink.

The terrestrial UE throughput and aerial UE throughput comparisons are summarized in Figure 1 and Figure 2, respectively.  The case numbers are shown in the horizontal axis.  As we can see from Table 1, for the same offered traffic across the 4 cases, increasing the ratio of aerial UEs in general leads to decreased DL packet throughput for both terrestrial UEs and aerial UEs.  The following are observed from these results:

In UMa-AV downlink, the impact of aerial traffic on terrestrial UE throughput in Case 3 is low under low offered traffic (i.e., 20% RU in Case 1) and notable in high offered traffic (i.e., 50% RU in Case 1)

- 5% throughput loss:  ~2% under low offered traffic and ~31% under high offered traffic

- mean throughput loss:  ~1% under low offered traffic and ~12% under high offered traffic

In UMa-AV downlink, the impact of aerial traffic on terrestrial UE throughput in Cases 4-5 is marginal under low offered traffic (i.e., 20% RU in Case 1) and high in high offered traffic (i.e., 50% RU in Case 1)

- 5% throughput loss:  less than 15% under low offered traffic and more than 42% under high offered traffic

- mean throughput loss:  less than 7% under low offered traffic and more than 23% under high offered traffic

In UMa-AV downlink, the aerial UE throughputs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

Figure 1: Downlink terrestrial UE throughput results for UMa-AV.

Figure 2: Downlink aerial UE throughput results for UMa-AV.

## UMa-AV Uplink

Table 2 presents evaluation results on UMa-AV uplink packet throughput for data traffic.  In Table 2, throughput statistics are separately presented for Terrestrial UEs and Aerial UEs.  For these results, we assume a power control setting of P0 = -85 dBm and alpha = 0.8.  Note that in Terrestrial UEs results, case 1 is chosen as the baseline for comparison. In Aerial UEs results, case 3 is chosen as the baseline for comparison.

The terrestrial UE throughput and aerial UE throughput comparisons are summarized in Figure 3 and Figure 4, respectively.  The case numbers are shown in the horizontal axis.

Table 2: Throughput statistics for UMa-AV uplink.

As we can see from Table 2Table 1, for the same offered traffic across the 4 cases, increasing the ratio of aerial UEs in general leads to decreased UL packet throughput for both terrestrial UEs and aerial UEs.  The following are observed from these results:

In UMa-AV uplink, the impact of aerial traffic on terrestrial UE throughputs in Case 3 is low under low offered traffic (i.e., 20% RU in Case 1) and marginal under high offered traffic (i.e., 50% RU in Case 1)

- 5% throughput loss: no loss under low offered traffic and ~8% under high offered traffic

- mean throughput loss: ~1% loss under low offered traffic and ~5% under high offered traffic

In UMa-AV uplink, the impact of aerial traffic on terrestrial UE throughputs in Cases 4-5 is marginal under low offered traffic (i.e., 20% RU in Case 1) and high under high offered traffic (i.e., 50% RU in Case 1)

- 5% throughput loss:  less than 16% under low offered traffic and higher than 26% under high offered traffic

- mean throughput loss: less than 9% under low offered traffic and higher than 13% under high offered traffic

In UMa-AV uplink, the aerial UE throughputs are higher than those of terrestrial UEs.  This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

Figure 3: Uplink terrestrial UE throughput results for UMa-AV.

Figure 4: Uplink aeriial UE throughput results for UMa-AV.

From the results, we make the following proposal:

Capture the results presented in this contribution in the TR 36.777

# Conclusions

In this contribution, we presented the baseline throughput results for UMa-AV.  We made the following observations:

Observation 1	In UMa-AV downlink, the impact of aerial traffic on terrestrial UE throughput in Case 3 is low under low offered traffic (i.e., 20% RU in Case 1) and notable in high offered traffic (i.e., 50% RU in Case 1)

	5% throughput loss:  ~2% under low offered traffic and ~31% under high offered traffic

	mean throughput loss:  ~1% under low offered traffic and ~12% under high offered traffic

Observation 2	In UMa-AV downlink, the impact of aerial traffic on terrestrial UE throughput in Cases 4-5 is marginal under low offered traffic (i.e., 20% RU in Case 1) and high in high offered traffic (i.e., 50% RU in Case 1)

	5% throughput loss:  less than 15% under low offered traffic and more than 42% under high offered traffic

	mean throughput loss:  less than 7% under low offered traffic and more than 23% under high offered traffic

Observation 3	In UMa-AV downlink, the aerial UE throughputs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

Observation 4	In UMa-AV uplink, the impact of aerial traffic on terrestrial UE throughputs in Case 3 is low under low offered traffic (i.e., 20% RU in Case 1) and marginal under high offered traffic (i.e., 50% RU in Case 1)

	5% throughput loss: no loss under low offered traffic and ~8% under high offered traffic

	mean throughput loss: ~1% loss under low offered traffic and ~5% under high offered traffic

Observation 5	In UMa-AV uplink, the impact of aerial traffic on terrestrial UE throughputs in Cases 4-5 is marginal under low offered traffic (i.e., 20% RU in Case 1) and high under high offered traffic (i.e., 50% RU in Case 1)

	5% throughput loss:  less than 16% under low offered traffic and higher than 26% under high offered traffic

	mean throughput loss: less than 9% under low offered traffic and higher than 13% under high offered traffic

Observation 6	In UMa-AV uplink, the aerial UE throughputs are higher than those of terrestrial UEs.  This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

Based on the discussion in this contribution, we propose the following:

Proposal 1	Capture the results presented in this contribution in the TR 36.777

# Appendix A: Evaluation Assumption

The below table summarizes the evaluation assumptions

| Parameter | Value |
| --- | --- |
| Cell Layout | 19 macro sites, 3 sectors per site |
| BS Antenna Configuration | 2Tx/2Rx cross polarized |
| BS Antenna pattern | (M,N,P) = (8,1,2) according to TR 36.873 with 100 degree downtilt angle for UMa-AV |
| Wrapping Method | Geographic Distance based |
| Handover Margin | 0 dB |
| UL Power control | P0=-85 dBm and alpha=0.8 |
| Fast Fading Model | Option 1 (CDL based) |
| Height of the Aerial UEs | Uniformly distributed between 1.5m and 300m |

# References

RP-170779, “New SID on Enhanced Support for Aerial Vehicles,” NTT DOCOMO INC, Ericsson.