3GPP TSG RAN WG1 Meeting #90bis                                    	R1-1718149

Prague, Czechia, 9th – 13th October 2017

Agenda item:    6.2.6.3

Source:             Qualcomm Incorporated

Title:                  Coexistence with NR

Document for:  Discussion and Decision

- Introduction

In RAN#76 a revised WID of Further NB-IoT enhancements [1] was approved with the objective to support TDD for in-band, guard-band and standalone operation modes of NB-IoT. The agreed work scope says the baseline design for NB-IoT TDD is Rel-13:

Support for TDD [RAN1, RAN2, RAN4]:

Specify TDD support for in-band, guard-band, and standalone operation modes of NB-IoT. The design shall assume no UL compensation gaps are needed by UE, and strive towards a common design among the deployment modes.

- Relaxations of MCL and/or latency and/or capacity targets to be considered by RAN1.

- Baseline is to support the same features as Rel-13 NB-IoT, additionally considering small-cells scenarios

In this contribution, we will discuss the aspects of facilitating close coexistence of NB-IoT TDD with NR.

- Discussion

To ensure good co-existence between NR and LTE is an important NR requirement since LTE and NR may be deployed with frequency-overlapping carriers as well as frequency adjacent non-overlapping carriers. Recently, there are also proposals to support eMTC/NB-IoT inband within an NR carrier as discussed in [2] and [3]. The motivation is to allow the operator to update the existing eMTC/NB-IoT enabled network to NR while continuing serving the deployed IoT devices given the expected long lifetime [4]. Therefore, it is desirable that eMTC/NB-IoT can be able to operate within an NR carrier.

The issues of supporting eMTC/NB-IoT inband within an NR carrier have been considered in [2] and [3]. A concept of reserved resources is proposed to support NR coexistence with LTE eMTC and NB-IoT. A reserved resource is a set of NR resource elements configured not to be transmitted, and NR UEs may puncture or rate match around the reserved resources. Therefore, the NR resources overlapping with critical eMTC and NB-IoT transmissions (e.g., PSS/SSS/PBCH/SIB1) will be configured as reserved resources thus allowing for blanking NR transmission without breaking eMTC/NB-IoT. However, it shall be noted that the solutions are mainly from NR perspective since it is assumed that an eMTC/NB-IoT UE is unware that it is operating in an NR carrier rather than LTE.

Observation 1: The concept of reserved resources for supporting for coexistence with eMTC/NB-IoT is mainly considered from NR perspective.

For NR, one subframe has a fixed length of 1ms same as LTE. A subframe may consist of one or multiple slots dependent on the subcarrier spacing. That is, for 15KHz subcarrier spacing one subframe contains only one slot, and for non-15KHz subcarrier there could be 2 or more slots in each subframe. Different from LTE, a slot may contain all DL, all UL or {at least one DL part and at least UL part}. For the latter one, a few number of OFDM symbols at the beginning of the slot is used for DL control and a few number of OFDM symbols at the end of the slot is used for UL control and the other parts can be configured for DL or UL or gap as shown in Figure 1 below. It can be seen that when co-existing with LTE eMTC/NB-IoT, the slot format with mixed DL/UL may not be allowed to use in the subframe overlapping with eMTC/NB-IoT. This would limit the flexibility for NR on dynamic changing the slot formats based on traffic and latency requirements especially when the NR bandwidth is sufficiently large and the eMTC/NB-IoT in-band carrier is relatively small.

Figure 1: Example of different slot formats in NR

Observation 2: There is a restriction on the applied slot format when multiplexing eMTC/NB-IOT and NR in the same subframe.

For NB-IoT TDD, since it is developed from scratch in Rel-15 it is possible to introduce some mechanisms to enable general forward compatibility and facilitate good coexistence with NR without breaking NR key features. The concept of reserved resources can also be considered for NB-IoT TDD to ensure good coexistence with NR. The reserved resources can be configured on per-OFDM symbol basis. Simply speaking, for DL subframes, the resources overlapping with NR UL control region could be semi-statically configured as reserved resources for NB-IoT TDD thus allowing for symbol level blanking. Similarly, for UL subframes the symbols overlapping with NR DL control region can be configured as reserved resources to be not transmitted for NB-IoT TDD. Alternatively, we can extend the special subframe configuration to normal DL and UL subframe. Any subframe in NB-IoT TDD can be configured as special subframe to support inband with NR frame structure.

Proposal 1: The design of NB-IoT TDD shall consider forward compatibility to support inband with NR frame structure by the use of reserved resources (e.g. symbol level).

- Summary

In this contribution, we presented our views on coexistence between NB-IoT TDD and with NR. We made the following observations and proposals.

Observation 1: The concept of reserved resources for supporting for coexistence with eMTC/NB-IoT is mainly considered from NR perspective.

Observation 2: There is a restriction on the applied slot format when multiplexing eMTC/NB-IOT and NR in the same subframe.

Proposal 1: The design of NB-IoT TDD shall consider forward compatibility to support inband with NR frame structure by the use of reserved resources (e.g. symbol level).

References

[1] RP-171428, “Revised WID on further NB-IoT enhancements”, Huawei, HiSilicon, 3GPP RAN#76 meeting, West Palm Beach, USA, June 5-8, 2017

[2] R1-1710912, “Reserved resources supporting NR co-existence with LTE and NB-IoT”, Ericsson, RAN1 NR-Ad-Hoc#2, Qingdao, PR China, 27-30 June, 2017.

[3] R1-1716252, “NR coexistence with eMTC”, Sony, RAN1 NR-Ad-Hoc#3, Nagoya, Japan, 18-21 September, 2017.

[4] R1-1612942, “On NB-IoT, eMTC and NR coexistence”, Ericsson, RAN1#87, Reno, USA, 14-18 November, 2016.