3GPP TSG RAN WG1 Meeting #90bis 	R1- 1717845

Prague, CZ, 9th – 13th, October 2017

Source:	CATT

Title:	Discussion on Polar codes for PBCH

Agenda Item:	7.4.3

Document for:	Discussion/Decision

Introduction

In 3GPP RAN1 meetings [1-3], following agreements have been reached for eMBB PBCH design.

Agreements: [#88bis]

- PBCH BW: 288 subcarriers, 2 OFDM symbols (additional symbols if MIB size larger than assumed)

Agreement: [#89]

- Polar coding is adopted for NR-PBCH

  - Using same polar code construction as for the control channel

  - Nmax = 512

- Working assumption that the data, including time index if carried by NR-PBCH excluding DMRS is transmitted explicitly

  - Can be revisited if significant benefit is shown from partial implicit transmission of time index if allowed by the polar code design

Agreements: [#89]

- RAN1 targets design of NR-PBCH payload size to be no larger than 72 bits and no less than 40 bits including CRC.

  - Note: Based on the performance evaluation done so far, the upper limit range is between 72 and 48 bits

Agreements:  [#NR-AH#2]

- For the case of 2 PBCH symbols within the SS block: PSS-PBCH-SSS-PBCH

- PBCH RE mapping: Alt. 1, NR-PBCH coded bits of the NR-PBCH code block(s) are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block

Agreement: [#NR-AH#2]

- Equal DMRS density across NR-PBCH with 3 RE/PRB/Symbol

Agreements: [#NR-AH#2]

- Rmin = 1/8

In this contribution, some aspects on polar code design are discussed to support PBCH transmission.

Discussion

In RAN1 NR AH#2, it was agreed that NR-PBCH coded bits are mapped to2 OFDM symbols with 1/4 density of DMRS and 288 subcarriers at each symbol.  the number of REs left for PBCH payload transmission is 432 (288*2*3/4).  With QPSK modulation, the total number of coded bits is 864.

- Mother code length

Polar code was agreed to have maximum sequence length Nmax of 512 for DL.  Mother code with length N=512 bits can be used to match 432 REs of two OFDM symbols. The minimum code rate Rmin=1/8 of Polar code has been agreed for the control channels. The mother codes length of Polar code should be determined by Rmin. PBCH should apply both N=512 and N=256 polar mother codes and minimum code rate Rmin=1/8 for payload size 40 <=K < 72 including CRC e.g. K<512/8=64.

Polar code with total 24-bit CRC bits is evaluated for PBCH. CRC bits are included as part of information bits K and M=864 coded bits can be used for PBCH transmission.

PBCH performance is evaluated with the following parameters.

Case 1: K= 60, N=256, M=864, QPSK modulation.

Case 2: K= 60, N=512, M=864, QPSK modulation.

Evaluation results are shown in Figure 1

Figure 1: Performance of Polar codes with K=60

Figure 1 shows that the BLER performance of case 1 with length 256 Polar code sequence is much worse than that of case 2 with length 512 Polar code sequence. Although the coding rate of case 2 is lower than 1/8, Polar code with length 512 mother code shows significant coding gain (about 0.4dB at BLER=10^-4) over that of Polar code with length 256 mother code.

Proposal 1: Polar code with length 512 mother code should be adopted for NR PBCH.

Conclusion

The above discussion is summarized with following observations and proposals:

Proposal 1: Polar code with length 512 mother code should be adopted for NR PBCH.

References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #88bis.

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #89.

- Chairman’s Notes, 3GPP TSG RAN WG1 NR-AH #2 Meeting.