TSG-RAN WG1 #90bis	R1-1719035

Prague, Czech Republic, October 9 – 13, 2017

Source:	Ericsson

Title:	Summary of offline discussion on 7.3.3.1 (TB size)

Agenda Item:	7.3.3.1

Document for:	Discussion and Decision

For the purpose of TB size determination

- Calculate an “intermediate” number of information bits NRE∙υ∙Qm∙R where

  - υ is the number of layers,

  - Qm is the modulation order,

  - R is the code rate

  - NRE is number of resource elements

- When determining NRE (number of REs) within a slot

  - Determine X =  12* #OFDM_symbols_scheduled – Xd – Xoh

    - Xd = #REs_for_DMRS_per_PRB

    - Xoh = accounts for overhead from CSI-RS, CORESET, etc. One value for UL, one for DL.

      - Option 1: Xoh is fixed (e.g. =0 for UL and =12 for DL)

      - Option 2: Xoh is semi-statically determined

      - Option 3: Xoh is dynamically indicated by the DCI, possibly in combination with semi-static signaling

  - Quantize X into one of a predefined set of values, resulting in Y

    - FFS: values, e.g. 12*n, 1<=n<=14

    - FFS: floor, ceiling or some other quantization

    - Note: quantization may not be needed for option 1 and 2 above

    - FFS: quant

    - ization step should ensure the same TB size can be obtained between transmission and retransmission, irrespective of the number of layers used for the retransmission. otherwise Xd has to be independent of the number of layers

  - #REs = Y * #PRBs_scheduled

- Obtain the actual TB size from the intermediate number of information bits according to the channel coding decisions