3GPP TSG RAN WG1 Meeting # 91 	R1-1719428

Reno, USA, November 27th – December 1st, 2017

Agenda Item:	7.2.2.6

Source:	Huawei, HiSilicon

Title:	Remaining issues for codebook subset restriction

Document for:	Discussion and decision

# Introduction

At last RAN1 meeting, the following agreement was achieved on codebook subset restriction [1]

Agreement:

- For Type I SP, rank 3-4 codebooks for 16, 24, and 32 ports

Use single bitfield to determine restricted vl2,m depending on restricted vl1,m

vl,m is restricted if at least one of v2l+1,m, v2l-1,m, v2l,m is restricted

FFS: Introduction of inter-group co-phasing restriction

This contribution mainly addresses the remaining FFS issue.

# Discussion

# CSR for rank 3-4 codebooks for 16, 24 and 32 ports

In the last meeting, the association between the beam vl1,m of length N1N2 and the beam vl2,m  of length N1N22 are determined. The remaining issue wether or not to introduce the inter-group co-phasing restriction.

Based on Fig.1, it can be seen that with different inter-group co-phasing, the directions of the beams formed from vl2,m  and the co-phasing are different, and the directions are aligned with the diretion of different beams vl1,m . Thus, when vl1,m is restricted, the related vl2,m and the cophasing should be restricted together.

Figure 1 b0 with all cophasings

Based on our analysis in the appendix, the restricted cophasing can be determined according to the following rule:

- If vl1,m is restricted, wideband co-phasing ψmod(l1,4)  is restricted as well, and ψmod(l1,4)   can not be used to construct precoder with the restricted vl2,m for rank 3/4.

This mapping can be concluded in a more generalized manner and is summarized in Table 2:

Table 2 Proposed association between CSR bitmap and beams/cophasing for rank 3/4

| Bit in the bitmap | Associated beams for rank 3/4 | Associated cophasing |
| --- | --- | --- |
| aN2O2l1+l2 (l1 is even) | vl12,m | ψmod(l1,4) |
| aN2O2l1+l2 (l1 is odd) | vl12,m, vmod(l12+1,12N1N2),m | ψmod(l1,4) |

Proposal: Supprot inter-group cophasing restriction for Type I rank 3/4 codebook for 16, 24 and 32 ports:

- If vl1,m is restricted, wideband co-phasing ψmod(l1,4)  is restricted together with the related vl2,m.

# Conclusions

In this contribution, we analyze the beam shape that rank3-4 codebook formed and have the following proposal:

Proposal: Supprot inter-group cophasing restriction for Type I rank 3/4 codebook for 16, 24 and 32 ports:

- If vl1,m is restricted, wideband co-phasing ψmod(l1,4)  is restricted together with the related vl2,m.

# References

RAN1 Chairman’s Notes 90 bis, Prague, CZ, 9th – 13th, October 2017.

- Appendix A

Mapping relation derivation:

- The relation between b2l and bl' :

b2l=1ej2π2lNO⋮ej2π(N2-1)2lNO⋮ej2π(N-1)2lNO=1ej2πlNO/2⋮ej2π(N2-1)lNO/2ej2πN2lNO/2×1⋮ej2πN2lNO/2×ej2π(N2-1)lNO/2=bl'φ2lbl'

where φ2l=ejπ2lO, and if O=4, then φ2l=ejπ2l4∈{0,±ej2π4}.

That is to say, beam b2l and beam constructed by bl' and φl generate strong power in the same spatial direction. Thus, if b2l is restricted, then the vector constructed by bl' and φl should also be restricted.

Observation 1: If DFT beam b2l∈CN×1 is restricted, then at least the combination of DFT beam bl'∈CN2×1 and inter-group cophasing φ2l=ejπ2l4 should be restricted as well.

- The relation between b2l+1 and bl', bl+1' :

b2l+1=1ej2π2l+1NO⋮ej2π(N2-1)(2l+1)NO⋮ej2π(N-1)(2l+1)NO=DNb2l

where DN=diag1, ej2πNO,…,ej2π(N-1)NO=DN2'00θDN2' is a rotation matrix for beams of length N， and DN2'=diag1, ej2π1/2NO/2, …,ej2π12(N2-1)NO/2 is a rotation matrix for beams of length N/2, which rotates an angle of  1/2NO/2.  θ=ejπO.

Thus,

b2l+1=DN2'bl'ω2l+1DN2'bl'

where ω2l+1=θ×φ2l. If O=4, then ω2l+1∈{±ejπ4,±ej3π4} .

Since DN2' only rotates a half of the angle interval of the beams {b0', b1',…,bNO'}, both bl' and bl+1' can generate strong power along the direction of b2l+1, when working with ω2l+1. Thus, we have the following observation:

Observation 2: If DFT beam b2l+1∈CN×1 is restricted, then at least the combination of DFT beam  bl'∈CN2×1 and co-phasing ω2l+1, and the combination of DFT beam  bl+1' and ω2l+1 should be restricted.