3GPP TSG RAN WG1 NR Meeting 91		R1- 1719783

Reno, USA, 27th November – 1st December, 2017

Source:	vivo

Title:	DCI contents and design

Agenda Item:	7.3.1.4

Document for:	Discussion and Decision

- Introduction

This is a revised contribution from R1-1717487.

Downlink control information (DCI) contents and relevant bit-field discussion is ongoing in the reflector based on [1]. This contribution firstly discusses some of the potential DCI fields for DL/UL grants in NR. Then, analyze and list the contents and bit-field of the general DCI and fallback DCI in our views.

- Potential NR DCI field

Some possible candidates from our perspective that acknowledge to the agreements include,

  - A-CSI on short PUCCH

A-CSI on short PUCCH can obtain more scheduling flexibility. For DL heavy scenarios, CSI for DL transmission is more often required. Based on short PUCCH transmission, fast CQI acquisition can be achieved. This document discusses the potential issue on A-CSI trigger.

    - DCI format design

A-CSI trigger on short PUCCH can base on DL-related DCI or UL-related DCI. A-CSI trigger based on UL grant is not preferred because A-CSI can be triggered on PUSCH transmission. No significant difference is observed between two manners. If UL grant scheduling a PUSCH transmission triggers A-CSI on short PUCCH in a self-contained slot, simultaneous PUCCH and PUSCH transmission with FDMed manner is required. This is not supported in Rel-15. Generally, accompanied by DL scheduling, A-CSI can be triggered, which does not lead to the additional blind decoding attempts at UE side. UE group specific DCI is transmitted periodically. Service condition may have large difference among UEs. Group DCI scheduling is not efficient.

Proposal 1: If A-CSI on short PUCCH is supported, CSI report triggering in DL-related DCI is preferred. UE specific DCI is suggested.

- dedicated DL DCI

A dedicated DCI can be defined to trigger A-CSI transmission on short PUCCH, which is different to DL or UL scheduling grant. The dedicated DCI includes a concrete PUCCH resource which can provide the higher flexibility. PUCCH resource indication, e.g. starting slot and symbols index, the number of symbols and number of PRB and so on, should be included in the dedicated DCI. For this method, blind decoding attempts would be increased and specification work seems to be large.

- DL scheduling grant

Another method is based on DL scheduling grant. This method does not worsen the blind decoding attempts. Specification work is manageable compared to the dedicated DL DCI. For A-CSI transmission on short PUCCH, triggering and resource indication are included in DCI. For the triggering, two alternatives can be selected.

Alt.1: Combination of some fields in DCI can be defined.

The additional trigger bits are not introduced in this method. For example, resource allocation field set to zero expresses the trigger of A-CSI on short PUCCH. The combination of other fields also can be considered when there is the unused code-point. This should be concluded after DCI contents discussion.

Alt. 2: An additional bit field can be added in DL scheduling DCI.

Similar to A-CSI request field in UL grant in LTE, triggering A-CSI on short PUCCH can be included in DL scheduling DCI.

For short PUCCH transmission, two options can be selected.

  - Opt.1: TDMed HARQ-ACK and A-CSI transmission on PUCCH

  - Opt.2: Multiplexed HARQ-ACK and A-CSI on short PUCCH

For these options, there may be different for short PUCCH resource indication. The corresponding discussion in our companion document [7].

A-CSI trigger is for DL scheduling with finer granularity, thus this field can be precluded in fallback DCI.

Proposal 2: For DL scheduling grant, two alternatives can be considered:

Alt.1: Combination of some fields can be defined.

Alt.2: one additional bit field can be added in DL grant scheduling DCI.

Proposal 3: Fallback DCI does not include A-CSI trigger field.

  - activation/deactivation of the BWP

It is agreed in RAN1 #90 that the activation/deactivation of DL and UL bandwidth part by explicit indication at least in DCI. One or multiple bandwidth part configurations for each component carrier can be semi-statically signaled to a UE (UE can only activate one bandwidth part for a given time instant), and then the DCI signals the activation/deactivation to UE to timely switch to the target BWP. In RAN1 #90bis, it is agreed that for each serving cell, the maximal number of DL/UL BWP configurations is 4 DL BWPs and 4 UL BWPs for paired spectrum, 4DL/UL BWP pairs for unpaired spectrum and 4 UL BWPs for SUL. RRC could configure the presence of BWP indication field in DCI. If the BWP index present in DCI, the payload of DCI will be 2 bits to indicate the index of BWP. In [8], it is also proposed only use fallback DCI to activate/deactivate BWP. Hence in such case, BWP index field is not necessary to be included in the normal DCI.

Proposal 4: The BWP index field should be added in NR DCI.

In addition to these fields, there are some other potential information may be needed, for example, the indication of beam index for PDSCH/PUSCH, the assigned PDSCH/PUSCH numerology, hopping position information for long PUCCH, etc. Mechanism for network to dynamically manage the UE soft buffer in order to achieve lower minimum coding rates and IR improvements, which may result in additional DCI modification, can also be considered.  However, they are still under discussion on related topics, therefore, it may be too early to make a decision right now.

- Estimated size of DCI

According to the email discussion about DCI formats, we give some reference number of bits for each DCI field. The LTE DCI could be the starting point.

- MCS field

In LTE, it supports up to 256QAM in MCS table containing 32 values. Based on channel coding discussion, the modulation order may be extend to 1024QAM. If it uses 5bits (32 values) in DCI to indicate MCS, the flexibility of NW may be limited in support of 1024QAM. On the other hand, if it uses 6bits in DCI, the range of MCS could be extended to up to 64 so that 1024QAM can be supported. Nevertheless, the overhead of MCS in DCI is increased in this case, although UE may not use all the MCSs simultaneously. As discussed in [4] multiple MCS tables for different kinds of services should be supported. Therefore, it is proposed that the MCS field can be kept to 5bits.

- Scheduling timing information for PDSCH/PUSCH

According to the agreement of RAN1 #90 meeting in [5], the type of scheduling can divide into three cases: one-slot case, multi-slot case and non-slot (i.e., mini-slot) case. The payload size for each case is calculated and presented in the following table.

| Payload(bits) |  | Starting symbol | Ending symbol | Starting slot | Ending slot | Total | comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| One-slot | Cross-slot | 2 | 1 | 1 | 0 | 4 | Assumption: crossing maximum 4 slots |
|  | Non cross-slot | 2 | 1 | 0 | 0 | 3 |  |
| Multi-slot | Cross-slot | 2 | 1 | 1 | 1 | 5 | Assumption: maximum number of aggregated slots is 4 |
|  | Non cross-slot | 2 | 1 | 0 | 1 | 4 |  |
| Non-slot | Non cross-slot | 3 | 1 | 0 | 0 | 4 |  |

Note: The maximum number of aggregated slots is selected so that four RVs can be scheduled by a single DCI (for TTI bundling, URLLC repetition, etc.).

If the starting/ending symbol and starting/ending slot are semi-statically configured for the UE, DCI indicates a pointer to the set.

For one-slot case, if the cross-slot scheduling applied, there are totally 4*2*2=16 entries, yielding a field of 4bits. If the cross-slot scheduling is not supported, there are totally 4*2=8 entries, yielding a field of 3bits.

For multi-slot case, if the cross-slot scheduling applied, there are totally 4*2*2*2=32 entries, yielding a field of 5bits. If the cross-slot scheduling is not supported, there are totally 4*2*2=16 entries, yielding a field of 4 bits.

For non-slot case, the starting symbol can be indicated as the number of symbols from the start of the PDCCH. The ending symbol can be indicated as the number of symbols from the starting symbol. There are totally 8*2=16 entries, yielding a field of 4bits.

Therefore, the DCI contents for time domain PDSCH/PUSCH allocation can be up to 5bits.

DL-related general DCI field show below:

| Field | bits | Notes |
| --- | --- | --- |
| Header | 2 | Distinguish different DCI formats with the same DCI size. |
| Carrier indicator | 3 | For cross-carrier scheduling fixed to 3 bits. |
| DL BWP indication | 0/2 | RRC configure the presence of BWP indication field in DCI. |
| Frequency-domain PDSCH resources - PRBs | 25 | Resources in the frequency domain, NRBDLis the number of RBs, and P is the RBG size. It’s DL Type 0 RA. |
| Time domain PDSCH resources | 5 | Resources in the time domain, including cross-slot scheduling |
| VRB-to-PRB mapping | 1 | Flag to control VRB-to-PRB mapping |
| PDSCH/PDCCH resource sharing | 2 | Indicates whether the PDSCH is mapped to the semi-statically configured resource sets or not. If have multiple resource sets, it needs multiple bits. |
| Bundling size | 1 | Indicates the PRB bundling size |
| MCS | 5 | Modulation and coding scheme per TB. |
| New data indicator | 1 | For one TB, multiple bits in case of multiple TBs |
| Redundancy version | 2 | For one TB, multiple bits in case of multiple TBs |
| HARQ process number. | 3~4 | HARQ process number |
| CBGFI | 0~1 | CBG flush indication. Consists of 1 bit (for one CW) if configured. |
| CBGTI | 8 | Indicates the CBG(s) (re)transmitted. Consists of N bits bitmap if CBG(s) are configured. |
| PUCCH power control | 2 |  |
| ARI | 2 | Ack/Nack resource index |
| HARQ timing indicator | 2 | Indicates PUCCH time domain resource allocation. |
| Downlink assignment index | 4 | Counter DAI and total DAI |
| Antenna port(s) | 4~5 | The antenna ports to receive, and the antenna ports field jointly encoded with number of layers. |
| TCI | 2 | Beam indication to indicate QCL assumption between DL RS antenna port(s) and DMRS antenna port(s) of DL data channel at least w.r.t. spatial QCL parameter |
| CSI request | 3/4/5/6/7/8 | CSI measurement request and CSI report trigger on PUCCH. The detail of the value is decided in RAN1 #91. |
| RNTI/CRC | 24 | RNTI is used to identity of the UE (or group of UEs), and RNTI is implicitly encoded in CRC. |

UL-related general DCI field show below:

| Field | bits | Notes |
| --- | --- | --- |
| Header | 2 | Distinguish different DCI formats with the same DCI size. |
| Carrier indicator | 3 | For cross-carrier scheduling, fixed at 3 bits. |
| UL/SUL indicator | 0/1 |  |
| UL BWP indication | 0/2 | RRC configure the presence of BWP indication field in DCI. |
| Frequency-domain PUSCH resources - PRBs | 25 | Resources in the frequency domain. It is UL type0 RA. |
| Time domain PUSCH resources | 5 | Resources in the time domain, including cross-slot scheduling |
| MCS | 5 | Modulation and coding scheme per TB. |
| New data indicator | 1 | For one TB, multiple bits in case of multiple TBs |
| Redundancy version | 2 | For one TB, multiple bits in case of multiple TBs |
| HARQ process number. | 3~4 | HARQ process number |
| CBGTI | 8 | Indicates the CBG(s) to (re)transmitted. Consists of N bits bitmap if CBG(s) are configured. |
| PUSCH power control | 2 |  |
| SRI/TRI/TPMI | 4~8/9 | 4~8 bits depending on configurations/UE coherence capability. 9 bits if independent coding is applied. |
| Antenna port(s) | 4~5 | The antenna ports to receive, and the antenna ports field jointly encoded with number of layers. |
| CSI request | 3/4/5/6/7/8 | CSI measurement request and CSI report trigger on PUCCH. The detail of the value is decide in RAN1 #91. |
| SRS triggering request | 4 | To trigger an SRS transmission in the uplink. |
| RNTI/CRC | 24 | RNTI is used to identity of the UE (or group of UEs), and RNTI is implicitly encoded in CRC. |

One purpose of fallback DCI is to handle various RRC configuration with various features. It is important to keep a constant DCI format as fallback DCI. In order to keep the size of the fallback DCI small, some fields are need to be deleted based on the general DCI. If the sizes of UL and DL fallback DCI are the same, a header needs to be used to indicate whether it is UL or DL.

The following shows our view on the UL fallback DCI and DL fallback DCI.

DL fallback DCI

| Field | bits | Notes |
| --- | --- | --- |
| Header | 2 | Distinguish it is UL fallback DCI or DL fallback DCI. |
| DL BWP indication | 0/2 | RRC configure the presence of BWP indication field in DCI. |
| Frequency-domain PDSCH resources - PRBs | 16 | Resources in the frequency domain. It’s DL Type 1 RA. |
| Time domain PDSCH resources | 5 | Resources in the time domain, including cross-slot scheduling |
| VRB-to-PRB mapping | 1 | Flag to control VRB-to-PRB mapping |
| New data indicator | 1 | For one TB, multiple bits if we have multiple TBs |
| MCS | 5 | Modulation and coding scheme per TB. |
| Redundancy version | 2 | For one TB, multiple bits in case of multiple TBs |
| HARQ process number. | 3~4 | HARQ process number |
| PUCCH power control | 2 |  |
| ARI | 2 | Indicates the PUCCH resource allocation. |
| DAI | 2 |  |
| Antenna ports | 4~5 | Antenna ports used(and the number of layers) |
| TCI | 2 |  |
| RNTI/CRC | 24 | RNTI is used to identity of the UE (or group of UEs), and RNTI is implicitly encoded in CRC. |
| Total number of bits | 71~75 |  |

UL fallback DCI

| Field | bits | Notes |
| --- | --- | --- |
| Header | 2 | Distinguish different DCI formats with the same DCI size. |
| UL BWP indication | 0/2 | RRC configure the presence of BWP indication field in DCI. |
| Frequency-domain PUSCH resources - PRBs | 16 | Resources in the frequency domain. It is UL type1 RA. |
| Time domain PUSCH resources | 5 | Resources in the time domain, including cross-slot scheduling |
| VRB-to-PRB mapping | 1 | Flag to control VRB-to-PRB mapping |
| MCS | 5 | Modulation and coding scheme per TB. |
| New data indicator | 1 | For one TB, multiple bits in case of multiple TBs |
| Redundancy version | 2 | For one TB, multiple bits in case of multiple TBs |
| HARQ process number. | 3~4 | HARQ process number |
| PUSCH power control | 2 |  |
| SRI/TRI/TPMI | 4 |  |
| Antenna port(s) | 4~5 | Antenna ports used(and the number of layers) |
| RNTI/CRC | 24 | RNTI is used to identity of the UE (or group of UEs), and RNTI is implicitly encoded in CRC. |
| Total number of bits | 69~73 |  |

- Conclusion

In the contribution, we have some investigations on the DCI content and design, and propose that,

Proposal 1: If A-CSI on short PUCCH is supported, CSI report triggering in DL-related DCI is preferred. UE specific DCI is suggested.

Proposal 2: For DL scheduling grant, two alternatives can be considered:

Alt.1: Combination of some fields can be defined.

Alt.2: one additional bit field can be added in DL grant scheduling DCI.

Proposal 3: Fallback DCI does not include A-CSI trigger field.

Proposal 4: The BWP index field should be added in NR DCI.

References

- R1-17XXXXX, “E-mail discussion on DCI content”, Ericsson, November 2017.

- TR 38.802, “Study on new radio access technology Physical layer aspects”, V14.2.0.

- R1-1719800, “Other aspects on bandwidth parts”, vivo, November 2017.

- R1-1719793, “On DL/UL resource allocation”, vivo, November 2017.

- “RAN1#90 Chairman Notes”, Prague, Czech Republic, 21st – 25th August 2017.

- “RAN1#90bis chairman Notes”, Prague, Czech Republic, 9th – 13th October 2017.

- R1-1719792, “On PUCCH resource allocation”, vivo, November 2017.

- R1-1719800, “Other aspects on bandwidth Parts”, vivo, November 2017.