3GPP TSG RAN WG1 Meeting 91 		R1-1720439

Reno, USA, November 27th – December 1st, 2017

Agenda item:	6.2.7.6

Source: 		Qualcomm Incorporated

Title: 	Field measurement results

Document for:	Discussion/Decision

# Introduction

In email discussion [90b-LTE-22] the following agreement was reached:

Agreements:

- One or more field measurement results are to be included in the TR, where the field measurement results are chosen from (a subset or all of them, up for further discussion in RAN1#91) at least the following:

  - Number of detected cells

  - Serving cell RSRP/RSRQ

  - Neighbour cell RSRP/RSRQ

  - RSSI

  - RSRP gap between serving cell and each other detected cells

  - Distance to serving cell

  - Distance to neighbor cells

  - RS-SINR (refer to section 5.1.23 of TS 36.214)

  - PDCCH BLER

  - UL data rate

In this document, we present field measurement results to be captured in the TR

# Field trial results

## 2.1	Setup

For this trial, data was collected as indicated in Table 1. Connectivity was provided and tested using a commercial cellular network during all flights.

Table 1: Trial Setup

| Data | Description |
| --- | --- |
| Location | Qualcomm UAS Flight Center, San Diego, California |
| Environment | Mixed suburban |
| Altitudes | Ground, 30, 60, 90, 120 meters AGL |
| Test types | Mobility route at 5 m/s with 0.5 Mbps UDP UL throughput requested Mobility route at 5 m/s and periodic RACH every 15 seconds Stop/Start route with 0.5 Mbps UDP UL throughput requested |
| LTE bands (locked to one band per flight) | PCS AWS 700MHz |
| Data collection | On device logging IPerf logs |

The results are derived from a single aerial UE performing a 2.5 km loop at different altitudes at a time, co-existing with other subscribers of the live commercial network. Each altitude was flown multiple times for data collection in each band, and to provide sufficient data for each case (at least 2 loops).

Table 2 lists the band and altitude permutations and the total duration for each case.

Table 2 Band, Altitude, and Duration

| Band | Altitude (m AGL) | Total Time (min:sec) |
| --- | --- | --- |
| PCS | 0 | 21:36 |
|  | 30 | 12:36 |
|  | 60 | 12:33 |
|  | 90 | 32:21 |
|  | 120 | 33:12 |
| AWS | 0 | 27:44 |
|  | 30 | 12:31 |
|  | 60 | 12:34 |
|  | 90 | 22:52 |
|  | 120 | 15:49 |
| 700 MHz | 0 | 34:50 |
|  | 30 | 18:44 |
|  | 60 | 12:36 |
|  | 90 | 16:50 |
|  | 120 | 15:47 |

The ground data was collected by mounting the drone to a car and driving the route on surface streets (the duration of these tests tended to be a bit longer than flying due to some stoplights and traffic.)

Each dataset was trimmed so the final data for analysis only includes samples where the drone is at its desired altitude and underway (i.e., excluding takeoff and landing). This prevents takeoff and landing transition data, as well as data when the drone is stationary on the landing pad, from impacting the analysis results.

## 2.2	Pathloss measurements

Figure 1 shows the pathloss measured for different bands and for different UE altitudes. It is observed that the pathloss for aerial UEs is substantially lower than for terrestrial UEs, resembling free space propagation (pathloss exponent of 2).

Figure 1: Pathloss measurements at different UE heights

## 2.3	Number of detected cells

Figure 2: Number of detected cells

Figure 2 shows the histograms of number of number of detected cells for different bands and heights. The bars represent the number of measurement samples for each value of number of detected cells. The figure shows that aerial UEs detect a higher number of cells than the ground UEs. For example, it was observed that while ground UE detects up to only 3 cells in band AWS, the aerial UE detected 5 or more with higher probability. The number of detected cells may be impacted by down-tilting and antenna pattern at the base station.

## 2.4	Distance to detected and serving cells

Figure 3: Distance to detected and serving cells

Figure 3 shows that for PCS band, all ground UEs are served by cell within less than 1km distance, while the aerial UEs are served by base stations as far as 6km. Similar trend was observed for 700MHz band but with less exaggeration. This can possibility be due to down-tilt or radiation pattern.

Similar trend was observed for neighbour cells: for example, for AWS band, up to 8km distances are observed for aerial UEs whereas ground UEs are limited to 1.8km.

## 2.5	RSRP/RSRQ distribution for serving cell

| Figure 4: RSRP distribution for serving cell | Figure 5: RSRQ distribution for serving cell |
| --- | --- |

It is seen that aerial UE observe higher RSRP than ground UEs. As a general trend, both RSRP and RSRQ decrease with altitude, but exact impact depends on antenna/frequency band. However, despite lower RSRQ, coverage for drone UE is not impacted.

## 2.5	Distribution of uplink transmit power

Figure 6 Uplink transmit power

Figure 6 shows the distribution of uplink transmit power for UEs at different altitudes. Due to the lower pathloss experienced by aerial UEs (and its effect in power control), it is observed that, in general, aerial UEs transmit at a lower transmit power than ground UEs (up to 13dB difference in 90% and 9dB difference in median transmit power).

Proposal: Incorporate the above results in the TR.