3GPP TSG RAN WG1 Meeting 91		                          R1-1720643

Reno, USA, November 27th – December 1st, 2017

Agenda Item:	7.4.1.2

Source:	InterDigital Inc.

Title:	On TB Size Design

Document for:	Discussion and Decision

# 1	Introduction

According to the discussions in RAN1 meeting #90bis [1], the TBS (transport block size) determination involves two steps. In the first step, a temporary TBS is calculated based on a formula. In the second step, the actual TBS is derived from the temporary TBS, based on the channel coding decisions.

In the contribution, we discuss the calculation of the actual TBS size from the temporary TBS, considering the two agreed LDPC codes in NR.

# 2	Discussion

## Background on TBS Calculation

In LTE MCS tables [2], each MCS index corresponds to a modulation order and a TBS index. The TBS index, together with the number of PRB (physical resource block), will be used to obtain a TBS based on the given TBS table. The MCS index and the number of PRBs are contained in the DCI message.

The maximum number of PRB in LTE is equal to 110, and correspondingly, the TBS table has 110 columns. In NR, the maximum number of PRB could be as large as 275, implying a larger TBS table than LTE. To avoid a large TBS table, a formula based TBS determination scheme is applied in NR. By this scheme, a temporary TBS is calculated as [1]

TBStemp=NRE∙υ∙Qm∙R,

where υ is the number of layers, Qm is the modulation order, R is the code rate and NRE is number of resource elements. The values of υ, Qm and R can be directly obtained in DCI message. The value of NRE is further calculated as Y*#PRBs_scheduled, where Y is a quantized value of

X=12*#OFDM_symbols_scheduled-Xd-Xoh.

Here, Xd is the number of REs for DMRS per PRB in the scheduled duration and Xoh accounts for overhead from CSI-RS, CORESET, etc.

## Actual TBS Calculation

The actual TBS could be derived from TBStemp, by considering the agreed LDPC codes in NR. The design criteria for the actual TBS, denoted by TBSfinal, are byte-aligned and ensure that no zero padding is necessary with BG1 segmentation [1]. This could be achieved by quantizing TBStemp to TBSfinal. Here, we could have

TBSfinal=TBStempQS∙QS-TBCRC,                          (1)

where QS is the quantization step and TBCRC is the length of TB level CRC. It was agreed that TBCRC is equal to 16 bits if TBSfinal is less than or equal to 3824. Otherwise, TBCRC is equal to 24 bits. We could set TBCRC, based on TBStemp. Specifically, we could have

TBCRC=16,TBStemp≤384024,TBStemp>3840.                           (2)

There are several options of QS.

Option 1: QS=8∙C, where C is the number of CB (code block) segments for this TB. The value of C depends on TBStemp and the code rate R indicated by the MCS index. Specifically, we have

C=1, if (R≤14& TBStemp≤3840)   or (R>14 & TBStemp≤8448),TBStemp3816,if R≤14& TBStemp>3840,TBStemp8424,if R>14 & TBStemp>8448  (3)

Option 2: QS=lcm(8, C), where lcm(8, C) is the least common multiple of 8 and C, and C is calculated by equation (3).

In both Option 1 and Option 2, we could obtain the TBSfinal by inserting (2) and (3) into (1).

Option 3: QS is a function of R and TBStemp such that QS monotonically increases with TBStemp at a given R. One reference function could be the set of information block size used in LDPC evaluation campaigns, i.e., [40:8:512, 528:16:1024, 1056:32:2048, 2112:64:6144, 6272:128:8448]. Here, the QS could be selected as a power of 2 starting from 8, depending on TBStemp.

Note that the maximum code block size of 8448 is used in the CB segmentation process when the code rate is larger than 1/4, and the maximum code block size of 3840 is used in the CB segmentation process when the code rate is no more than 1/4. The number of CB segments depends on the code rate. Hence, the QS values in Option 3 may be different for code rates above or below 1/4.

Based on the above discussions, we have the following proposal:

Proposal 1: Use the formula to calculate the actual TBS from the temporary TBS

TBSfinal=TBStempQS∙QS-TBCRC,

where TBCRC is determined as

TBCRC=16,TBStemp≤384024,TBStemp>3840,

and QS is chosen from one of the three options above.

In the above calculations, the TBCRC depends on TBStemp via equation (2). The actual TBCRC depends on TBSfinal. There may be some gap between TBStemp and TBSfinal such that the actual TBCRC is different from the selected TBCRC in (2). An adjustment of TBSfinal may be needed to address this gap. Hence, we have the following proposal:

Proposal 2: The actual TBS may need to be adjusted for the gap between TBStemp and TBSfinal.

# 3	Conclusion

In this contribution, we discussed the derivation of the actual TBS, based on the temporary TBS. We have the following two proposals:

Proposal 1: Use the formula to calculate the actual TBS from the temporary TBS

TBSfinal=TBStempQS∙QS-TBCRC,

where TBCRC is determined as

TBCRC=16,TBStemp≤384024,TBStemp>3840,

and QS is chosen from one of the three options above.

Proposal 2: The actual TBS may need to be adjusted for the gap between TBStemp and TBSfinal.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting 90bis, Prague, Czech Republic, Oct. 2017.

- 3GPP TS 36.213, v14.4.0, “E-UTRA; Physical layer procedures (release 14),” Sept., 2017.