3GPP TSG-RAN WG1 Meeting #91	R1-1721144

Reno Nevada, USA 27th Nov – 1th Dec 2017

Agenda Item:	6.2.6.3.3

Source: 	Feature lead Ericsson

Title:  	Summary of NB-IoT TDD Downlink aspects

Document for:	Discussion and Decision

# Introduction

In RAN1#90bis meeting, regarding the DL design of NB-IoT TDD, RAN1 made the following agreements

“

•	TDD UL:DL configuration 0 is not supported in TDD NB-IoT in Rel-15

•	MIB-NB is transmitted on the same NB-IoT carrier as NPSS/NSSS.

•	The single NB-IoT carrier for all the other SIBs than SIB1-NB, when not the anchor carrier, is:

•	In a PRB indicated by SIB1-NB with exact signaling design left to RAN2 including whether to signal anything in case these other SIBs are on the anchor carrier

•	It is supported that SIB1-NB is transmitted only on the anchor carrier

•	In at least subframe #0 in odd frames

•	It is supported that SIB1-NB can be transmitted on non-anchor carrier, FFS details

•	It is necessary to consider SFN wraparound as part of FFS

•	Periodicity of SIB1-NB in TDD is the same as FDD (i.e. 2560ms)

•	One transport block of SIB1-NB is transmitted over 8 SIB1-NB subframes (i.e. same as FDD)

•	For NPSS, NSSS and NPBCH transmission in TDD:

•	NPSS is transmitted on subframe #5 in every radio frame

•	NSSS is transmitted on subframe #0 in every even-numbered radio frame

•	NPBCH is in subframe 9 in every radio frame on the same carrier as NPSS/NSSS.

•	Confirm the working assumptions from RAN1#90, i.e. NPSS uses the lower 11 subcarriers in one

subframe and the same cover code for TDD as FDD.

•	The NPSS and NSSS sequences for TDD are the same as FDD.

•	TDD and FDD NB-IoT are distinguished by the relative location of NPSS and NSSS.

”.

In RAN1#90bis meeting, regarding the DL design of NB-IoT TDD, RAN1 made the following working assumptions were made

“

•	TDD UL:DL configuration 6 is not supported in TDD NB-IoT in Rel-15

Working assumption to be automatically confirmed if RAN4 reply LS to R1-1715304 does not raise a problem:

•	TDD NB-IoT will support all LTE special subframe configurations

”.

# Summary of offline discussion

Email discussion ongoing.

# WFs

- WFs circulated offline about SIB1-NB scheduling and SIB1-signalling:

  - R1-1721156	WF on SIB1-NB transmission for TDD NB-IoT ZTE, Sanechips, Qualcomm, Intel, Ericsson

  - R1-17xxxx	Way forward on SIB1-NB transmission in TDD NB-IoT

- WF circulated offline about NRS in TDD NB-IoT:

  - R1-17xxxx 	Way forward on NRS for NB-IoT

- WFs circulated offline about other issues

  - R1-17xxx	WF on Other issues on SIB1-NB in TDD, LG

# Summary of open issues

- SIB1-NB scheduling issue:

  - Whether SIB1-NB can be transmitted both on anchor and non-anchor carrier, or just on anchor or non-anchor carrier.

  - Which subframe to use for SIB1-NB both on anchor and non-anchor carriers.

  - Number of repetitions that should be used for SIB1-NB on non-anchor carriers.

  - How to signal the SIB1-NB configurations (can also let RAN2 to design the detail signaling, and RAN1 provide guidelines).

  - How to signal the DL bit map of different carriers.

- NRS in TDD NB-IoT

  - NRS pattern in special subframes.

  - How many NRS antenna ports should be supported.

- NPDCCH and NPDSCH design

  - Whether to extend α and G in NPDCCH.

  - Whether to drop the entire NPDCCH search space or only candidates when there is an overlapping between two NPDCCH search space.

  - How to interpret the DL gap for NPDCCH and NPDSCH, i.e., whether to count all the subframes or only DL subframes.

- NPRS configuration

  - Currently no WF circulated offline

- Whether to include some new features in Rel-15 to NB-IoT TDD

  - For example, whether to include NSSS based measurements and DL interference randomization enhancements by using new scrambling sequence.

- Other issues:

  - RSRP measurement on the non-anchor carrier

# References