3GPP TSG RAN WG1 Meeting #91	 R1-1721640

Reno, USA, November 27th – December 1st, 2017

Source:	Qualcomm

Title:	Summary of Beam Mgmt.

Agenda Item:	7.2.2.3

Document for:	Discussion and Decision

# Introduction

This document provides a summary of remaining critical issues on beam management for December finalization. The highlighted titles are the ones with likely RRC impact.

# Open issues for Beam Management

## Behavior of Spatial QCL for PDSCH when scheduling offset < k.

Agreement last meeting:

- For the case when at least spatial QCL is configured/indicated, NR supports the beam indication for PDSCH as follows, if TCI field is present:

  - The TCI field is always present in the associated DCI for PDSCH scheduling irrespective of same-slot scheduling or cross-slot scheduling.

  - If the scheduling offset < threshold K: PDSCH uses a pre-configured/pre-defined/rule-based spatial assumption (details: FFS)

    - FFS: The other QCL parameters are still obtained from the N-bit TCI state field in the DCI.

    - FFS: How to update pre-configured/pre-defined spatial assumption (if applicable)

    - Threshold K can be based on UE capability only if multiple candidate values of K are supported.

  - If the scheduling offset >= threshold K: PDSCH uses the beam (spatial QCL parameter) indicated by the N-bit TCI field in the assignment DCI.

- Note: this proposal does not apply to the case of beam management without beam-related indication (ref: Annex)

The remaining critical open issues:

- What is the assumption of the PDSCH beam when scheduling offset is lesser than k?

- How is the offset threshold K determined?

Contribution summary:

- Use the spatial QCL indicated earlier by a scheduling offset. (Qualcomm, Ericsson, ZTE, Mediatek, DOCOMO, LGE, Sharp)

  - For further decision on whether the earlier indication through DCI is from an allocation with scheduling offset >k or not.

- Use spatial QCL indicated by upper layer signaling (Intel, Samsung, vivo, OPPO, CATT, InterDigital [second preference])

- Use same spatial QCL for the associated CORESET (InterDigital)

- Use spatial QCL corresponding to the UE’s latest reporting (Huawei)

  - At least for the case that the UE’s latest reporting is more recent than the latest spatial QCL indication

- Use TCI state with the lowest index (Huawei)

  - No RRC impact

Possible Agreement:

For the case of scheduling offset < k, when at least spatial QCL is configured/indicated and the beam indication for PDSCH is through TCI field associated with the DCI, the spatial QCL used is indicated by an earlier offset.

- Proposal – Alt 1: In slot n, when the scheduling offset < threshold K and Is-TCI-Present = ‘True’, PDSCH uses the following rule-based approach for determining its spatial QCL assumptions

  - If the most recent DCI is received within X, X<Xmax slots before the DCI for the scheduled PDSCH, apply the QCL assumption on the scheduled  PDSCH based on the most recent DCI-based spatial QCL indication.

    - The most recent DCI means the latest DCI that the UE reported its ACK/NACK before slot n-k where k depends on the HARQ timing.

  - Otherwise when no DCI-based indication can be found within X slots before, apply the QCL assumption pre-configured by higher layer if it is configured.

    - FFS: When L1-RSRP of the pre-configured beam is below a threshold, the UE should include the beam quality of the pre-configured beam in beam reporting

    - If this QCL assumption is not configured, the UE should assume the PDSCH is spatially QCLed with the scheduling PDCCH

  - The UE should report its capability of Xmax.

  - When there are multiple CORESETs in the same slot with different configured TCI states

    - The UE applies the most recently indicated TCI state for PDSCH in any CORESET

    - Supported by: ZTE/Sanechips, Intel, Vivo, LG, ASTRI, Qualcomm

- Proposal – Alt 2

  - When the scheduling offset is <=k, the PDSCH uses QCL assumption that is based on a default TCI state (e.g. the first state of the 2^N states used for PDSCH QCL indication)

    - Supported by: Huawei/HiSi, CATT, Ericsson, Samsung

    - Note: Issues with multiple CORESETs if the PDSCH beam has to be CORESET specific.

    - Note: Whether or not beam indication rule depends on the number of CORESETs configured is FFS

## Configuration of TCI states

Agreement earlier meetings:

Agreement from 90-AH

Agreement:

A UE is RRC configured with a list of up to M candidate Transmission Configuration Indication (TCI) states at least for the purposes of QCL indication

- Whether M equal to or larger than 2N is for further study, where N is the size of the DCI field for PDSCH

- FFS: Mapping between the candidate states to the states described by N bit DCI field for PDSCH

- Each TCI state can be configured with one RS Set

- Each ID (FFS: details of ID) of DL RS at least for the purpose of spatial QCL in an RS Set can refer to one of the following DL RS types:

  - SSB

  - Periodic CSI-RS

  - Aperiodic CSI-RS

  - Semi-persistent CSI-RS

- FFS: Other RS (e.g. TRS, PTRS) in an RS set depending on outcome of discussions in the QCL agenda item

- FFS: Mechanisms to initialize/update the ID of a DL RS(s) in the RS Set used at least for spatial QCL purposes

  - At least the following two mechanisms are FFS: (1) explicit signalling to the UE of the DL RS(s) ID and corresponding TCI state (2) implicit association of the DL RS ID(s) to a TCI state based on measurements by the UE.

  - The mechanisms used for different RS types are FFS

- FFS: Whether or not a TCI state includes other parameters(s), e.g., for PDSCH rate matching purposes

- FFS: Value of N, where N is at most [3] bits

Note: More details on specification of more than one DMRS port group and more than one RS Set per TCI state is to be completed after the December release.

Agreement:

The QCL configuration for PDCCH contains the information which provides a reference to a TCI state

- Alt 1: The QCL configuration/indication is on a per CORESET basis

  - The UE applies the QCL assumption on the associated CORESET monitoring occasions. All search space(s) within the CORESET utilize the same QCL.

- Alt 2: The QCL configuration/indication is on a per search space basis

  - The UE applies the QCL assumption on an associated search space. This could mean that in the case where there are multiple search spaces within a CORESET, the UE may be configured with different QCL assumptions for different search spaces.

- Note: The indication of QCL configuration is done by RRC or RRC + MAC CE (FFS: by DCI)

Note: The above options are provided as input to the control channel agenda item discussion

Agreements [90b-NR-23]:

- For the DMRS of NR-PDCCH in a CORESET,

  - The QCL configuration/indication is on a per CORESET basis (Alt.1).

The remaining critical open issues:

- How are the DL-RSs associated with a TCI state first populated and then updated?

- Are Time/Freq references included in TCI?

- What are the steps to configuring the TCI states in a UE – what is the split between RRC and MAC-CE?

- Whether to configure/activate the same table for all channels/RS/CORESET?

Contribution summary:

- Samsung: Each TCI state may be configured with a candidate set of DL-RSs. (Possibly more than 1)

  - MAC-CE is used to activate/deactivate DL-RS from the candidate set- per TCI state

- Ericsson: UE configured with a list of DL-RS IDs within each TCI state; MAC-CE selects one ID from the list to be used for spatial QCL purposes per TCI state

- ZTE: A UE is RRC configured with a list of up to M candidate Transmission Configuration Indication (TCI) states at least for the purposes of QCL indication

  - M can be larger than 2N, where N is the size of the DCI field for PDSCH

- MAC-CE is used to activate 2N candidate TCI states (if M > 2N) and map the activated candidate TCI states into TCI states described by N bit DCI field for PDSCH

  - If M is equal to 2N, the candidate TCI states are mapped into TCI states in order by default.

- Qualcomm/DCM: RRC may configure a TCI state with a default DL-RS during configuration

- CATT: RRC configuration of M candidate TCI states, each of which is associated with one RS resource for spatial QCL indication; MAC-CE to activate 2^N state; fine with M>=2^N; also fine with M=2^N if N>=3; support different activated states for different channels;

- HW: RRC configuration of M candidate TCI states, each of which is associated with one RS resource for spatial QCL indication. Support different activated state configuration at least for DL and UL

Possible Agreement:

- A candidate set of DL RSs are configured using RRC mechanism

  - Alt 1: Each of M TCI state is RRC configured with a fixed QCL reference(s), and MAC-CE is used to select 2^N TCI states out of M

    - Supported: ZTE, Sanechips, IDCC, LGE, ViVO, Huawei, CATT

  - Alt 2: Candidate Pool of DL RS per TCI state is configured using RRC and selection/activation through MAC-CE

    - Supported: Ericsson, Samsung, QC, Sharp,

- MAC-CE is used activate/deactivate the DL-RS from the candidate set per TCI state

- FFS: If the set of DL RSs are chosen on a per TCI state or a common pool for all TCI states

Agreement:

- A candidate set of DL RSs are configured using RRC mechanism

  - Each state of M TCI states is RRC configured with a downlink RS set used as a QCL reference, and MAC-CE is used to select up to 2^N TCI states out of M for PDSCH QCL indication

    - The same set of M TCI states are reused for CORESET

    - K TCI states are configured per CORESET

    - When K>1, MAC CE can indicate which one TCI state to use for control channel QCL indication

    - When K=1, no additional MAC CE signaling is necessary

## Number of TCI states and mapping to DCI bits

Agreement last meeting:

- Down-select to one of the following 2 options for the DCI field size for TCI in RAN1#91

  - Alt-1: Fixed number of bits [2 or 3] bits

  - Alt-2: A higher layer signaling parameter indicates the number of bits (2 or 3)

The remaining critical open issues:

- Is the number of bits for DCI field fixed? If fixed, is it 2 or 3?

- How do the M states map to DCI field?

Contribution summary:

- N is fixed, and M = 2^N (Ericsson, Samsung, CATT-if-N=3, OPPO, MediaTek, InterDigital, Sharp)

- N is fixed and M > 2^N (Qualcomm, vivo, CATT-if-N=2, ZTE)

- N is configurable (Interdigital, HW)

Possible Agreement:

- N is fixed.

Agreement:

- For the number of TCI states and mapping to DCI bits, N is 3 bits.

  - Note: The number of TCI states supported by a UE depends on its capability

## UE spatial behavior prior to configuration of TCI state

Agreement last meeting:

- None

The remaining critical open issues:

- What is expected UE behaviour before the TCI states are configured using a combination of RRC and/or MAC-CE?

Contribution summary/possibilities:

- Use the beam pair/spatial QCL identified during initial access until TCI states are configured (Ericsson, Samsung, Qualcomm,OPPO, MediaTek, CATT, HW, Sharp)

- A default TCI state 0 is signaled based on the initial access ( Nokia, ?)

- Use initial access based beam, no need to signal this TCI state, Is-TCI-present is considered OFF until further configuration (vivo)

- A default TCI state 0 is used after TCI states are configured and TCI-PresentInDCI=Disable

Possible Agreement:

- Use the spatial QCL identified during initial access until TCI states are activated.

  - FFS: If one of the TCI states is reserved for this beam

## Configuration of RRC state Is-TCI-present

Agreement last meeting:

- Proposal: For the case when at least spatial QCL is configured/indicated, support higher-layer UE-specific configuration of whether or not TCI field is present in DL-related DCI

  - Not present: No dynamic indication of QCL parameters for PDSCH is provided in DL-related DCI

    - For PDSCH, UE applies higher-layer signalling of QCL parameters/indication for determining QCL parameters (details: FFS) except for the case of beam management without beam-related indication (ref:Annex) where no spatial QCL parameters are higher layer configured

  - Present: Details on Slide 7.

  - Proposed candidate solutions should consider

    - Below and above 6GHz DL beam related operation with and without beam indication

    - Downlink beam management with and without beam indication (ref Annex)

- Note: this proposal does not apply to the case of beam management without beam-related indication (ref:Annex)

The remaining critical open issues:

- The issue is if the state Is-TCI-Present configured on a per-CORESET basis or on a per-UE basis

- Details of higher-layer signalling of QCL parameters/indication for determining QCL parameters.

Contribution summary on configuration of Is-TCI-Present:

- On a per-CORESET basis (Samsung, MediaTek, Qualcomm, vivo, HW, Sharp)

- On a per-UE basis (  OPPO [some default/fallback DCIs may not contains the field, but all the other DCIs should contains the field],)

Contribution summary on indication of QCL if TCI is absent on per-CORESET/per-UE basis

- For CORESET configured with TCI not present in DL-related DCI, a reference TCI state (provided by higher-layer signaling) is used to obtain the QCL parameters for PDSCH scheduled on that CORESET. (Samsung)

Possible Agreement:

- The state Is-TCI-Present is configured on a per-CORESET basis

- NR concludes on the behavior of upper layer indication or signaling when the Is-TCI-Present is false.

Agreement:

- The state Is-TCI-Present is configured on a per-CORESET basis

- For beam management with beam indication, on all CORESETs configured with Is-TCI-Present=false, the TCI state used for PDCCH is reused for PDSCH reception

## Provision of spatial QCL for CSI-RS (periodic, semi-persistent, aperiodic)

Agreement last meeting:

Agreement:

SP CSI-RS is activated with RRC + MAC CE

The remaining critical open issues:

- The agreement from RAN1#90b on activation of semi-persistent (sp)-CSI-RS by MAC-CE is requires clarification of whether MAC-CE activates a set of sp-CSI-RS resources as a whole, or it activates selected resources from the set.

- Mechanism to indication of source QCL for:

  - P-CSI-RS

  - SP-CSI-RS

  - AP-CSI-RS

- Whether the same TCI table for PDSCH and CSI-RS should be used?

Offline Agreement:

- Mechanism to indication of source QCL for a resource:

  - P-CSI-RS – through RRC configuration

    - FFS: If the spatial QCL can be configured through a reference to a configured TCI state

  - SP-CSI-RS – configuring the resource(s) through RRC, activation/deactivation through MAC-CE;

    - The QCL for SP- CSI-RS is indicated in the same MAC-CE message that activates the SP- CSI-RS.

    - The QCL is provided through an association with one of the M candidate TCI states

  - AP-CSI-RS –

    - Alt 1: the QCL is provided through a TCI state indication using DCI

    - Alt 2: through AP-CSI-report-triggering state indication using DCI

Agreement:

- Mechanism to indication of source QCL for a resource:

  - P-CSI-RS – through RRC configuration

    - FFS: If the spatial QCL can be configured through a reference to a configured TCI state

  - SP-CSI-RS – configuring the resource(s) through RRC, activation/deactivation through MAC-CE;

    - The QCL for SP- CSI-RS is indicated in the same MAC-CE message that activates the SP- CSI-RS.

    - The QCL is provided through an association with one of the M candidate TCI states

  - AP-CSI-RS –

    - Through DCI (AP-CSI-report-triggering state indication)

      - For each AP-CSI-RS resource associated with each triggering state, QCL configuration is provided through an association with one of the M candidate TCI states by RRC

        - FFS: Value of M

      - FFS: TCI association on NZP-CSI-RS/ZP-CSI-RS based IMR

## Configuration of SRS for UL beam management

## Agreement in last meeting:

- For UL beam management,

  - NR supports gNB configuration of transmitting SRS with same Tx beam across multiple symbols via either of followings

    - configuring one SRS resource spanning multiple symbols

    - configuring UE to apply the same Tx beam across the SRS resources in a SRS resource set.

  - UE can apply different Tx beams to different SRS resources if it is not configured to apply the same Tx beam across SRS resources in a SRS resource set, where the beams can be determined either (1) via a gNB-transparent way, or (2) via gNB indication.

## Agreement in email discussion:

- FFS the inclusion of the parameter Apply_Same_SpatialFilter_Mult-SRS-Resource.

The remaining critical open issues:

- Configuring the parameter “Apply_Same_SpatialFilter_Multi-SRS-Resource” to a set of SRS resources for UL beam management

Contribution summary:

- Supporting parameter “Apply_Same_SpatialFilter_Multi-SRS-Resource”. (Samsung, Mediatek, Vivo, HW)

- No need to support parameter “Apply_Same_SpatialFilter_Multi-SRS-Resource” (OPPO, ZTE)

- The relationship between SRI indication and Spatial-relation indication

  - SRI indication: the same Tx beam of reference SRS is applied to the target SRS.

  - UE can sweep neighbouring beams on different SRS resources in a SRS resource set.

Possible Agreement:

## ??

## Provision of spatial relationship for SRS, PUCCH and PUSCH

Agreement last meeting:

Agreement:

- NR adopts the SRS Tx beam indication, i.e., by a SRS resource or by a DL RS

  - The DL RS supported at least include CSI-RS and SSB.

- NR supports the indication of at least the spatial relations between the DL RS and the UL SRS Tx beam via at least the following mechanisms.

| Spatial parameter | Reference RS | Target RS | Signalling mode |
| --- | --- | --- | --- |
| Spatial | SSB/CSI-RS (at least P-CSIRS and SP -CSI-RS), P-SRS FFS: AP-CSI-RS, SP-SRS | P SRS | RRC |
| Spatial | SSB/CSI-RS(at least P-CSIRS and SP -CSI-RS), P-SRS/ SP-SRS FFS:AP-SRS, AP-CSI-RS | SP-SRS | RRC + MAC-CE |
| Spatial | SSB/CSI-RS (at least P-CSIRS and SP -CSI-RS), P-SRS, SP-SRS, AP-SRS Working assumption: AP-CSI-RS | AP SRS | RRC or RRC+MAC CE for configuration, indication with DCI |

FFS: The use of spatial relation across CCs and/or BWPs.

The remaining critical open issues:

- How is the signalling of spatial QCL for SRS, PUCCH and PUSCH achieved?

- Is it only through higher layer signalling or is DCI based signalling included?

- Is the TCI state framework reused?

Contribution summary:

- Support to configure UL TCI states and indicate spatial QCL per PUCCH resource using UL TCI by RRC, similar with DL (HW, Samsung)

- Support to configure UL TCI states to indicate spatial QCL via following (Samsung)

  - Provide UL-TCI field in DCI triggering AP SRS

  - Provide UL-TCI field in MAC-CE activating SP SRS

Agreement:

PUCCH beam indication is introduced by RRC signalling

- Introduce one RRC parameter: PUCCH-Spatial-relation-info

  - Information associating an SSB ID or, a CRI, or a SRI

  - This is per PUCCH resource configuration

## Repetition of CSI-RS for P2/P3 procedures

Agreement last meeting:

Agreement:

Working assumption from RAN1#90 is confirmed:

For beam management CSI-RS, NR supports higher layer configuration of a set of single-symbol CSI-RS resources where

The set configuration contains an information element (IE) indicating whether repetition is “on/off”

Note: In this context, repetition “on/off” means:

“On”: The UE may assume that the gNB maintains a fixed Tx beam

“Off”: The UE cannot assume that the gNB maintains a fixed Tx beam

Note: This does NOT necessarily mean that the CSI-RS resources in a set occupy adjacent symbols

Furthermore, the following details are agreed

CSI-RS resources in the resource set are TDMed if repetition is ON

If repetition is ON, The UE does not expect different values for the following parameters across different CSI-RS resources within a resource set

Transmission periodicity

Number of antenna port subject to RAN4 decision

FFS for other parameters

Agreement:

NR supports the following configurations for beam management where a resource set is formed from multiple beam management CSI-RS resources and is contained within a resource setting:

Single resource set with repetition = “OFF”

UE reports CSI-RS resource indicator(s) within this resource set for CRI feedback

Single resource set with repetition = “ON”

UE does not report CRI

FFS: Further support additional configuration by down selection from the following two alternatives:

(a) Multiple resource sets, all with repetition = “ON”

UE reports CSI-RS resource set indicator(s) for CRI feedback

      - FFS: Whether set ID(s) are local within a resource setting or global across all resource settings

(b) Multiple equal-size resource sets, all with repetition = “OFF”

UE reports distinct local CSI-RS resource indicator(s) within one or more resource sets. The UE can assume that the gNB applies the same Tx beams in the same order for each of the sets

Note: Not all configurations are applicable for P1/P2/P3

FFS: Dimensioning of the bit width of the UCI field which can carry either CRI(s) or CSI-RS resource set indicator(s)

The remaining critical open issues:

- Sub-selection from the above options for FFS.

- Association of resource set ID(s) with a TCI state

Contribution summary:

- Multiple resource sets, all with repetition = “ON” (Qualcomm, Huawei, ZTE, , Samsung, CATT, OPPO [second preference], MediaTek[second preference])

- Multiple equal-size resource sets, all with repetition = “OFF” (Intel, Docomo )

- Neither option is required (Ericsson, vivo, OPPO [first preference], MediaTek, Sharp)

Possible Agreement:

- Multiple resource sets, all with repetition = “ON”

UE reports CSI-RS resource set indicator(s) for CRI feedback

    - The reported set ID(s) are local within a resource setting

- Supported by: LG, ZTE, Samsung, Huawei, CATT, OPPO, MediaTek, ITRI

- NR supports the association of a global resource set ID(s) only with repetition ON with a TCI state, including scenarios with a single configured resource set with repetition=”ON”

Agreements:

- For both CSI acquisition and beam management, an aperiodic Resource Settings can contain more than one CSI-RS Resource set

- For CSI acquisition, periodic and semi-persistent Resource Settings contain only one CSI-RS Resource set

The remaining critical open issue:

For beam sweeping purpose, each CSI-RS resources within a slot occupies a OFDM symbol. But the available OFDM symbols within a slot, which can be used for beam training, is limited. Currently there is no such configuration for multi-slot configuration of aperiodic CSI-RS, which is very essential use case for beam management like stages P2 or P3.

Contribution summary:

- (Qualcomm) For aperiodic CSI-RS triggering offset X, X = 0 is not applicable if the aperiodic CSI-RS triggering includes spatial QCL information.

- (Ericsson) For the beam management use case, consider S = 8 CSI-RS resources sets per resource setting, and Ks = 2 – 32 aperiodic CSI-RS resources per resource set as a starting point.

- (Huawei) Consider to have slot level time offset for aperiodic CSI-RS transmission for beam management

- Possible Agreement:

## Priority rule for beam reporting

Agreement last meeting:

- None

The remaining critical open issues:

- When there are conflicts between multiple possible reports on a limited capacity UL channel -e.g. PUCCH, there need to be some dropping rules.

Contribution summary:

Huawei : Support the following dropping rules when beam reporting and/or CSI reporting conflicts on short PUCCH and the total payload exceed the maximal PUCCH payload:

- AP beam reporting vs P CSI reporting and/or P beam reporting vs AP CSI reporting

AP beam reporting > P CSI reporting;

AP CSI reporting > P beam reporting.

- AP beam reporting vs P beam reporting

AP beam reporting > P beam reporting.

- AP/P beam reporting vs CSI reporting

Beam reporting part 1 > CSI reporting> beam reporting part 2.

Intel: The priority rule of PUCCH for beam reporting and SRS should be the same as that of PUCCH for CSI reporting and SRS

Vivo: Joint reporting at the same resource should be possible if the resource is enough for the simultaneously reported payload.

Possible Agreement:

## Implicit update of TCI states

Agreement last meeting:

Agreement

Support at least the explicit approach for the update of spatial QCL reference in a TCI state.

- FFS: Additional support for implicit update.

- Note: In the explicit approach, the TCI state is updated using either RRC or RRC + MAC-CE based approach

- Note: In the implicit approach, when a set of aperiodic CSI-RS resources are triggered, the triggering DCI includes a TCI state index which provides spatial QCL reference for the triggered set of CSI-RS resources. Following the measurement, the spatial QCL reference in the RS set corresponding to the indicated TCI state is updated based on the preferred CSI-RS determined by the UE. Other operations of implicit approaches are not precluded.

The remaining critical open issues:

- Support of the implicit TCI state update after a report.

Contribution summary:

- Support of the implicit TCI state update after a report. (Ericsson, Qualcomm, Vivo)

- Do not support – OPPO, Intel, InterDigital, HW, Sharp

Possible Agreement:

## Beam reporting issues (differential RSRP, quantities in addition to RSRP, beam measurement configuration)

Agreement last meeting:

- For non-grouping based beam reporting, support the following reports parameters:

  - Maximal number of configured Tx beams for beam measurement: K equals 64

  - Maximal number of configured Tx beams to be reported in one instance: N_max = 2, 4  where a subset of N (N<=N_max where N = 1, 2, 4) beams can be selected by the gNB and indicated to the UE (FFS signaling mechanism)

  - Reporting differential L1-RSRP when multiple beams are reported in one reporting instance. Reference is the largest L1-RSRP in that reporting instance. FFS other reference for differential reporting.

    - FFS applicable reporting channels and number of beams, and associated reporting contents

    - FFS: the UE adjusts the L1-RSRP of multiple RS resources according to the power offset between them

  - Bit-width: 7bit for L1-RSRP ranging from -140dBm to -44dBm with 1dB stepping size (analogous with LTE) and 4bit for differential L1-RSRP

    - FFS stepping size of differential quantization

  - gNB selection of a subset of N beams (DOCOMO):

    - Add an RRC signaling to turn on or turn off gNB selection of a subset of N beams for the UE to measure and report

      - When turned on, the UE reports M (M<=N) beams sQCLed with beam failure detection RS, and the rest N-M beams with highest RSRPs.

      - When turned off, the UE reports N beams with highest RSRPs.

The remaining critical open issues:

- Ordering of the RSRP for differential grouping (from strongest to weakest)

- Differential RSRP around the average ? (IDCC)

- Inclusion of values for report other than L1-RSRP – e.g. RI (ZTE, HW), PMI/RI – (Huawei?)

- Computation of L1-RSRP as a linear average of each port’s RSRP for the 2-port CSI-RS for beam mgmt..

- Support of differential RSRP on short/long PUCCH and PUSCH, value of N supported on short/long PUCCH and PUSCH (CATT)

- Configurable stepping size of differential quantization, e.g. 1 or 2dB (HW)

- Including out-of-range at least for 4-bit differential RSRP reporting (HW)

Possible Agreement:

- Ordering of the RSRP for differential grouping (from strongest to weakest).

- Computation of L1-RSRP as a linear average of each port’s RSRP for the 2-port CSI-RS for beam mgmt..

- FFS: Values other than L1-RSRP, CRI/SSBI for beam report.

Proposal:

- In beam reporting, NR supports the following aspects:

  - UE can be configured with K DL RS resources (or sets) for beam reporting (including group and non-group based) where K={1,2,..,64}.  The bitwidth of reported CRI depends on the number of configured resources (or sets).

    - Support UE reports its capability of K

  - L1-RSRP is reported per selected RS resource in report instance

    - L1-RSRP is determined per reported Tx beam for both non group based beam reporting and group based beam reporting.

      - The same mechanism of differential RSRP as non-group based beam reporting is supported for group based reporting.

- Configurable stepping size of differential quantization, e.g. 1 or 2dB (HW)

Offline proposal:

Differential RSRP is computed between adjacent entries in the ordered set of reported beams

## Spatial QCL latency

Agreement last meeting:

- No agreements yet.

The remaining critical open issues:

- Range of values to be expected at the UE for the latency between signalling of spatial QCL and its application.

Possible values:

For 120kHz Subcarrier Spacing.

| Parameter | Possible values |
| --- | --- |
| Minimum time between the transmission of the indication of the spatial QCL for the reception of AP-CSI-RS and aperiodic CSI-RS for beam management shall be at least KB symbols. Symbols measured from last symbol containing the indication to first symbol of CSI-RS. - For further discussion on the scenarios e.g. whether UE beam is changed or not changed from the last symbol containing the indication to the first symbol containing the CSI-RS - Note: The latency may include other latencies e.g. DCI decoding latency - Note: This is the minimum time required by the UE . | [14], [26] |
| The number of symbols between CSI-RS for beam management and the associated report shall be at least RB. Note: The start is calculated from the last symbol of the CSI-RS used for the measurement report and end time is calculated to be the first symbol of the associated report Note: CSI-RS maybe for P1/P2/P3 procedures | Alt 1: [14,28, 42] Alt2: Same as minimum CSI acquisition time |
| The minimum number of symbols between a DCI with spatial QCL for PDSCH and the start of the associated PDSCH TTI shall be at least DB | Options: [14], [26], [28] |
| The minimum number of slots between transmission of the spatial QCL for PDCCH and the application of the PDCCH beam switch shall be at least SB assuming MAC-CE based messaging. | Alt 1: 4, 8, 12, 20 (Values for UE capability) Alt2: Based on HARQ |
| Maximum number of Rx and Tx beam changes a UE can conduct during a slot shall not exceed BB | Options: 7, Every symbol |
| The number of measurement reports with L1-RSRP only per slot, per TRP, per component carrier shall not exceed C1B | 1 |
| The max number of CSI resources to measure L1-RSRP within a slot shall not exceed MB | [16], [64] |
| Minimum time between aperiodic SRS for beam management and its DCI shall be at least D1 symbols. Symbols measured from last symbol containing the DCI to first symbol of SRS. | Options: [14], [26], [42] |
| Maximum index of SRI for beam indication is D2 | [1], [4] |

Possible Agreement:

## UE triggered beam measurement and/or reporting

Agreement last meeting:

- No prior agreement

The remaining critical open issues:

- Should a UE triggered method for beam training request or beam reporting be included (e.g. event based)?

Contribution summary:

- UE to support a method to signal request for beam training (Qualcomm) or beam report (Qualcomm, Apple, vivo) or terminate current beam training(Huawei) based on UE events

Possible Agreement:

## Beam Management without Beam Indication

Previous agreements

Agreement: (RAN1#86bis):

- For downlink, NR supports beam management with and without beam-related indication

  - When beam-related indication is provided, information pertaining to UE-side beamforming/receiving procedure used for data reception can be indicated through QCL to UE

    - FFS: Information other than QCL

  - FFS: When beam-related indication is provided, information pertaining to the Tx beam used for data transmission is indicated to UE

Agreement: (RAN1#87):

- NR supports with and without a downlink indication to derive QCL assumption for assisting UE-side beamforming for downlink control channel reception

  - FFS: details

    - E.g., QCL assumption details

    - E.g., indication signaling (e.g. DCI, MAC CE, RRC, etc.)

    - E.g., beam-related indication for DL control and data channels

Agreement: (RAN1 NR Ad Hoc #1):

- For downlink, NR supports CSI-RS reception with and without beam-related indication,

  - When beam-related indication is provided, information pertaining to UE-side beamforming/receiving procedure used for CSI-RS-based measurement can be indicated through QCL to UE

  - QCL information includes spatial parameter(s) for UE side reception of CSI-RS ports

    - FFS: information other than QCL

Agreement: (RAN1#89):

- Support spatial QCL assumption between antenna port(s) within a CSI-RS resource(s) and antenna port of an SS Block (or SS block time index) of a cell

  - The other QCL parameters not precluded

  - FFS: indication either explicit or implicit or configurable or a default

  - Note: default assumption may be no QCL

Remaining critical open issues:

- Configuration of TCI framework for the case of no beam indication

- P3 procedure for the case of no beam indication

- Configuration of default QCL assumptions

Contribution summary:

- Ericsson: configuration of single TCI state, no TCI in DCI, CSI-RS set ID for P3 as QCL reference

- Samsung: UE assumes that its UE-specific PDSCH, PDCCH and CSI-RS (if configured) are QCL-ed with the SSB/PBCH determined during its random access procedure.

Possible agreement: