3GPP TSG RAN WG1 Meeting #91                                          R1-1720230

Reno, USA, 27th November – 1st December 2017

Source: 	ETRI

Title:	Calibration results for Phase 2 NR MIMO link level simulation

Agenda Item:	7.2.4   Other

Document for:	Discussion

Introduction

In this contribution, we provide Phase-2 NR MIMO link-level simulation (LLS) calibration results at 4GHz and 30GHz carrier frequency.

Phase-2 LLS calibration results

In Table 1, evaluation assumptions for Phase-2 LLS calibration are described [1]. Further clarifications on some parameters can be found in [2].

Table 1. Simulation assumptions for Phase 2 link-level calibration

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 4 GHz | 30 GHz |
| Subcarrier Spacing | 15kHz | 60kHz |
| Data allocation | 8 RBs - Note: Error free PDCCH decoding is assumed. - First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel - One-port DMRS (12 REs) |  |
| Channel Model | CDL-A and CDL-B - delay spread =100ns - UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly. |  |
| TXRU mapping to antenna elements | One TXRU per vertical dimension per polarization | One TXRU per panel per polarization |
| TXRU mapping weights | TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT, i.e., 1D sub-array partition model defined in TR36.897 | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming1). |  |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,4,2,1,1). (dV,dH) = (0.5, 0.5)λ | (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ |
| BS antenna element radiation pattern | According to TR36.873 | See Table A.2.1-6 in TR 38.802 |
| UE antenna configurations | (M,N,P,Mg,Ng) = (1,1,2,1,1) | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. 2) Θmg,ng=90; Ω0,1=Ω0,0+180; |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |  |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 0 degree, ΩUT,g = 0 degree |  |
| UE antenna element radiation pattern | Omnidirectional | See Table A.2.1-8 in TR 38.802 |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |  |
| MIMO mode | SU-MIMO with rank=1 |  |
| UE receiver type | MMSE-IRC |  |
| MCS | LTE MCS = 1 |  |
| DMRS channel estimation | 2DMMSE Based on LTE antenna port 7 |  |
| Metrics | BLER w/ beamforming as a function of transmission SNR ranging from -20 dB to 0dB |  |

- Note1: the DFT beam candidate in this beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows:

, for i=1,…,rN

where r=1 (which is analogous to oversampling factor of 1), N denotes the number of vertical/horizontal antennas.

- Note2: The polarization angles are 0 and 90 and the panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels for above 6GHz. (Ωmg,ng, Θmg,ng)  is for orientation of the panel (mg, ng), 0≤mg<Mg, 0≤ng<Ng,  where the orientation of the first panel (Ω0,0, Θ0,0) is the same as UE orientation, Ωmg,ng is the array bearing angle and Θmg,ng is the array downtilt angle defined in [TR 36.873].

Based on the evaluation assumptions in Table 1, BLER results at 4GHz and 30GHz carrier frequency are shown in Fig. 1 and Fig. 2, respectively.

Fig. 1. BLER results at 4GHz carrier frequency

Fig. 2. BLER results at 30GHz carrier frequency

Conclusion

In this contribution, calibration results for Phase-2 NR MIMO LLS on 4GHz and 30GHz carrier frequency are provided.

References

[1] R1-1703535, “Evaluation assumptions for Phase 2 NR MIMO link level calibration,” ZTE, 3GPP RAN1 #88, February 2017.

[2] R1-1701517, “Summary of email discussion [87-30] on NR-MIMO calibration,” ZTE, 3GPP RAN1 NR Ad-Hoc #1, January 2017.