- 3GPP TSG RAN WG1 Meeting #91	                                     R1-1720569

Reno, USA, November 27th – December 1st , 2017

Source:	ZTE, Sanechips

Title:	Baseline evaluation results for LTE aerials

Agenda Item:	6.2.7.1

Document for: 	 Discussion and Decision

- Introduction

For evaluating the performance of LTE with UE aerial vehicles, the following evaluation assumptions and key performance indicator are achieved [1]:

- Packet throughput

  - UL and DL packet throughput statistics of all UTs Data traffic

  - UL and DL packet throughput statistics of aerial UTs Data traffic

  - UL and DL packet throughput statistics of terrestrial UTs Data traffic

- Interference

  - UL IoT (interference over thermal) and DL wideband SINR statistics for reference

  - NOTE: UL IoT above refers to effective IoT defined in clause A.2.1.8 of [6]

- Reliability as defined in [10] for evaluation of C&C traffic for aerial UTs with X = 1250 bytes and L = 50 ms, wherein X and L are defined in [10]

- Other metrics are not precluded

Moreover, the details of channel model are agreed in RAN1#90 at least for baseline simulation and the following agreement and conclusion based on the 5%ile geometry results collected from multiple contributions are drawn [2]:

- For aerial UE uniformly distributed between 1.5 m and 300 m in UMa-AV, it is observed from the 5%ile geometry results that aerial UEs statistically experience worse downlink geometry than terrestrial UEs

  - The degradation is due to more downlink inter-cell interference from multiple cells

- Capture the above conclusion in the TR along with results in Slide 3, 4, and 5 of R1-1714675 in TR.

  - Companies are encouraged to update the geometry results with fast fading.

In this contribution, the baseline simulation results for UL/DL throughput and UL IoT statistics for UMa scenarios are updated based on the agreed assumptions [2].

- Simulation Results

- UL baseline result

Based on the agreed simulation assumption mentioned above, the statistics of IoT and throughput for UL are listed in Table 1 and Table 2, respectively.

UL IoT:

According to following results, it can be observed that the IoT for both terrestrial and aerial UEs are dramatically enlarged with the increasing number of aerial UE in each cell. It demonstrates that the scheduling of one aerial UE will introduce significant interference for the rest UEs in adjacent cell. More specially, we can find that the IoT for terrestrial UE is relative stable in case 1~3, which means that the influence of aerial UE is tolerable if there is only one aerial UE per cell. Besides, with increasing the traffic load, the IoT is also enlarged since more UEs are scheduled simultaneously.

Table 1 Statistics of UL IoT for UMa-AV

|  | Terrestrial UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RU | 0.2 |  |  |  |  | 0.5 |  |  |  |  |
| Effective IoT | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| 5% | 0.11 | / | 0.27 | 0.07 | 0.31 | 2.19 | / | 1.45 | 4.01 | 3.78 |
| 50% | 2.17 | / | 3.41 | 3.29 | 5.83 | 6.40 | / | 5.54 | 12.95 | 13.26 |
| 95% | 7.81 | / | 11.14 | 16.62 | 18.69 | 11.33 | / | 12.78 | 21.03 | 21.84 |
|  | Aerial UEs |  |  |  |  |  |  |  |  |  |
| 5% | / | / | 0.15 | 0.04 | 0.14 | / | / | 0.30 | 2.06 | 2.86 |
| 50% | / | / | 1.75 | 1.36 | 8.03 | / | / | 2.66 | 11.23 | 12.91 |
| 95% | / | / | 6.19 | 13.98 | 19.44 | / | / | 9.37 | 20.49 | 21.48 |

UL Throughput:

Based on these results, similar conclusion w.r.t the increasing number of aerial UE in one cell can be achieved as above. More specifically, the performance of both terrestrial and aerial UEs degrades when the number of aerial vehicles increases.

Table 2 Statistics of UL throughput for UMa-AV

|  | Terrestrial UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RU | 0.2 |  |  |  |  | 0.5 |  |  |  |  |
| Throughput CDF | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| 5% | 2.50 | / | 1.57 | 1.81 | 0.90 | 1.25 | / | 0.79 | 0.59 | 0.56 |
| 50% | 12.23 | / | 10.01 | 10.39 | 7.92 | 8.08 | / | 6.56 | 2.98 | 2.21 |
| 95% | 27.40 | / | 26.77 | 25.64 | 26.67 | 22.68 | / | 19.37 | 14.11 | 12.95 |
|  | Aerial UEs |  |  |  |  |  |  |  |  |  |
| 5% | / | / | 12.77 | 9.91 | 4.09 | / | / | 5.04 | 1.16 | 0.95 |
| 50% | / | / | 25.53 | 22.54 | 16.60 | / | / | 18.22 | 6.99 | 5.81 |
| 95% | / | / | 31.00 | 32.51 | 31.06 | / | / | 30.35 | 20.98 | 20.42 |

Observation 1: The IoT of both terrestrial and aerial UEs are enlarged with increasing number of aerial UE.

Observation 2: The UL throughput of both terrestrial and aerial UEs degrades with increasing number of aerial UE.

- DL baseline result

DL Throughput:

In downlink case, the statistics of throughput are listed in Table 3. Comparing with UL, it can be observed that the performance of terrestrial UE is more stable regardless the number of aerial UE in each cell. It is reasonable since the similar distribution of SINR can be observed for terrestrial UEs in all cases, e.g., 5%ile SINR in both case 1 and 5.

However, for aerial UEs, which are more vulnerable to inter-site interference, the performance degrades with the increasing number of aerial. Moreover, the total throughput is much less than terrestrial UE due to the lower SINR.

Table 3 Statistics of DL throughput in UMa-AV

|  | Terrestrial UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.2 |  |  |  |  | 0.5 |  |  |  |  |
| Throughput CDF | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| 5% | 3.31 | / | 2.93 | 3.25 | 1.65 | 1.75 | / | 1.52 | 1.37 | 1.12 |
| 50% | 22.22 | / | 22.47 | 22.10 | 16.99 | 13.31 | / | 12.53 | 12.31 | 9.46 |
| 95% | 56.34 | / | 56.34 | 56.34 | 56.34 | 50.02 | / | 46.29 | 51.82 | 48.33 |
|  | Aerial UEs |  |  |  |  |  |  |  |  |  |
| 5% | / | / | 1.43 | 2.31 | 1.33 | / | / | 0.97 | 0.64 | 0.69 |
| 50% | / | / | 8.01 | 7.17 | 5.92 | / | / | 2.64 | 2.34 | 2.21 |
| 95% | / | / | 43.36 | 33.03 | 25.15 | / | / | 19.17 | 16.36 | 11.70 |

Observation 3: The DL throughput of terrestrial UE slightly degrades with increasing number of aerial UEs.

Observation 4: The DL throughput of aerial UEs is poor due to the significant inter-site interference.

Proposal 1: Update the TR via capturing the latest results in this contribution.

Proposal 2: Interference mitigation schemes should be considered for UL transmission of both aerial UE and terrestrial UE.

- Conclusions

In this contribution, the simulation results for geometry, UL/DL Throughput and IoT are provided with following observations and proposals:

Observation 1: The IoT of both terrestrial and aerial UEs are enlarged with increasing number of aerial UE.

Observation 2: The UL throughput of both terrestrial and aerial UEs degrades with increasing number of aerial UE.

Observation 3: The DL throughput of terrestrial UE slightly degrades with increasing number of aerial UEs.

Observation 4: The DL throughput of aerial UEs is poor due to the significant inter-site interference.

Proposal 1: Update the TR via capturing the latest results in this contribution.

Proposal 2: Interference mitigation schemes should be considered for UL transmission of both aerial UE and terrestrial UE.

- References

Chairman’s notes for RAN1#90

R1-1714859, “TP for key performance indicator, identified problem, evaluation assumptions, channel modelling, and evaluation results”