3GPP TSG RAN WG1 Meeting #91                                                                 R1-1720806

Reno, USA, November 27th – December 1st, 2017

Source:	NTT DOCOMO

Title:	Remaining details on BM and CSI framework

Agenda Item:	7.2.2.6

Document for: 	Discussion

# Introduction

At the last meeting, there were intensive discussions on CSI acquisition schemes including CSI-RS design and CSI reporting. Several agreements were reached as follows [1].

R1-1718910 Summary of CSI measurement ZTE, Sanechips Agreement: For ZP CSI-RS based IMR, following combinations of P/SP/AP CMR and IMR are supported. For semi-persistent CSI reporting, PR CMR; SP CMR; AP CMR. PR IMR; YES; NO; NO. SP IMR; NO; YES; NO. AP IMR; NO; NO; NO. For aperiodic CSI reporting, PR CMR; SP CMR; AP CMR. PR IMR; YES; NO; NO. SP IMR; NO; YES; NO. AP IMR; NO; NO; YES. Agreement: At least the following patterns are supported for ZP CSI-RS for IMR: One pattern from (2,1), (2,2), (4,1) R1-1719005 Offline Summary of CSI measurement ZTE, Sanechips Agreement: - Configuration of resources for rate matching is separate from configuration of resources for interference measurement. Working Assumption: For ZP CSI-RS based IMR, support (2,2) and (4,1) which are configurable by RRC signalling - Note: Study until next meeting whether support of both (2,2) and (4,1) are both necessary. If not, RRC configurability will be removed. R1-1719186 WF on aperiodic CSI-RS triggering ZTE, Sanechips, Samsung, LG Electronics, NTT DOCOMO, Qualcomm, Intel, ASTRI, Verizon, KDDI, IITH, CEWiT, IITM, Tejas Networks, CHTTL, Sharp, NEC, MediaTek, KT Corporation, Nokia, NSB Working Assumption: - Aperiodic CSI-RS is triggered by RRC+DCI or RRC+MAC-CE+DCI - If number of RRC configured resource sets across all CCs is less than 2N, RRC+DCI is used. Otherwise, RRC+MAC-CE+DCI is used. - FFS on the value of N - The triggering is done per CSI-RS resource set Email discussion on the value of N for aperiodic CSI-RS is triggering until 27th of Oct – Ruyue (ZTE) Agreement: For non-PMI feedback, support the following port index indication method: - Port index indication is signaled to UE for RI/CQI calculation in non-PMI feedback - Port index indication per CSI-RS resource is configured by RRC to select the CSI-RS port(s) used for RI/CQI calculation per rank - Identity matrix is assumed by UE on the selected CSI-RS ports for RI/CQI calculation - N ports are selected for rank N - The CSI-RS resource can be dynamically selected for CSI reporting in CSI framework R1-1719178 WF on NZP CSI-RS based interference measurement Huawei, HiSilicon, Qualcomm, Ericsson, IITH, Intel, Spreadtrum, OPPO, China Unicom, vivo, Interdigital, IITM, Deutsche Telekom, CeWiT, KDDI, NEC, Vodafone, CMCC, Sharp, Tejas Networks, ASUSTeK, Softbank, Sony, CATR, China Telecom, III, Mitsubishi, ZTE, Sanechips, CATT, Samsung Agreement: - A set of NZP CSI-RS resource(s) is configured to a UE for channel and interference measurements, where - A subset of the set of NZP CSI-RS resource(s) are for channel measurement and another subset of the set of NZP CSI-RS resource(s) are for interference measurement - Network indicates via DCI the subset of NZP CSI-RS resource(s) for channel measurement and the subset of CSI-RS resource(s) for interference measurement - FFS: Whether the DCI indication is the dynamic triggering of one or multiple CSI reporting setting(s) or not - FFS: some CSI-RS resource(s) from two NZP CSI-RS resource subsets can be overlapped or not - UE assumes each port of channel measurement NZP CSI-RS resource(s) corresponds to a desired layer if no PMI and RI feedback

In this contribution, we present our views on further details on CSI framework. This is a revision of R1-1716084.

# Discussion

## 2.1 Maximum values for CSI framework

Figure 1 shows possible procedures of beam determination for higher frequency bands that is performed during initial access, beam management and CSI acquisition. In the initial access agenda, it has been agreed that the maximum number of SS-blocks (SSBs) within SS burst set for the frequency above 6 GHz is 64. In order to reduce UE complexity, we need to reduce the number of beams that is measured for beam management. In example 1, eight beams are selected for beam management that is determined based on SSB. And then CSI acquisition is performed based on one selected beam from beam management. Width of the beams for SSBs, beam management (SSBs or CSI-RSs) and CSI acquisition (CSI-RS) can be the same in this example. As another example, in example 2, we assume less number of SSB beams with broader beam width considering SSB overhead. For beam management, we can apply narrower beams to achieve higher resolution control compared to L3 based beam control. Table I and Figs. 2 show possible parameter values that determines whole picture of CSI framework. In order to perform efficient beam management without frequent RRC re-configuration, we need to have at least eight (=64/8) CSI-RS resource sets as presented in the top two figures in Figs. 2. The value can be larger when we allow overlap of CSI-RS resource among different CSI-RS resource sets, e.g., 21.

Proposal 1: The value of Ks should not be smaller than 8.

Proposal 2: The value of S should not be smaller than 8, e.g., 21.

Figure 1: Procedures for beam determination

Table I: Example configuration of beam management and CSI acquisition

Figure 2: The number of S and KS

## 2.2 Triggering mechanism of semi-persistent CSI-RS and CSI reporting

In the previous meetings, there were discussions on combinations of CSI-RS transmission and CSI reporting schemes. We should strive for simple and common triggering mechanism for different combinations of time domain behaviors as much as possible. We should also consider reducing the overhead of DCI, since the number of DCI bit can’t be dramatically different for NR compared to LTE-A. For NR, aperiodic CSI-RS and CSI reporting is supported to obtain fine CSI for PDSCH scheduling. It has been agreed that the reporting is triggered with DCI in order to acquire CSI within coherence time. In addition, aperiodic CSI-RS can be also triggered with DCI that is common to reporting trigger.

Here, we need to consider about triggering mechanism for semi-persistent CSI reporting as well. There is tradeoff between two triggering options, i.e., DCI and MAC CE. MAC CE is advantageous in terms of reducing DCI overhead and signaling reliability, i.e., possible error propagation for DCI. In addition, MAC CE based scheme can have higher operational flexibility, since payload for MAC CE is larger than that for DCI. On the other hand, DCI is advantageous in terms of control delay. Here, in terms of the use cases, semi-persistent CSI acquisition can be used for similar functionality as that for periodic CSI acquisition by reducing overhead of CSI-RS and CSI reporting. In addition, it is also suitable for CSI acquisition using CSI-RS resource set switching (resource pool switching) as in Fig. 3. The resource set can be changed depending on UE movement that generally requires tracking mid-to-long-term channel characteristics. Considering those operations (with example parameter values identified in previous subsection) and another choice of aperiodic CSI acquisition, it is preferable that semi-persistent CSI acquisition is triggered with MAC CE.

Figure 3: Switching of CSI-RS resource set

Proposal 3: Semi-persistent CSI reporting is activated/deactivated with MAC CE.

If the delay of MAC CE is still a concern, the method in Fig. 4 can be also considered as discussed in the previous meeting [2]. In this method, the DCI for CSI reporting trigger can be also used to activate semi-persistent CSI-RS without introducing additional MAC CE/DCI bits. This method is applicable for the combination of semi-persistent CSI-RS and aperiodic CSI reporting. Finally, there might be an argument that activation of semi-persistent CSI-RS with two-steps, i.e., RRC + MAC CE, requires unnecessary delay as in Fig. 5(a). In this context, it may be beneficial that RRC configuration include default setup (activated/deactivated) for semi-persistent CSI-RS as in Fig. 5(b). Same functionality can be also introduced for semi-persistent CSI reporting.

Figure 4: Triggering of semi-persistent CSI-RS and aperiodic CSI reporting with reduced delay

(a) RRC + MAC CE                               (b) RRC with default setup

Figure 5: Initial activation of semi-persistent CSI-RS transmission

Finally, Table II summarizes discussion above.

Table II: Proposed summary for triggering mechanism on CSI-RS transmission and CSI reporting

Proposal 4: For the combination of semi-persistent CSI-RS and aperiodic CSI reporting, DCI for CSI reporting can also activates semi-persistent CSI as in Fig. 4.

Proposal 5: Semi-persistent CSI-RS can be RRC configured with default procedure (activated or deactivated).

Proposal 6: Semi-persistent CSI reporting can be RRC configured with default procedure (activated or deactivated).

## 2.3 DM-RS based CSI acquisition

In the previous meetings, we have agreed that CSI-RS is configured in resource setting as a channel measurement resource. We believe that CSI-RS is not an only option for CSI acquisition but DM-RS is also suitable and can be additionally used for CSI acquisition. Fig. 6 shows possible mechanisms for DM-RS based CSI acquisition. In this example, blue REs and green REs are allocated as DM-RS for own UE and that for co-scheduled UE (for MU-MIMO). In this case, DM-RS based CSI acquisition becomes effective, since influence of beamforming can be accurately accommodated when deriving CSI. In particular, it is very efficient that UE can measure MU-CSI with green REs, which usually requires large overhead on downlink RS, e.g., NZP CSI-RS, and feedback. On the other hand, there was an argument that DM-RS is not multiplexed in wideband and not enough to have accurate CSI. However CSI can derived with high accuracy thanks to denser RE mapping compared to CSI-RS. Furthermore, considering that partial band transmission is supported for CSI-RS, there is no strong reason for accuracy of CSI measurement. If DM-RS based CSI acquisition is triggered by DL grant, CSI can be fed back together with HARQ. We presented some simulation results on DM-RS-based CSI acquisition in our companion contribution [3].

Figure 6: CSI acquisition based using DM-RS

Proposal 7: DM-RS can be configured in the resource setting as channel measurement resource and interference measurement resource.

# Summary

In this contribution, we presented general views on CSI acquisition schemes for downlink beam management. Based on the discussion, we made the following proposals.

Proposal 1: The value of Ks should not be smaller than 8.

Proposal 2: The value of S should not be smaller than 8, e.g., 21.

Proposal 3: Semi-persistent CSI reporting is activated/deactivated with MAC CE.

Proposal 4: For the combination of semi-persistent CSI-RS and aperiodic CSI reporting, DCI for CSI reporting can also activates semi-persistent CSI as in Fig. 4.

Proposal 5: Semi-persistent CSI-RS can be RRC configured with default procedure (activated or deactivated).

Proposal 6: Semi-persistent CSI reporting can be RRC configured with default procedure (activated or deactivated).

Proposal 7: DM-RS can be configured in resource setting as channel measurement resource and interference measurement resource.

# References

- 3GPP, “Draft Report of 3GPP TSG RAN WG1 #90b v0.1.0,” Nov. 2017.

- 3GPP, R1-1706505, NTT DOCOMO, “WF on Triggering Mechanism for CSI Reporting,” Apr. 2017.

- 3GPP, R1-1720801, NTT DOCOMO, “Views on CSI measurement  for NR,” Nov. 2017.