3GPP TSG RAN WG1 Meeting #92	R1-1801330

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.1.4.1

Source:	Huawei, HiSilicon

Title:	Remaining details of RACH Preambles

Document for:	Discussion and decision

Introduction

The RACH configuration tables for paired spectrum and the long sequence of TDD FR1 have been agreed. In this contribution, we consider some proposed changes for the RACH configuration tables of long sequence for FDD and for short sequence of unpaired FR1 and FR2.

RACH configuration table design

Subframe number for preamble format 1 of FDD FR1

In RAN1#89, it was agreed that NR supports format 0 and format 1 of length 839 as follows:

| Format | L | SCS (kHz) | BW (MHz) | N_OS | N_RP | T_SEQ (Ts) | T_CP (Ts) | T_GP (Ts) | Format |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 | 1.08 | 1 | 1 | 24576 | 3168 | 2975 | LTE refarming |
| 1 | 839 | 1.25 | 1.08 | 2 | 1 | 2*24576 | 21024 | 21904 | Large cell, up to 100km |

It is noted that for format 1, the duration is (2*24576+21024+21904)/30720=3ms. However in current RACH configuration table for long sequence of FDD FR1, the candidate subframe numbers of format 1 contain 8 and 9, which will lead to overflow to the subsequent frames.  Thus, we propose to modify the working assumption and remove the entries for format 1 of subframe number larger than 7, as illustrated in the appendix A.

Proposal 1: Remove the entries for format 1 of subframe number larger than 7 in Table 6.3.3.2-2 and confirm the working assumption as in appendix A.

RACH configuration table for TDD of FR1 and FR2

Compared with FDD, the RACH configuration table for TDD FR1 and FR2 should consider the downlink resources (e.g. SS/PBCH block, RMSI) and semi-static DL/UL locations, in order to reduce the potential collisions between RACH transmission occasions (ROs) and SS/PBCH block/DL part.

In the symbol level, we propose to introduce starting symbols “7” in one RACH slot. In case that the subcarriers of PRACH and PUSCH are 15 KHz and 30 KHz for FR1 or 60 KHz and 120 KHz for FR2 respectively, one RACH slot will cover two UL slots. Introducing starting symbol “7” can benefit that only one UL slot is occupied instead of two, as illustrated in Figure 1. Then network has the freedom to assign the symbols in the first UL data slot as the DL or flexible (e.g., for DL and UL switching gap). Furthermore, if network has very limited number of SSBs, e.g., 1 or 2, there is no need for the network to allocate ROs covering the whole RACH slot. If network wants more RACH capacity, the frequency domain RO allocation is enough. Even if there are more SSBs, frequency domain RO allocation is also enough. With digital beamforming capability for network in FR1, FDMed RO could be associated with different SSBs.

Besides, if the SCS of PRACH and PUSCH is the same, based on the slot format Table 4.3.2-3 in TS38.211, most of the SFI configuration entries could support it. Specifically, for starting symbol “7”, if PRACH is transmitted on the UL symbols, total 16 entries are applicable (e.g., entries 10~15). If PRACH is transmitted on the flexible and UL symbols, total 33 entries are applicable (e.g., entries 8~27). Note that several entries with DL symbols at the second half of the slot is not desirable for PRACH transmission, which we could not consider such cases in the PRACH configuration table design, e.g. entries 46~55. Since limited RACH configuration entries, the starting symbol “2” should be deprioritized or not supported in the RACH configuration table. If “2” is supported, considering DL/UL switching gap, only 1 control symbol is allowed in front of the RACH resources.

Figure 1. Example of format C2 with UL SCS 30KHz

In the frame/subframe/slot level, in order to avoid the collision between ROs and SSB/RMSI CORESET. For TDD FR1, the following rules for the RACH configuration table should apply:

- The odd frame has fewer opportunities to be occupied by RMSI CORESET, since the starting time always starts from the even frame. The value of SFN mod Configuration Period is proposed to be 1 when RACH Configuration Period > 1(10ms).

- Since the SS/PBCH blocks can only occur in the front 4ms of a 5ms window, candidate locations of RO subframes (in terms of 15 KHz) in 4 and 9 should be prioritized.

Based on the discussion above, the RACH configuration table for unpaired spectrum and short sequence of FR1 is illustrated in Appendix B and we have the proposal below.

Proposal 2: For FR1 and unpaired spectrum, NR adopts PRACH configuration of short sequence as shown in appendix B.

- Introducing starting symbol “7” for format A1, A2, A3, C0, C2, B1, A1/B1, A2/B2, A3/B3

- SFN mod Configuration Period is 1 when RACH Configuration Period > 1(10ms)

- Prioritize RO subframes locations in 4 and 9

For TDD FR2 table,  NR should try to avoid the collision between RO and DL part in both large and small granularity. In large granularity, the semi-static DL/UL periodicity should be taken into consideration. We take the semi-static DL/UL periodicities of {10, 5, 2.5, 2, 1.25} ms as the baseline and the periodicities are illustrated in Figure 1. After each DL slot, one slot is left to consider the DL-UL switching and considering to allocate as much ROs as possible to ensure sufficient RACH capacity. As a result, the candidate slots that can be used for PRACH are {2, 3, 4, 12, 13, 14, 27, 28, 29, 37, 38, 39}.

Figure 2. Potential RO locations in time domain for semi-static DL/UL periodicities {10, 5, 2.5, 2, 1.25}ms

In a small granularity, NR should try to avoid the collision between RO and SSB/RMSI CORESET. For FR2, the RACH occasions are much more likely to collide with DL resources than FR1, because the number of SSB and associated RMSI is up to 64. The time domain candidate locations of SSBs and RMSI CORESETs with different numerologies and 20ms duration are shown in Figure 2, where the periodicity of SSB burst set is 5ms and the SSB and CORESET multiplexing pattern 1 is considered. For RMSI CORESET SCS=60kHz and 120kHz, the RMSI CORESET time locations are based on Table 13-9 and Table 13-10 in TS 38.213, respectively. From Figure 2, it can be found that the first half of an even frame and the second half of an odd frame have a relatively small possibility to collide with RMSI CORESET when the RMSI CORESET group offset X>=5ms and X<5ms, respectively. In order to prevent excessive collision with DL resources and improve the efficiency of random access, the following design criteria of PRACH configuration table for FR2 are proposed:

- For pattern 1, the RMSI CORESET and SSB occur in the different time locations.

  - The SFN of RACH occasion is determined by the value of RMSI CORESET group offset X carried in PBCH (Table 13-9~13-10 in 38.213).

- If X>=5ms, SFN mod Configuration Period = 0.

- If X<5ms, SFN mod Configuration Period = 1.

  - The slot index of RACH occasion is determined by the SFN.

- Case A: if SFN mod 2=0, the slots of RACH occasions can be selected from the slots exclude the SSB candidate locations in the first half of the frame. For SSB SCS=120 kHz, the candidate slot indexes are {4, 9, 14, 19} and will be prioritized; For SSB SCS=240 kHz, the candidate slot indexes are {4, 9~19}, which are also considered in the table.

- Case B: if SFN mod 2=1, the slots of RACH occasions can be selected from the slots exclude the SSB candidate locations in the second half of the frame. For SSB SCS=120 kHz, the candidate slot indexes are {24, 29, 34, 39} and will be prioritized; For SSB SCS=240 kHz, the candidate slot indexes are {24, 29~39}, which are also considered in the table.

- For SSB and CORESET multiplexing pattern 2 and pattern 3, the RMSI CORESET and SSB occur in the same slot.

  - Note that it is enough to just choose the slots configured for PRACH from the slots that are not occupied by the SSB candidate locations. As a result, the candidate RO slots can be selected from {4, 9, 14, 19, 24, 29, 34, 39} for SSB SCS=120 kHz. For SSB SCS=240 kHz, the candidate RO slots can be selected from {4, 9~19, 24, 29~39}. Therefore, {4, 9, 14, 19, 24, 29, 34, 39} will be prioritized in the table, and addition slots {10~13,15~18,30~33,35~38} will also be considered in the table.

  - Considering that the configuration designed for Pattern 1 can also be used for Pattern 2/3, since X is not configured in MIB, X is always assumed to be 0ms for simplicity, i.e., SFN mod Configuration Period = 1 and Case B configuration in the Table. It because that the RACH resources in Case B are on the latter part of the frame which is more favorable for the DL-UL configuration.

Figure 3. SSB and RMSI CORESET candidate locations

Based on the considerations described above, the RACH configuration table for unpaired spectrum and RF2 is shown in Appendix C, and we have the proposal below.

Proposal 3: For FR2 and unpaired spectrum, NR adopts PRACH configuration as shown in appendix C.

- Introducing starting symbol “7” for format A1, A2, A3, C0, C2, B1, A1/B1, A2/B2, A3/B3

- SFN of RO depends on the RMSI CORESET group offset X (X=0ms for RMSI pattern 2/3):

  - If X>=5ms, SFN mod Configuration Period = 0,

  - If X<5ms, SFN mod Configuration Period =1.

- Slot index of RACH occasion is function of whether odd or even SFN.

It should be noted that even though the configuration table design takes the semi-static periodicity into consideration, the collisions cannot be avoid entirely due to a variety of semi-static DL/UL periodicities. Rules should be defined when the collision occurs between RO and DL/flexible part.

The priority among RACH and DL-UL configurations

Note that the periodicity of semi-static DL-unknown-UL configuration could be smaller than the PRACH configuration periodicity. RACH resources may overlap with unknown part only in some periods. It is reasonable to allow RACH being transmitted in the unknown part to ensure sufficient RACH capacity. In such unknown part, gNB will configure it as UL part to all the UE by UE specific RRC signaling. For other unknown parts that do not overlap with the RACH resource in some periods, gNB still has the freedom to adjust it to DL or UL based on the dynamic load. If PRACH resources can only be allocated in the UL part, it will highly restricts the slot configuration at gNB side. The symbols for PRACH resources can only be used for the UL in any periods.

Furthermore, since the purpose for unknown part is to allow at least semi-static switching between DL and UL transmission. Allowing RACH transmission in the unknown part will not introduce any additional penalty at gNB and UE side.

In addition, the permission of PRACH resource transmission on flexible part could not only allow the gNB to allocate more RACH resources on some flexible parts to improve the overload in the uplink, but also allow gNB to configure a longer preamble format (probably with a smaller SCS than the UL BWP) to extend the cell coverage.

Proposal 4: The PRACH resources configured in RMSI are also allowed to be transmitted in the flexible part configured by UL-DL-configuration-common without collision with SS/PBCH block.

Frequency locations of RACH resources

In NR, the mapping from PRACH resource to physical resource block in frequency is absent in the current TS 38.211. In LTE, a PRACH resource is mapped to physical resource block in frequency according to either a frequency offset parameter prach-FrequencyOffset or two sides of the uplink channel bandwidth if there are multiple resources in the same PRACH time instance. For example in TDD preamble format 0~3, the hoping is [2]:

nPRB=prach­FrequencyOffset+6fRA2,    if fRA mod 2=0NRB-prach­FrequencyOffset-6-6fRA2,    if fRA mod 2=1

where fRA is the PRACH frequency index with the same time and NRB is the uplink bandwidth.

The NR should also consider frequency hopping so as to provide frequency diversity during Msg1 retransmission. In different PRACH configuration periods, the SS/PBCH block can be mapped to PRACH resources with different frequency so as to provide better diversity, e.g., as shown in the following equation:

nPRB=prach­FrequencyOffset+NRAfRA,    if SFNTRA mod 2=0NRBIAU BWP-prach­FrequencyOffset-NRA(fRA+1),    if SFNTRA mod 2=1

where  fRA is the PRACH frequency index with the same time instance, SFN is radio frame number of PRACH resource, TRA is PRACH configuration period (with unit frame), NRBIAU BWP is the bandwidth of initial active uplink bandwidth part, NRA is the bandwidth of the PRACH preamble in the subcarrier spacing of initial active uplink bandwidth part. Based on the above discussions, we have following proposal.

Proposal 5: NR defines the mapping from RACH resources to physical resource block.

Proposal 6: NR supports the mapping from RACH resources to physical resource block with frequency hopping across PRACH configuration periods.

A proposed text for this correction is provided in the Appendix D for Section 6.3.3.2 of TS 38.211[2].

Figure 4. PRACH frequency hopping across configuration period

Conclusion

Proposal 1: Remove the entries for format 1 of subframe number larger than 7 in Table 6.3.3.2-2 and confirm the working assumption as in appendix A.

Proposal 2: For FR1 and unpaired spectrum, NR adopts PRACH configuration of short sequence as shown in appendix B.

- Introducing starting symbol “7” for format A1, A2, A3, C0, C2, B1, A1/B1, A2/B2, A3/B3

- SFN mod Configuration Period is 1 when RACH Configuration Period > 1(10ms)

- Prioritize RO subframes locations in 4 and 9

Proposal 3: For FR2 and unpaired spectrum, NR adopts PRACH configuration as shown in appendix C.

- Introducing starting symbol “7” for format A1, A2, A3, C0, C2, B1, A1/B1, A2/B2, A3/B3

- SFN of RO depends on the RMSI CORESET group offset X (X=0ms for RMSI pattern 2/3):

  - If X>=5ms, SFN mod Configuration Period = 0,

  - If X<5ms, SFN mod Configuration Period =1.

- Slot index of RACH occasion is function of whether odd or even SFN.

Proposal 4: The PRACH resources configured in RMSI are also allowed to be transmitted in the flexible part configured by UL-DL-configuration-common without collision with SS/PBCH block.

Proposal 5: NR defines the mapping from RACH resources to physical resource block.

Proposal 6: NR supports the mapping from RACH resources to physical resource block with frequency hopping across PRACH configuration periods.

- Appendix A: Text proposal for preamble sequence generation

---------------------------------- Start of Text Proposal ---------------------------------

<Unchanged parts omitted>

Table 6.3.3.2-2: Random access configurations for FR1 and paired spectrum/supplementary uplink.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | Number of time-domain PRACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 19 | 0 | 4 | 1 | 9 | - | - | - |
| 20 | 0 | 8 | 1 | 1 | - | - | - |
| 21 | 0 | 8 | 1 | 4 | - | - | - |
| 22 | 0 | 8 | 1 | 7 | - | - | - |
| 23 | 0 | 8 | 1 | 9 | - | - | - |
| 24 | 0 | 16 | 1 | 1 | - | - | - |
| 25 | 0 | 16 | 1 | 4 | - | - | - |
| 26 | 0 | 16 | 1 | 7 | - | - | - |
| 27 | 0 | 16 | 1 | 9 | - | - | - |
| 28 | 1 | 2 | 1 | 1 | - | - | - |
| 29 | 1 | 2 | 1 | 4 | - | - | - |
| 30 | 1 | 2 | 1 | 7 | - | - | - |
| 31 | 1 | 1 | 0 | 1 | - | - | - |
| 32 | 1 | 1 | 0 | 4 | - | - | - |
| 33 | 1 | 1 | 0 | 7 | - | - | - |
| 34 | 1 | 1 | 0 | 1,6 | - | - | - |
| 35 | 1 | 1 | 0 | 2,7 | - | - | - |
| 36 | 1 | 1 | 0 | 3,8 | - | - | - |
| 37 | 1 | 1 | 0 | 1,4,7 | - | - | - |
| 38 | 1 | 1 | 0 | 2,5,8 | - | - | - |
| 39 | 1 | 1 | 0 | 3,6,9 | - | - | - |
| - 40 | - 1 | - 2 | - 1 | - 9 | - | - | - |
| 41 | 1 | 4 | 1 | 1 | - | - | - |
| 42 | 1 | 4 | 1 | 4 | - | - | - |
| 43 | 1 | 4 | 1 | 7 | - | - | - |
| - 44 | - 1 | - 4 | - 1 | - 9 | - | - | - |
| 45 | 1 | 8 | 1 | 1 | - | - | - |
| 46 | 1 | 8 | 1 | 4 | - | - | - |
| 47 | 1 | 8 | 1 | 7 | - | - | - |
| - 48 | - 1 | - 8 | - 1 | - 9 | - | - | - |
| 49 | 1 | 16 | 1 | 1 | - | - | - |
| 50 | 1 | 16 | 1 | 4 | - | - | - |
| 51 | 1 | 16 | 1 | 7 | - | - | - |
| - 52 | - 1 | - 16 | - 1 | - 9 | - | - | - |
| 53 | 2 | 4 | 0 | 1 | 0 | - | - |
| 54 | 2 | 2 | 0 | 1 | 0 | - | - |
| 55 | 2 | 2 | 0 | 5 | 0 | - | - |
| 56 | 2 | 1 | 0 | 1 | 0 | - | - |
| 57 | 2 | 1 | 0 | 5 | 0 | - | - |
| 58 | 2 | 8 | 1 | 1 | 0 | - | - |

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------

- Appendix B: Text proposal for RACH configuration for unpaired spectrum FR1

| PRACH Configuration Index | Preamble format | SFN mod x = y |  | subframe number (1 ms) | Starting symbol | Number of PRACH slots within a subframe | Number of ROs within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | x | y |  |  |  |  |
| 71 | A1 | 1 | 0 | 4 | 0 | 1 | 6 |
| 72 | A1 | 1 | 0 | 9 | 7 | 1 | 3 |
| 73 | A1 | 1 | 0 | 8 | 0 | 2 | 6 |
| 74 | A1 | 1 | 0 | 3 | 0 | 2 | 6 |
| 75 | A1 | 1 | 0 | 4,9 | 0 | 1 | 6 |
| 76 | A1 | 1 | 0 | 3,8 | 0 | 2 | 6 |
| 77 | A1 | 1 | 0 | 2,7 | 0 | 1 | 6 |
| 78 | A1 | 1 | 0 | 4,8,9 | 0 | 2 | 6 |
| 79 | A1 | 1 | 0 | 3,4,8,9 | 0 | 2 | 6 |
| 80 | A1 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 6 |
| 81 | A1 | 2 | 1 | 4,9 | 7 | 1 | 3 |
| 82 | A1 | 2 | 1 | 3,8 | 0 | 1 | 6 |
| 83 | A1 | 2 | 1 | 4 | 0 | 2 | 6 |
| 84 | A1 | 2 | 1 | 9 | 0 | 2 | 6 |
| 85 | A1 | 2 | 1 | 8 | 0 | 2 | 6 |
| 86 | A1 | 4 | 1 | 4 | 0 | 1 | 6 |
| 87 | A1 | 4 | 1 | 4,9 | 0 | 1 | 6 |
| 88 | A1 | 8 | 1 | 4 | 0 | 1 | 6 |
| 89 | A1 | 8 | 1 | 4,9 | 7 | 1 | 3 |
| 90 | A1 | 16 | 1 | 4 | 0 | 1 | 6 |
| 91 | A1 | 16 | 1 | 4,9 | 0 | 1 | 6 |
| 92 | B1 | 1 | 0 | 4 | 0 | 1 | 7 |
| 93 | B1 | 1 | 0 | 9 | 0 | 2 | 7 |
| 94 | B1 | 1 | 0 | 8 | 0 | 1 | 7 |
| 95 | B1 | 1 | 0 | 3 | 0 | 2 | 7 |
| 96 | B1 | 1 | 0 | 4,9 | 7 | 1 | 3 |
| 97 | B1 | 1 | 0 | 3,8 | 0 | 2 | 7 |
| 98 | B1 | 1 | 0 | 2,7 | 0 | 1 | 7 |
| 99 | B1 | 1 | 0 | 4,8,9 | 0 | 2 | 7 |
| 100 | B1 | 1 | 0 | 3,4,8,9 | 0 | 2 | 7 |
| 101 | B1 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 7 |
| 102 | B1 | 2 | 1 | 4 | 0 | 1 | 7 |
| 103 | B1 | 2 | 1 | 9 | 0 | 2 | 7 |
| 104 | B1 | 2 | 1 | 8 | 0 | 2 | 7 |
| 105 | B1 | 2 | 1 | 4,,9 | 7 | 1 | 3 |
| 106 | B1 | 2 | 1 | 3,8 | 0 | 2 | 7 |
| 107 | B1 | 4 | 1 | 4 | 0 | 1 | 7 |
| 108 | B1 | 4 | 1 | 4,9 | 0 | 1 | 7 |
| 109 | B1 | 8 | 1 | 4 | 0 | 1 | 7 |
| 110 | B1 | 8 | 1 | 4,9 | 7 | 1 | 3 |
| 111 | B1 | 16 | 1 | 4 | 0 | 1 | 7 |
| 112 | B1 | 16 | 1 | 4,9 | 0 | 1 | 7 |
| 113 | A2 | 1 | 0 | 4 | 0 | 1 | 3 |
| 114 | A2 | 1 | 0 | 9 | 0 | 2 | 3 |
| 115 | A2 | 1 | 0 | 8 | 0 | 1 | 3 |
| 116 | A2 | 1 | 0 | 4,9 | 0 | 2 | 3 |
| 117 | A2 | 1 | 0 | 4,9 | 7 | 1 | 1 |
| 118 | A2 | 1 | 0 | 3,8 | 0 | 2 | 3 |
| 119 | A2 | 1 | 0 | 2,7 | 0 | 1 | 3 |
| 120 | A2 | 1 | 0 | 4,8,9 | 0 | 2 | 3 |
| 121 | A2 | 1 | 0 | 3,4,8,9 | 0 | 2 | 3 |
| 122 | A2 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 3 |
| 123 | A2 | 2 | 1 | 4,8,9 | 0 | 1 | 3 |
| 124 | A2 | 2 | 1 | 4,9 | 7 | 1 | 1 |
| 125 | A2 | 2 | 1 | 4 | 0 | 2 | 3 |
| 126 | A2 | 2 | 1 | 9 | 0 | 2 | 3 |
| 127 | A2 | 2 | 1 | 8 | 0 | 2 | 3 |
| 128 | A2 | 4 | 1 | 4,9 | 7 | 1 | 1 |
| 129 | A2 | 4 | 1 | 4,8,9 | 0 | 1 | 3 |
| 130 | A2 | 8 | 1 | 4,9 | 7 | 1 | 1 |
| 131 | A2 | 8 | 1 | 4,8,9 | 0 | 1 | 3 |
| 132 | A2 | 16 | 1 | 4,9 | 0 | 1 | 3 |
| 133 | A2 | 16 | 1 | 4,8,9 | 0 | 1 | 3 |
| 134 | A3 | 1 | 0 | 4 | 0 | 2 | 2 |
| 135 | A3 | 1 | 0 | 9 | 0 | 2 | 2 |
| 136 | A3 | 1 | 0 | 8 | 0 | 2 | 2 |
| 137 | A3 | 1 | 0 | 4,9 | 7 | 1 | 1 |
| 138 | A3 | 1 | 0 | 4,9 | 0 | 1 | 2 |
| 139 | A3 | 1 | 0 | 3,8 | 0 | 2 | 2 |
| 140 | A3 | 1 | 0 | 2,7 | 0 | 1 | 2 |
| 141 | A3 | 1 | 0 | 4,8,9 | 0 | 2 | 2 |
| 142 | A3 | 1 | 0 | 3,4,8,9 | 0 | 2 | 2 |
| 143 | A3 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 2 |
| 144 | A3 | 2 | 1 | 4,8,9 | 0 | 1 | 2 |
| 145 | A3 | 2 | 1 | 4,9 | 7 | 1 | 1 |
| 146 | A3 | 2 | 1 | 4 | 0 | 2 | 2 |
| 147 | A3 | 2 | 1 | 9 | 0 | 2 | 2 |
| 148 | A3 | 2 | 1 | 8 | 0 | 2 | 2 |
| 149 | A3 | 4 | 1 | 9 | 0 | 2 | 2 |
| 150 | A3 | 4 | 1 | 4,9 | 7 | 1 | 1 |
| 151 | A3 | 8 | 1 | 9 | 0 | 2 | 2 |
| 152 | A3 | 8 | 1 | 4,9 | 7 | 1 | 1 |
| 153 | A3 | 16 | 1 | 9 | 0 | 2 | 2 |
| 154 | A3 | 16 | 1 | 4,9 | 7 | 1 | 1 |
| 155 | B4 | 1 | 0 | 4 | 0 | 2 | 1 |
| 156 | B4 | 1 | 0 | 9 | 0 | 2 | 1 |
| 157 | B4 | 1 | 0 | 8 | 0 | 2 | 1 |
| 158 | B4 | 1 | 0 | 4,9 | 0 | 2 | 1 |
| 159 | B4 | 1 | 0 | 4,9 | 0 | 1 | 1 |
| 160 | B4 | 1 | 0 | 3,8 | 0 | 2 | 1 |
| 161 | B4 | 1 | 0 | 2,7 | 0 | 1 | 1 |
| 162 | B4 | 1 | 0 | 4,8,9 | 0 | 2 | 1 |
| 163 | B4 | 1 | 0 | 3,4,8,9 | 0 | 2 | 1 |
| 164 | B4 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 1 |
| 165 | B4 | 2 | 1 | 4,9 | 0 | 1 | 1 |
| 166 | B4 | 2 | 1 | 4,9 | 0 | 2 | 1 |
| 167 | B4 | 2 | 1 | 4 | 0 | 2 | 1 |
| 168 | B4 | 2 | 1 | 9 | 0 | 2 | 1 |
| 169 | B4 | 2 | 1 | 8 | 0 | 2 | 1 |
| 170 | B4 | 4 | 1 | 9 | 0 | 2 | 1 |
| 171 | B4 | 4 | 1 | 4,9 | 0 | 1 | 1 |
| 172 | B4 | 8 | 1 | 9 | 0 | 2 | 1 |
| 173 | B4 | 8 | 1 | 4,9 | 0 | 1 | 1 |
| 174 | B4 | 16 | 1 | 9 | 0 | 2 | 1 |
| 175 | B4 | 16 | 1 | 4,9 | 0 | 1 | 1 |
| 176 | C0 | 1 | 0 | 4,9 | 7 | 1 | 3 |
| 177 | C0 | 1 | 0 | 4,9 | 0 | 2 | 7 |
| 178 | C0 | 1 | 0 | 1,4,7 | 0 | 1 | 7 |
| 179 | C0 | 1 | 0 | 9 | 0 | 2 | 7 |
| 180 | C0 | 1 | 0 | 4 | 0 | 2 | 7 |
| 181 | C0 | 1 | 0 | 3,8 | 0 | 2 | 7 |
| 182 | C0 | 1 | 0 | 2,7 | 0 | 1 | 7 |
| 183 | C0 | 1 | 0 | 4,8,9 | 0 | 2 | 7 |
| 184 | C0 | 1 | 0 | 3,4,8,9 | 0 | 2 | 7 |
| 185 | C0 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 7 |
| 186 | C0 | 2 | 1 | 4,8,9 | 0 | 1 | 7 |
| 187 | C0 | 2 | 1 | 4,9 | 7 | 1 | 3 |
| 188 | C0 | 2 | 1 | 4 | 0 | 2 | 7 |
| 189 | C0 | 2 | 1 | 9 | 0 | 2 | 7 |
| 190 | C0 | 2 | 1 | 8 | 0 | 2 | 7 |
| 191 | C0 | 4 | 1 | 4 | 0 | 1 | 7 |
| 192 | C0 | 4 | 1 | 4,9 | 7 | 1 | 3 |
| 193 | C0 | 8 | 1 | 4 | 0 | 1 | 7 |
| 194 | C0 | 8 | 1 | 4,9 | 0 | 1 | 7 |
| 195 | C2 | 1 | 0 | 4 | 0 | 2 | 2 |
| 196 | C2 | 1 | 0 | 9 | 0 | 2 | 2 |
| 197 | C2 | 1 | 0 | 8 | 0 | 2 | 2 |
| 198 | C2 | 1 | 0 | 4,9 | 0 | 2 | 2 |
| 199 | C2 | 1 | 0 | 4,9 | 7 | 1 | 1 |
| 200 | C2 | 1 | 0 | 3,8 | 0 | 2 | 2 |
| 201 | C2 | 1 | 0 | 2,7 | 0 | 1 | 2 |
| 202 | C2 | 1 | 0 | 4,8,9 | 0 | 2 | 2 |
| 203 | C2 | 1 | 0 | 3,4,8,9 | 0 | 2 | 2 |
| 204 | C2 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 2 |
| 205 | C2 | 2 | 1 | 4,8,9 | 0 | 1 | 2 |
| 206 | C2 | 2 | 1 | 4,9 | 7 | 1 | 1 |
| 207 | C2 | 2 | 1 | 4 | 7 | 1 | 1 |
| 208 | C2 | 2 | 1 | 9 | 7 | 1 | 1 |
| 209 | C2 | 2 | 1 | 8 | 0 | 2 | 2 |
| 210 | C2 | 4 | 1 | 4,9 | 7 | 1 | 1 |
| 211 | C2 | 4 | 1 | 4,8,9 | 0 | 1 | 2 |
| 212 | C2 | 8 | 1 | 4,9 | 0 | 1 | 2 |
| 213 | C2 | 8 | 1 | 4,8,9 | 0 | 1 | 2 |
| 214 | C2 | 16 | 1 | 4,9 | 7 | 1 | 1 |
| 215 | C2 | 16 | 1 | 4,8,9 | 0 | 1 | 2 |
| 216 | A1/B1 | 16 | 1 | 4 | 0 | 1 | 7 |
| 217 | A1/B1 | 16 | 1 | 9 | 7 | 1 | 3 |
| 218 | A1/B1 | 8 | 1 | 4 | 0 | 1 | 7 |
| 219 | A1/B1 | 8 | 1 | 9 | 0 | 2 | 7 |
| 220 | A1/B1 | 4 | 1 | 4 | 0 | 1 | 7 |
| 221 | A1/B1 | 4 | 1 | 9 | 0 | 2 | 7 |
| 222 | A1/B1 | 2 | 1 | 4 | 0 | 1 | 7 |
| 223 | A1/B1 | 2 | 1 | 9 | 0 | 2 | 7 |
| 224 | A1/B1 | 1 | 0 | 3,4,8,9 | 0 | 2 | 7 |
| 225 | A1/B1 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 7 |
| 226 | A1/B1 | 1 | 0 | 4,9 | 7 | 1 | 3 |
| 227 | A1/B1 | 2 | 1 | 4,9 | 0 | 1 | 7 |
| 228 | A2/B2 | 16 | 1 | 4,9 | 7 | 1 | 1 |
| 229 | A2/B2 | 16 | 1 | 9 | 0 | 2 | 3 |
| 230 | A2/B2 | 8 | 1 | 4,9 | 0 | 1 | 3 |
| 231 | A2/B2 | 8 | 1 | 9 | 0 | 2 | 3 |
| 232 | A2/B2 | 4 | 1 | 4,9 | 7 | 1 | 1 |
| 233 | A2/B2 | 4 | 1 | 9 | 0 | 2 | 3 |
| 234 | A2/B2 | 2 | 1 | 4,9 | 0 | 1 | 3 |
| 235 | A2/B2 | 2 | 1 | 9 | 0 | 2 | 3 |
| 236 | A2/B2 | 1 | 0 | 3,4,8,9 | 0 | 2 | 3 |
| 237 | A2/B2 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 3 |
| 238 | A2/B2 | 2 | 1 | 4,8,9 | 0 | 1 | 3 |
| 239 | A2/B2 | 1 | 0 | 4,9 | 7 | 1 | 1 |
| 240 | A2/B2 | 2 | 1 | 4 | 0 | 2 | 3 |
| 241 | A2/B2 | 1 | 0 | 9 | 0 | 2 | 3 |
| 242 | A3/B3 | 16 | 1 | 4,9 | 7 | 1 | 1 |
| 243 | A3/B3 | 16 | 1 | 9 | 0 | 2 | 2 |
| 244 | A3/B3 | 8 | 1 | 4,9 | 7 | 1 | 1 |
| 245 | A3/B3 | 8 | 1 | 9 | 0 | 2 | 2 |
| 246 | A3/B3 | 4 | 1 | 4,9 | 7 | 1 | 1 |
| 247 | A3/B3 | 4 | 1 | 9 | 0 | 2 | 2 |
| 248 | A3/B3 | 2 | 1 | 4,9 | 7 | 1 | 1 |
| 249 | A3/B3 | 2 | 1 | 9 | 0 | 2 | 2 |
| 250 | A3/B3 | 1 | 0 | 3,4,8,9 | 0 | 2 | 2 |
| 251 | A3/B3 | 1 | 0 | 1,3,4,7,9 | 0 | 2 | 2 |
| 252 | A3/B3 | 2 | 1 | 4,8,9 | 0 | 1 | 2 |
| 253 | A3/B3 | 1 | 0 | 4,9 | 7 | 1 | 1 |
| 254 | A3/B3 | 2 | 1 | 4 | 0 | 2 | 2 |
| 255 | A3/B3 | 1 | 0 | 9 | 0 | 2 | 2 |

- Appendix C: Text proposal for RACH configuration for unpaired spectrum FR2

| PRACH Config. Index | Preamble format | SFN mod x = y |  | slot number (60 kHz SCS) | Starting symbol | Number of PRACH slots within a 0.25 ms | Number of ROs within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | x | y |  |  |  |  |
| 0 | A1 | 1 | 0 | 2,3,4,12,13,14 | 0 | 1 | 6 |
| 1 | A1 | 1 | 0 | 27,28,29,37,38,39 | 7 | 1 | 3 |
| 2 | A1 | 1 | 0 | 2,3,4,12,13,14 | 0 | 2 | 6 |
| 3 | A1 | 1 | 0 | 27,28,29,37,38,39 | 7 | 1 | 3 |
| 4 | A1 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 1 | 6 |
| 5 | A1 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 7 | 1 | 3 |
| 6 | A1 | 1 | 0 | case A: 4,9,14,19 | 0 | 2 | 6 |
|  |  |  |  | case B: 24,29,34,39 |  |  |  |
| 7 | A1 | 1 | 0 | case A: 9,11,13,15,17,19 | 0 | 2 | 6 |
|  |  |  |  | case B: 29,31,33,35,37,39 |  |  |  |
| 8 | A1 | 1 | 0 | case A: 4,9,14,19,24,29 | 0 | 1 | 6 |
|  |  |  |  | case B: 14,19,24,29,34,39 |  |  |  |
| 9 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 |
| 10 | A1 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 6 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 11 | A1 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 6 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 12 | A1 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 6 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 13 | A1 | 1 | 0 | 4,9,11,13,15,17,19, 24,29,31,33,35,37,39 | 0 | 2 | 6 |
| 14 | A1 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 6 |
| 15 | A1 | 1 | 0 | 4,9,10,11,12,13,14,15,16, 17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 6 |
| 16 | A1 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 17 | A1 | 2 | 1 | 27,28,29,37,38,39 | 0 | 2 | 6 |
| 18 | A1 | 2 | X>=5: 0 | case A: 4,9,14,19 | 0 | 1 | 6 |
|  |  |  | X<5: 1 | case B: 24,29,34,39 |  |  |  |
| 19 | A1 | 2 | X>=5: 0 | case A: 9,11,13,15,17,19 | 0 | 1 | 6 |
|  |  |  | X<5: 1 | case B: 29,31,33,35,37,39 |  |  |  |
| 20 | A1 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 6 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 21 | A1 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 6 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 22 | A1 | 2 | X>=5: 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 6 |
|  |  |  | X<5: 1 |  |  |  |  |
| 23 | A1 | 2 | X>=5: 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 6 |
|  |  |  | X<5: 1 |  |  |  |  |
| 24 | A1 | 4 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 25 | A1 | 4 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 6 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 26 | A1 | 8 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 27 | A1 | 8 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 6 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 28 | A1 | 16 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 29 | A1 | 16 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 6 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 30 | A2 | 1 | 0 | 2,3,4,12,13,14 | 0 | 1 | 3 |
| 31 | A2 | 1 | 0 | 27,28,29,37,38,39 | 0 | 2 | 3 |
| 32 | A2 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 7 | 1 | 1 |
| 33 | A2 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 2 | 3 |
| 34 | A2 | 1 | 0 | case A: 4,9,14,19 | 0 | 1 | 3 |
|  |  |  |  | case B: 24,29,34,39 |  |  |  |
| 35 | A2 | 1 | 0 | case A: 16,17,18,19 | 0 | 2 | 3 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 36 | A2 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 3 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 37 | A2 | 1 | 0 | case A: 9,11,13,15,17,19 | 0 | 2 | 3 |
|  |  |  |  | case B: 29,31,33,35,37,39 |  |  |  |
| 38 | A2 | 1 | 0 | case A: 4,9,14,19,24,29 | 0 | 1 | 3 |
|  |  |  |  | case B: 14,19,24,29,34,39 |  |  |  |
| 39 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 |
| 40 | A2 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 3 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 41 | A2 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 42 | A2 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 43 | A2 | 1 | 0 | 4,9,11,13,15,17,19, 24,29,31,33,35,37,39 | 0 | 2 | 3 |
| 44 | A2 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 3 |
| 45 | A2 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 3 |
| 46 | A2 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 47 | A2 | 2 | X>=5: 0 | case A: 9,11,13,15 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case B: 33,35,37,39 |  |  |  |
| 48 | A2 | 2 | X>=5: 0 | case A: 4,9,14,19 | 0 | 1 | 3 |
|  |  |  | X<5: 1 | case B: 24,29,34,39 |  |  |  |
| 49 | A2 | 2 | X>=5: 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 |
|  |  |  | X<5: 1 |  |  |  |  |
| 50 | A2 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 51 | A2 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 52 | A2 | 2 | X>=5: 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 3 |
|  |  |  | X<5: 1 |  |  |  |  |
| 53 | A2 | 2 | X>=5: 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 3 |
|  |  |  | X<5: 1 |  |  |  |  |
| 54 | A2 | 4 | 1 | 2,3,4,12,13,14 | 0 | 1 | 3 |
| 55 | A2 | 4 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 56 | A2 | 8 | X>=5: 0 | case A: 9 ,11, 13,15,17,19 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case A: 29 ,31, 33,35,37,39 |  |  |  |
| 57 | A2 | 8 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 58 | A2 | 16 | 1 | 2,3,4,12,13,14 | 0 | 1 | 3 |
| 59 | A2 | 16 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 60 | A3 | 1 | 0 | 2,3,4,12,13,14 | 0 | 1 | 2 |
| 61 | A3 | 1 | 0 | 27,28,29,37,38,39 | 0 | 2 | 2 |
| 62 | A3 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 7 | 1 | 1 |
| 63 | A3 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 2 | 2 |
| 64 | A3 | 1 | 0 | case A: 4,9,14,19 | 0 | 1 | 2 |
|  |  |  |  | case B: 24,29,34,39 |  |  |  |
| 65 | A3 | 1 | 0 | case A: 16,17,18,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 66 | A3 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 2 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 67 | A3 | 1 | 0 | case A: 9,11,13,15,17,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 29,31,33,35,37,39 |  |  |  |
| 68 | A3 | 1 | 0 | case A: 4,9,14,19,24,29 | 0 | 1 | 2 |
|  |  |  |  | case B: 14,19,24,29,34,39 |  |  |  |
| 69 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
| 70 | A3 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 71 | A3 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 72 | A3 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 73 | A3 | 1 | 0 | 4,9,11,13,15,17,19, 24,29,31,33,35,37,39 | 0 | 2 | 2 |
| 74 | A3 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 2 |
| 75 | A3 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 2 |
| 76 | A3 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 77 | A3 | 2 | X>=5: 0 | case A: 9,11,13,15 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 33,35,37,39 |  |  |  |
| 78 | A3 | 2 | X>=5: 0 | case A: 4,9,14,19 | 0 | 1 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,34,39 |  |  |  |
| 79 | A3 | 2 | X>=5: 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
|  |  |  | X<5: 1 |  |  |  |  |
| 80 | A3 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 81 | A3 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 82 | A3 | 2 | X>=5: 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 2 |
|  |  |  | X<5: 1 |  |  |  |  |
| 83 | A3 | 2 | X>=5: 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 2 |
|  |  |  | X<5: 1 |  |  |  |  |
| 84 | A3 | 4 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 85 | A3 | 4 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 86 | A3 | 8 | X>=5: 0 | case A: 9 ,11, 13,15,17,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 29 ,31, 33,35,37,39 |  |  |  |
| 87 | A3 | 8 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 88 | A3 | 16 | 1 | 2,3,4,12,13,14 | 0 | 1 | 2 |
| 89 | A3 | 16 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 90 | B1 | 1 | 0 | 2,3,4,12,13,14 | 0 | 1 | 7 |
| 91 | B1 | 1 | 0 | 27,28,29,37,38,39 | 7 | 1 | 3 |
| 92 | B1 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 1 | 7 |
| 93 | B1 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 2 | 7 |
| 94 | B1 | 1 | 0 | case A: 4,9,14,19 | 0 | 1 | 7 |
|  |  |  |  | case B: 24,29,34,39 |  |  |  |
| 95 | B1 | 1 | 0 | case A: 16,17,18,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 96 | B1 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 7 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 97 | B1 | 1 | 0 | case A: 9,11,13,15,17,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 29,31,33,35,37,39 |  |  |  |
| 98 | B1 | 1 | 0 | case A: 4,9,14,19,24,29 | 0 | 1 | 7 |
|  |  |  |  | case B: 14,19,24,29,34,39 |  |  |  |
| 99 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 100 | B1 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 101 | B1 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 102 | B1 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 103 | B1 | 1 | 0 | 4,9,11,13,15,17,19, 24,29,31,33,35,37,39 | 0 | 2 | 7 |
| 104 | B1 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 7 |
| 105 | B1 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 7 |
| 106 | B1 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 107 | B1 | 2 | 1 | 27,28,29,37,38,39 | 0 | 2 | 7 |
| 108 | B1 | 2 | X>=5: 0 | case A: 4,9,14,19 | 0 | 1 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,34,39 |  |  |  |
| 109 | B1 | 2 | X>=5: 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
|  |  |  | X<5: 1 |  |  |  |  |
| 110 | B1 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 111 | B1 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 112 | B1 | 2 | X>=5: 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 7 |
|  |  |  | X<5: 1 |  |  |  |  |
| 113 | B1 | 2 | X>=5: 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 7 |
|  |  |  | X<5: 1 |  |  |  |  |
| 114 | B1 | 4 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 115 | B1 | 4 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 116 | B1 | 8 | X>=5: 0 | case A: 9 ,11, 13,15,17,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 29 ,31, 33,35,37,39 |  |  |  |
| 117 | B1 | 8 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 118 | B1 | 16 | 1 | 2,3,4,12,13,14 | 0 | 1 | 7 |
| 119 | B1 | 16 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 120 | B4 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,47 | 0 | 1 | 1 |
| 121 | B4 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,48 | 0 | 2 | 1 |
| 122 | B4 | 1 | 0 | case A: 4,9 | 0 | 1 | 1 |
|  |  |  |  | case B: 29,39 |  |  |  |
| 123 | B4 | 1 | 0 | case A: 4,9,19 | 0 | 1 | 1 |
|  |  |  |  | case B: 29,34,39 |  |  |  |
| 124 | B4 | 1 | 0 | case A: 4,9,14,19 | 0 | 1 | 1 |
|  |  |  |  | case B: 24,29,34,39 |  |  |  |
| 125 | B4 | 1 | 0 | case A: 16,17,18,19 | 0 | 2 | 1 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 126 | B4 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 1 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 127 | B4 | 1 | 0 | case A: 9,11,13,15,17,19 | 0 | 2 | 1 |
|  |  |  |  | case B: 29,31,33,35,37,39 |  |  |  |
| 128 | B4 | 1 | 0 | case A: 4,9,14,19,24,29 | 0 | 1 | 1 |
|  |  |  |  | case B: 14,19,24,29,34,39 |  |  |  |
| 129 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 |
| 130 | B4 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 1 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 131 | B4 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 1 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 132 | B4 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 1 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 133 | B4 | 1 | 0 | 4,9,11,13,15,17,19, 24,29,31,33,35,37,39 | 0 | 2 | 1 |
| 134 | B4 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 1 |
| 135 | B4 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 1 |
| 136 | B4 | 2 | 1 | 2,3,4,12,13,14,27,28,29,37,38,45 | 0 | 1 | 1 |
| 137 | B4 | 2 | 1 | 2,3,4,12,13,14,27,28,29,37,38,46 | 0 | 2 | 1 |
| 138 | B4 | 2 | X>=5: 0 | case A: 4,9,14,19 | 0 | 1 | 1 |
|  |  |  | X<5: 1 | case B: 24,29,34,39 |  |  |  |
| 139 | B4 | 2 | X>=5: 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 |
|  |  |  | X<5: 1 |  |  |  |  |
| 140 | B4 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 1 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 141 | B4 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 1 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 142 | B4 | 2 | X>=5: 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 1 |
|  |  |  | X<5: 1 |  |  |  |  |
| 143 | B4 | 2 | X>=5: 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 1 |
|  |  |  | X<5: 1 |  |  |  |  |
| 144 | B4 | 4 | X>=5: 0 | case A: 9 ,11, 13,15,17,19 | 0 | 2 | 1 |
|  |  |  | X<5: 1 | Case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 145 | B4 | 4 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 1 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 146 | B4 | 8 | 1 | 2,3,4,12,13,14,27,28,29,37,38,41 | 0 | 1 | 1 |
| 147 | B4 | 8 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 1 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 148 | B4 | 16 | 1 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 1 | 1 |
| 149 | B4 | 16 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 1 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 150 | C0 | 1 | 0 | 2,3,4,12,13,14 | 0 | 1 | 7 |
| 151 | C0 | 1 | 0 | 27,28,29,37,38,39 | 0 | 2 | 7 |
| 152 | C0 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 7 | 1 | 3 |
| 153 | C0 | 1 | 0 | 7,8,9,17,18,19,27,28,29,37,38,39 | 0 | 2 | 7 |
| 154 | C0 | 1 | 0 | case A: 4,9,14,19 | 0 | 1 | 7 |
|  |  |  |  | case B: 24,29,34,39 |  |  |  |
| 155 | C0 | 1 | 0 | case A: 16,17,18,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 156 | C0 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 7 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 157 | C0 | 1 | 0 | case A: 9,11,13,15,17,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 29,31,33,35,37,39 |  |  |  |
| 158 | C0 | 1 | 0 | case A: 4,9,14,19,24,29 | 0 | 1 | 7 |
|  |  |  |  | case B: 14,19,24,29,34,39 |  |  |  |
| 159 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 160 | C0 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 161 | C0 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 162 | C0 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 163 | C0 | 1 | 0 | 4,9,11,13,15,17,19, 24,29,31,33,35,37,39 | 0 | 2 | 7 |
| 164 | C0 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 7 |
| 165 | C0 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 7 |
| 166 | C0 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 167 | C0 | 2 | 1 | 27,28,29,37,38,39 | 0 | 2 | 7 |
| 168 | C0 | 2 | X>=5: 0 | case A: 4,9,14,19 | 0 | 1 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,34,39 |  |  |  |
| 169 | C0 | 2 | X>=5: 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
|  |  |  | X<5: 1 |  |  |  |  |
| 170 | C0 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 171 | C0 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 172 | C0 | 2 | X>=5: 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 7 |
|  |  |  | X<5: 1 |  |  |  |  |
| 173 | C0 | 2 | X>=5: 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 7 |
|  |  |  | X<5: 1 |  |  |  |  |
| 174 | C0 | 4 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 175 | C0 | 4 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 176 | C0 | 8 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 177 | C2 | 1 | 0 | 2,3,4,12,13,14 | 0 | 1 | 2 |
| 178 | C2 | 1 | 0 | 27,28,29,37,38,39 | 7 | 1 | 1 |
| 179 | C2 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 1 | 2 |
| 180 | C2 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 7 | 1 | 1 |
| 181 | C2 | 1 | 0 | case A: 4,9,14,19 | 0 | 1 | 2 |
|  |  |  |  | case B: 24,29,34,39 |  |  |  |
| 182 | C2 | 1 | 0 | case A: 16,17,18,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 183 | C2 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 2 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 184 | C2 | 1 | 0 | case A: 9,11,13,15,17,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 29,31,33,35,37,39 |  |  |  |
| 185 | C2 | 1 | 0 | case A: 4,9,14,19,24,29 | 0 | 1 | 2 |
|  |  |  |  | case B: 14,19,24,29,34,39 |  |  |  |
| 186 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
| 187 | C2 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 188 | C2 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 189 | C2 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 190 | C2 | 1 | 0 | 4,9,11,13,15,17,19, 24,29,31,33,35,37,39 | 0 | 2 | 2 |
| 191 | C2 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 2 |
| 192 | C2 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 2 |
| 193 | C2 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 194 | C2 | 2 | 1 | 27,28,29,37,38,39 | 7 | 1 | 1 |
| 195 | C2 | 2 | X>=5: 0 | case A: 4,9,14,19 | 0 | 1 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,34,39 |  |  |  |
| 196 | C2 | 2 | X>=5: 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
|  |  |  | X<5: 1 |  |  |  |  |
| 197 | C2 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 198 | C2 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 199 | C2 | 2 | X>=5: 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 2 |
|  |  |  | X<5: 1 |  |  |  |  |
| 200 | C2 | 2 | X>=5: 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 2 |
|  |  |  | X<5: 1 |  |  |  |  |
| 201 | C2 | 4 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 202 | C2 | 4 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 203 | C2 | 8 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 204 | C2 | 8 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 205 | C2 | 16 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 206 | C2 | 16 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 207 | A1/B1 | 1 | 0 | 2,3,4,12,13,14 | 0 | 1 | 7 |
| 208 | A1/B1 | 1 | 0 | 27,28,29,37,38,39 | 0 | 2 | 7 |
| 209 | A1/B1 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 1 | 7 |
| 210 | A1/B1 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 2 | 7 |
| 211 | A1/B1 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 7 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 212 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 213 | A1/B1 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 214 | A1/B1 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 215 | A1/B1 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 216 | A1/B1 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 7 |
| 217 | A1/B1 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 7 |
| 218 | A1/B1 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 3 |
| 219 | A1/B1 | 2 | X>=5: 0 | case A: 4,9,14,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,34,39 |  |  |  |
| 220 | A1/B1 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 221 | A1/B1 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 7 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 222 | A2/B2 | 1 | 0 | 2,3,4,12,13,14 | 0 | 1 | 3 |
| 223 | A2/B2 | 1 | 0 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 224 | A2/B2 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 2 | 3 |
| 225 | A2/B2 | 1 | 0 | case A: 4,9,14,19 | 0 | 1 | 3 |
|  |  |  |  | case B: 24,29,34,39 |  |  |  |
| 226 | A2/B2 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 3 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 227 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 |
| 228 | A2/B2 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 3 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 229 | A2/B2 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 230 | A2/B2 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 231 | A2/B2 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 3 |
| 232 | A2/B2 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 3 |
| 233 | A2/B2 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 234 | A2/B2 | 2 | X>=5: 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 235 | A2/B2 | 2 | X>=5: 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 3 |
|  |  |  | X<5: 1 | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 236 | A2/B2 | 4 | 1 | 27,28,29,37,38,39 | 0 | 2 | 3 |
| 237 | A3/B3 | 1 | 0 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 238 | A3/B3 | 1 | 0 | 27,28,29,37,38,39 | 7 | 1 | 1 |
| 239 | A3/B3 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 1 | 2 |
| 240 | A3/B3 | 1 | 0 | 2,3,4,12,13,14,27,28,29,37,38,39 | 0 | 2 | 2 |
| 241 | A3/B3 | 1 | 0 | case A: 9,11,13,15 | 0 | 2 | 2 |
|  |  |  |  | case B: 33,35,37,39 |  |  |  |
| 242 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
| 243 | A3/B3 | 1 | 0 | case A: 4,9,11,13,14,15,17,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 24,29,31,33,34,35,37,39 |  |  |  |
| 244 | A3/B3 | 1 | 0 | case A: 12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 32,33,34,35,36,37,38,39 |  |  |  |
| 245 | A3/B3 | 1 | 0 | case A: 4,9,10,11,12,13,14,15,16,17,18,19 | 0 | 2 | 2 |
|  |  |  |  | case B: 24,29,30,31,32,33,34,35,36,37,38,39 |  |  |  |
| 246 | A3/B3 | 1 | 0 | 4,9,11,13,14,15,17,19, 24,29,31,33,34,35,37,39 | 0 | 2 | 2 |
| 247 | A3/B3 | 1 | 0 | 4,9,10,11,12,13,14,15,16,17,18,19,24,29,30,31,32,33,34,35,36,37,38,39 | 0 | 2 | 2 |
| 248 | A3/B3 | 2 | 1 | 2,3,4,12,13,14 | 7 | 1 | 1 |
| 249 | A3/B3 | 2 | 1 | 27,28,29,37,38,39 | 0 | 2 | 2 |
| 250 | A3/B3 | 8 | 1 | 27,28,29,37,38,39 | 0 | 2 | 2 |
| 251 | A3/B3 | 4 | 1 | 2,3,4,12,13,14 | 0 | 1 | 2 |
| 252 |  |  |  |  |  |  |  |
| 253 |  |  |  |  |  |  |  |
| 254 |  |  |  |  |  |  |  |
| 255 |  |  |  |  |  |  |  |

Notes:

Case A: if SFN mod 2=0;

Case B: if SFN mod 2=1.

- Appendix D: Text proposal for PRACH frequency multiplexing

---------------------------------- Start of Text Proposal ---------------------------------

<Unchanged parts omitted>

6.3.3.2	Mapping to physical resources

The preamble sequence shall be mapped to physical resources according to

where physical resources overlapping resources declared as ‘reserved’ according to clause 4.4.3 shall be counted in the mapping process but not used for transmission,  is an amplitude scaling factor in order to conform to the transmit power specified in [5, TS38.213], and  is the antenna port. Baseband signal generation shall be done according to clause 5.3 using the parameters in Table 6.3.3.1-1 or Table 6.3.3.1-2 with  given by Table 6.3.3.2-1.

Random access preambles can only be transmitted in the time resources given by the higher-layer parameter PRACHConfigurationIndex according to Table 6.3.3.2-2 depends on FR1 or FR2 and the spectrum type as defined in [TS38.101]. For the purpose of slot numbering in the tables, the following subcarrier spacing shall be assumed:

-	 for PRACH preamble formats 0 – 3

-	for PRACH preamble formats A1, A2, A3, B1, B2, B3, B4, C0, C2 where  is the configured PRACH subcarrier spacing.

The frequency multiplexing shall be done according to

nPRB=prach­FrequencyOffset+NRAfRA,    if SFNTRA mod 2=0NRBIAU BWP-prach­FrequencyOffset-NRA(fRA+1),    if SFNTRA mod 2=1

Where prach­FrequencyOffset is the relative offset of the first PRACH occasion in frequency domain with respective to PRB 0 of initial active UL BWP, fRA is the PRACH frequency index in the time instance, SFN is radio frame number of PRACH occasion, TRA is PRACH configuration period (with unit frame), NRBIAU BWP is the bandwidth of initial active uplink bandwidth part, NRA is the bandwidth of the PRACH preamble as given in Table 6.3.3.2-1.

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------