3GPP TSG RAN WG1 Meeting #92	R1-1801467

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.4.3

Source:	Huawei, HiSilicon

Title:	Channel coding for URLLC

Document for:	Discussion and Decision

# Introduction

This contribution is a revision of R1-1800104.

Reliability requirement, latency requirement, and diverse application scenarios have been proposed and discussed for URLLC (Ultra-Reliable and Low Latency Communications). As a result, the URLLC system design and requirements are different from eMBB ones. For channel coding, a typical URLLC application scenario requires a scheme with a more reliable transmission and/or a shorter latency [1][2]. In this contribution we discuss the channel coding to meet the requirements of URLLC scenario.

# Discussion

## Finalizing NR Channel Coding

A large amount of  data (simulations, comparisons, etc.) have been accumulated during the work of channel coding design for NR eMBB to help unveil the properties and advantages of both polar code and LDPC code. Below is a summary of the channel coding schemes for eMBB:

- eMBB Control channels adopt Polar codes mostly for its higher reliability and lower decoding latency for small packets

  - DL design: distributed CRC, information block length up to 140 bits (the maximum DCI length depends on the DL control channel design)

  - UL Design: PC and CA, information block length from 12 bits to ~500 bits or longer (the maximum UCI length depends on the UL control channel design)

  - Target BLER at 1e-3~1e-4

- eMBB Data channels adopt LDPC codes mostly for its peak-throughput enhancement and lower latency for large packets

  - Two base-graphs (BG1 and BG2)

  - Selection of a BG is based on code rate and block length

  - Target BLER at 1e-1~1e-2

URLLC scenario is supported in Rel.-15 and channel coding for URLLC is one fundamental aspect that needs to be specified. As implementation of NR eMBB channel coding schemes is ongoing, the following factors should be considered in order to complete URLLC standardization and implementation:

- At the network side, a Rel.-15-compatible implementation should support URLLC traffic.

- At the UE side, a Rel.-15-compatible implementation should support URLLC traffic, even though UE categories are currently under discussion.

- At both UE and network sides, it is preferred to maximize reuse of implementations to support both eMBB and URLLC, while adopting solutions capable of satisfying the requirements of the various scenarios.

Observation-1: It is necessary to complete channel coding specifications for URLLC to support URLLC in Rel.-15, design efficient Rel.-15 products and optimize NR deployments.

## Diverse URLLC Requirements

A typical URLLC scenario has the following characteristics [1]:

- Typical length of an information packet is around 32 bytes

- User plane latency is no longer than 0.5ms for UL and 0.5ms for DL

- Error-floor-free BLER is as low as 10-5 with or without HARQ support during 1-ms period

In [2], URLLC scenarios are further categorized into three different use case families, e.g. “higher reliability and lower latency”, “higher reliability, higher availability and lower latency”, and “very low latency”, as illustrated in Table 1.

Table 1	Summary of URLLC use cases from [2]

| Use case family | Use case | Payload size | Latency | Data rate | Reliability | Jitter | Deployment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Higher reliability and lower latency | Industrial Factory Automation | Typical small (<50 bytes). | The cycle time ranges between 2 and 20 ms setting stringent latency constraints on telegram forwarding (from <1 ms to10 ms) |  | BLER<10-9 | 10-100us | a controller interacts with large number of sensors and actuators (e.g., up to 300) integrated in a manufacturing unit (e.g., 10 m x 10 m x 3 m). The resulting sensor/actuator density is often very high (e.g., up to 1/m3). |
|  | Industrial Process Automation | typically <100 bytes | 100ms~1s |  | BLER<10-5 |  | a large number of sensors (~10,000) that are distributed over the plant forward measurement data to process controllers on a periodic or event-driven base |
|  | Ultra-reliable communications (Substation protection and control) |  | 1 ms end to end | 80 samples/cycle for protection application, 256 samples/cycle (12.5Mbps) for quality analysis and recording | BLER<10-5 |  | Merging Units (MUs) perform periodic measurements of power system components, and send sampled measurement data to a Protection Relay. When the Protection Relay detects a fault, it sends signals to trip circuit breakers |
|  | Ultra-reliable communications (Smart grid system) | 200~1521 bytes | 8 ms between any two communicating points |  | BLER<1-99.999% |  | dense urban hundreds of UEs per km2 urban around 15 UEs per km2 populated rural max 1 UE per km2 |
|  | Speech, audio and video in VR and AR |  | 10 ms, including encoding and decoding are derived from human perception |  |  |  |  |
|  | Unmanned Aerial Vehicles (UAVs) |  | 5 ms one-way latency |  |  |  |  |
| Higher reliability, higher availability and lower latency | Industrial control |  | ~1 ms when data rate is relatively low 75 ms for data rate 20Mbps | low when latency is very low, 10s of Mbps per user for video application in a dense environment |  |  |  |
|  | High reliability, high availability, high mobility |  |  |  | BLER<1-99.999% |  | pre-hospital: medical equipment and the machines in the ambulance. route to the hospital is known or can be predicted |
| Very low latency | Tactile internet |  | 1ms one-way delay |  | very high |  |  |

As examples of different requirements, the reliability requirement can be below BLER of 10-9 in some URLLC scenarios but with the correspondent latency longer than 1ms (up to 20ms). The payload sizes vary from 20 bits to 1000 bits or longer.

Observation-2: URLLC scenarios require higher reliability and lower latency than eMBB. These requirements should be considered when deciding the channel coding scheme for URLLC.

## Channel Coding for URLLC

Given the different requirements for URLLC respect to eMBB scenario, the selection of the channel coding scheme for URLLC data channel cannot be based solely on what selected for eMBB and requires a thorough analysis.

### LDPC Code for URLLC

The boundary between the two eMBB LDPC base graphs (BG1 and BG2) is illustrated in Figure 1.

Figure 1	Agreed allocation between BG2 and BG1

BG1 is designed for high-throughput transmissions of large payload sizes and high code rates; whereas BG2 is for modest throughput and code rates in eMBB data channels.

BG1 was not optimized for small payload sizes and low code rates. A practical implementation of a BG1 decoder optimizes its architecture for efficient high-throughput decoding. These optimizations may become inefficient and power-thirsty for small payload sizes and low code rates of URLLC.

Observation-3: The eMBB LDPC BG1 does not suit the URLLC scenarios.

Although typically covering modest payload sizes and code rates, eMBB LDPC BG2 has some known reliability issues for URLLC scenarios.

Error-floor: an error floor above BLER of 10-5 for LDPC BG2 codes has been observed in [3] and [4]. By setting the code length to 1920 and using 16QAM as typical 4RB allocation unit, BLER performance as shown in Figure 2 is obtained. It can be observed that an error floor appears in the high reliable zone (BLER target around 1e-5 or lower) at several payload sizes.

Figure 2 Error floor of LDPC

Figure 3 Required SNR for LDPC at BLER=1e-2, 1e-4 and 1e-6

Another view of the error floor of LDPC BG2 is shown in Figure 3, where the required SNR points for BLER=1e-2, 1e-4 and 1e-6 are given with code rate 1/3. The gap between required SNR of BLER=1e-2 and 1e-4 is smooth among difference payload size, while it varies dramatically between required SNR of BLER=1e-4 and 1e-6. The reason for this fluctuation is that error floor appears between 1e-4 and 1e-6, which makes the slope of BLER curve more flat, and a higher required SNR than expected is needed. Further results can be found in the Appendix.

Observation-4:  The eMBB LDPC BG2 shows error floor around BLER of 10-5.

This observation reduces the span of applicability of BG2 for URLLC, because some scenarios request a reliability below 10-5, even 10-9.

At some payload sizes, performance suddenly degrades (“spikes” in the SNR curves) as can be observed in Figure 2. The occurrence of these spikes increases when the target BLER decreases, especially for BLER smaller than 1e-4 which is of interest for the URLLC scenario. During the evaluation of LDPC codes for eMBB, it was observed that the curves of required SNR vs. information block length become substantially less smooth as the target BLER decreases for example from 1e-4 to 1e-5. This phenomenon is more severe for small block lengths. Figure 4 is an example for K=70 plus 24-CRC bits where a spike occurs: the gap between polar and LDPC performance widens as the BLER decreases.

Figure 4 Example of performance when a spike occurs in LDPC BG2 performance.

Observation-5: There exist payload sizes for which performance of LDPC BG2 degrades substantially.

These reliability issues pose no problem for eMBB scenarios which requires BLER above 10-4 (mostly 1e-2) but are a concern for URLLC. To solve such issues, the base graph and its PCMs should be re-designed, which would require a substantial effort and amount of time, and it might end up with a more complicated decoder. Also as experienced during the work of designing LDPC codes for eMBB, it is challenging to design an LDPC code that it is efficient and has high reliability while covering payload sizes from few bits to thousands bits.

### Polar Code for URLLC

Polar code for eMBB control channel is designed to cover typical payload sizes up to ~200 bits in downlink and ~500 bits or longer (depending on the UL control channel design) in uplink. Polar code exhibits superior performance and complexity/latency advantages in a typical NR-PDCCH payload sizes and code rates over any known codes.

Reliability:

Many simulation results have demonstrated the performance advantage of polar code vs LDPC codes (BG2). For example, with a TDL-C300 fading channel and typical K =200 bit, we compare the two codes in Figure 5.

Figure 5 Polar code (List8) vs LDPC BG2 (LOMS-20) in Fading channel

Besides to its superior performance for small payload sizes, it was proved and verified that polar code has no error floor [5]. No plateau on polar code curves was observed in any evaluation. Figures 6.a and 6.b show polar code results with fine granularity when the BLER is 1e-5 or lower.

Figure 6.a Fine granularity performance for Polar code (List8), e.g. M=960, K=[40:8:864], QPSK

Figure 6.b Fine granularity performance for Polar code (List8), e.g. M=1920, K=[384:8:1728], 16QAM

Observation-6: Polar code has no error floor.

Latency:

A polar code constructor tends to place the information bits on the highest indices bit positions (the ending part), because the higher indices bit positions have higher reliability in general. This results into a large number of frozen bits placed before any information bit. It is known that a SC (successive-cancellation)-based polar decoder is able to skip the f/g function computations on all these precedent frozen bits without performance loss, thereby shortening the latency and reducing decoding power consumption. These advantages are improved along with a lower coding rate.

In comparison with polar SC-based decoder, a LOMS (offset-min-sum) decoding architecture of LDPC code results into a uniform decoding latency regardless code rates. Moreover, since it is hard to switch off a part of the interconnection of a BP (believe-propagation) decoder for a small block and low cod rate, the power consumption for a low coding rate may not be significantly reduced as polar SC-based decoding does.

The latency ratio between polar (SCL-8 decoding) and LDPC (LOMS with 20 iterations) is shown in Table 2. If the maximum mother code length of polar is limited to be 2048, the superiority of polar over LDPC is more obvious as shown in Table 3.

Table 2	Latency comparison of polar (Nmax=8192) over LDPC

Table 3	Latency comparison of polar (Nmax=2048) over LDPC

Observation-7: Polar code has reliability and latency advantages over LDPC code for small blocks.

### Hybrid Channel Code Scheme for URLLC

A hybrid channel coding scheme, as illustrated in Figure 7, might be worth considering for the URLLC data channel: polar code is used for small blocks and low code rates, and LDPC BG2 is used for longer blocks and higher code rates. Although the evaluation of the BLER performance shows no clear advantage of LDPC codes over Polar codes for longer blocks, the initial analysis of the latency shows an advantage of LDPC codes when the information block length is larger than about 800 bits for certain code rates. Further analysis of the latency is required since latency is largely dependent on implementation.

Figure 7	Hybrid Channel Code Scheme for URLLC

One key consideration is that while such hybrid solution would offer the best performance for URLLC and satisfy the requirements, the coding schemes adopted for eMBB can be reused, which would be a major advantage for the implementation. It is also worth noting that the selection of the channel coding scheme for URLLC scenario should take into consideration the target block length of URLLC, similarly as done for eMBB, and should be optimized accordingly.

We evaluated the two channel codes as shown in Figure 8. Here, GA is used as the baseline to show the performance, since the maximum mother code length of polar code adopted in eMBB control channel is limited to be 1024. A simple extension can be made to the construction sequence of polar code in eMBB control channel to serve URLLC better.

Figure 8 Performance comparison between LDPC and Polar codes

Observation-8: When the payload is less than 400 ~ 500 bits, polar code has better performance. For larger payloads up to 1000 bits, both schemes have similar performance although further investigation is needed when the target BLER is lower than 1e-5.

Evaluation of Polar and LDPC BG2 codes with higher order modulation is still ongoing. Some results with 16QAM and 64QAM are shown in the Appendix in [7]. Such results clearly indicate that LDPC BG2 performance substantially degrades as the modulation order increases and the target BLER decreases. With 16QAM or 64QAM, the slope change of the BLER curves starts to appear at target BLER below 1e-3.

# Conclusion

In this contribution, we discuss the channel coding scheme candidates for URLLC scenario. Polar and LDPC codes are compared in terms of BLER performance, error floor and decoding latency. Polar codes show better performance than LDPC codes.

Observation-1: It is necessary to complete channel coding specifications for URLLC to support URLLC in Rel.-15, design efficient Rel.-15 products and optimize NR deployments.

Observation-2: URLLC scenarios require higher reliability and lower latency than eMBB. These requirements should be considered when deciding the channel coding scheme for URLLC.

Observation-3: The eMBB LDPC BG1 does not suit the URLLC scenarios.

Observation-4: The eMBB LDPC BG2 shows error floor around BLER of 10-5.

Observation-5: There exist payload sizes for which performance of LDPC BG2 degrades substantially.

Observation-6: Polar code has no error floor.

Observation-7: Polar code has reliability and latency advantages over LDPC code for small blocks.

Observation-8: When the payload is less than 400 ~ 500 bits, polar code has better performance. For larger payloads up to 1000 bits, both schemes have similar performance although further investigation is needed when the target BLER is lower than 1e-5.

Since the requirements for URLLC scenario are different from the requirements for eMBB scenario, further discussion is needed on the channel coding design for the URLLC scenario.

Proposal 1: Channel coding for URLLC shall be further studied.

# References

[1] TR 38.913, Study on Scenarios and Requirements for: Study on New Radio Access Technology.

[2] TR 22.862, Feasibility Study on New Services and Markets Technology Enablers - Critical Communications.

[3] T. Richardson, Error floors of LDPC codes, in Proceedings of the Annual Allerton Conference on Communication Control and Computing, Vol. 41. No. 3, 2003.

[4] Lin, Shu, and D. J. Costello. Error control coding: Fundamentals and Applications, Prentice-Hall. 1983.

[5] M. Mondelli, S. H. Hassani, R. Urbanke, Unified Scaling of Polar Codes: Error Exponent, Scaling

Exponent, Moderate Deviations, and Error Floors, IEEE Trans. on Information Theory, 2016.

[6] R1-1719412, Link adaption and CSI reporting for URLLC transmission, Huawei, HiSilicon.

[7] R1-1801354, MCS/CQI design for URLLC transmission, Huawei, HiSilicon, RAN1#92.

# Appendix A - Error floor for LDPC

Figure 9 Error floor of eMBB LDPC BG2 with AWGN channels

We verified LDPC error floor by simulating 200-bit, 400-bit, 600-bit, and 1000-bit payload sizes with AWGN channel, with up to 107 simulated blocks.