3GPP TSG-RAN WG1 #92	R1- 1801498

Athens, Greece, 26th February – 2nd March 2018

Source:	Ericsson

Title:	NPRACH false alarm reduction for NB-IoT

Agenda Item:	   6.2.6.5.1

Document for:	Discussion and Decision

# Introduction

The objective of the work item on further enhanced NB-IoT is to introduce further support of enhanced features for NB-IoT, supporting standalone, guard-band, and in-band operation modes. One objective is to study, and if found necessary improve, NPRACH reliability and range enhancements as listed below.

NPRACH reliability and range enhancements

- If found necessary, reduce false alarm probability for NPRACH detection due to inter-cell interference on NPRACH [RAN1, RAN2, RAN4]

- If found necessary, introduce at least additional cyclic prefixes for NPRACH to support cell radius of at least 100 km [RAN1, RAN2, RAN4]

In RAN1#89, the following agreements were made with regards to NPRACH reliability enhancements:

- For reduction of NPRACH false alarm probability, FFS between:

  - Alt 1: Sharing the same NPRACH resources as Rel-13 NPRACH formats, with symbol or symbol-group level scrambling; maintaining feasibility of FFT processing and orthogonality of preambles on different tones

  - Alt 2: A frequency shift of k*0.75 kHz is applied to all NPRACH signal in a Cell.

    - FFS k=[-2, -1, 0, 1, 2] or [-2, -1 1 2].

    - FFS if to apply a phase rotation of m*pi/2 with m=0,1,2,3 is applied to the 4th symbol group of each repetition.

    - Signaling of the above frequency shift and phase rotation is FFS.

  - Alt 3: 3.75 kHz subcarrier spacing with minimum hop distance 3.75 kHz with new hopping pattern

- Combinations of the above alternatives are not precluded

In view of the agreements made during RAN1#89, we present a way forward for addressing the false alarm issue in Release 15 NPRACH design.

# Discussion of the alternatives identified at RAN1#89

## Alternative 1: “Symbol(-group) level scrambling”

### Sharing the same NPRACH resources as Rel-13 NPRACH formats

The NPRACH is a collision based access channel that needs to be dimensioned to facilitate targets in terms of reliability, latency and capacity. Typically, a preamble collision rate in the range of 1% to 10% is targeted when deploying a system, which implies that the available access opportunities should exceed the expected number of access attempts during peak hour with a factor ~10 to ~100 per time unit. This implies that the NPRACH may occupy a significant amount of the available uplink resources. This is also illustrated in Figure 1 where a configuration to cope with 2% collision ratio at ~12 access attempts per second is shown.

Figure 1: NPRACH resource configuration to cope with a high load of 12 accesses/s.

It is furthermore so that the NPRACH as minimum is occupying 12 out of 48 sub-carriers. This implies that even in a cell with a single user the NPRACH may occupy a significant amount of the resources, especially if that user has latency requirements since this requires a short NPRACH periodicity.

It is therefore beneficial if the NPRACH Release 15 design would provide the flexibility to reuse the resources configured for NPRACH Release 13. This would facilitate trunking efficiency gains and maintain today’s minimal configuration of 12 sub-carriers per NPRACH CE level. This will also maintain today’s ability to multiplex NPRACH with NPUSCH Format 2 (F2), which is important considering the somewhat limited scheduling flexibility supported by NPUSCH F2.

At the same time, it is acknowledged that a reuse of the Release 13 NPRACH resources may increase the computational complexity in the eNB. It is therefore beneficial to also allow configuration of Release 15 NPRACH resources separate from the Release 13 resources.

The NPRACH Release 15 design should support the flexibility to use dedicated time-frequency resources as well as to reuse the resources configured for Release 13/14 NPRACH.

### Symbol or symbol-group level scrambling

Scrambling of the NPRACH symbols may be an attractive solution due to its simplicity at the transmitter side where the Release 13/14 design to large parts can be reused. At the receiver side, the impact from the chosen solution may be more profound, as discussed in detail in R1-1712627 “NPRACH range enhancement for NB-IoT” [1]. Care should be taken to secure that the physical layer design also supports a reuse of the NPRACH resources as suggested in Proposal 1. To further limit the impact on the NB-IoT design, it is also our preference to, if possible, have a solution for improving false alarm performance that can also be applicable to all NPRACH formats including the format specified for supporting extended range.

The NPRACH Release 15 scrambling design should be applicable to all Release 13/14/15 NPRACH formats.

- To illustrate the meaning of Proposal 2 Figure 2 and Figure 3 illustrates how the pair wise scrambling concept (see [1] for details) can be applied on top of the Release 13 3.75 kHz and 6 symbol NPRACH format and also on top of a 1.25 kHz and 4 symbol NPRACH design. Both these designs are aligned with Proposal 1.

- Figure 2 Illustration of pairwise 3.75 kHz NPRACH symbol scrambling for the case of two repetitions with the same scrambling word used in each repetition.

- Figure 3 Illustration of pairwise 1.25 kHz NPRACH symbol scrambling for a symbol group having CP+3 symbols and 5 symbol groups per preamble repetition unit.

- The false alarm performance will depend on the length of the scrambling code word and its properties in terms of auto- and cross correlations. It is our expectation that there will be an association between a cell ID and a scrambling code word, so a large space of scrambling patterns will be useful since it reduces the required reuse of a scrambling pattern when deploying NB-IoT Release 15.

- The example in Figure 2 supports a codeword containing 12 scrambling symbols per preamble repetition unit. A potential design could be based on the Zadoff Chu sequence design:

- ZCRn=e-jRπn(n+1)N, n=0,1,…,N-1.

- Zadoff Chu is attractive due to providing favourable properties in terms of a constant amplitude zero autocorrelation waveform (CAZAC).  The constant amplitude has the potential to facilitate optimized transmitter power performance. The zero autocorrelation will help resolve the phase ambiguity associated with the Release 13 NPRACH design when attempting operation at a cell range beyond 40 km. To generate a multitude of orthogonal sequences a cyclic time shift, corresponding to the targeted cell size, of the scrambling symbols can be used just as in LTE. In addition, selecting the sequence length N as a prime number, e.g. 11, will support the generation of N-1 sequences, with a guaranteed cross correlation of 1/N, by means of varying the root R. To obtain a length-12 sequence, one way is to repeat the first scrambling symbol of the length-11 sequence at the end.

- An alternative to Zadoff Chu design is to base the scrambling pattern on Gold design. A pseudo random scrambling pattern can be designed per cell based on generating a pair of gold sequences that are mapped to QPSK constellations. A coupling between the scrambling pattern and the cell ID may be defined through the initialization of the scrambling pattern. Compared to the Zadoff Chu design this technique allows for generation of a larger number of sequences. The drawback is however that the auto- and cross-correlation properties are not well defined as in the case of Zadoff Chu.

- Based on this discussion, we believe that:

The NPRACH Release 15 scrambling design should seek to optimize the number of scrambling patterns and their properties in terms of auto- and cross correlation.

- For the example shown in Figure 3, the exemplified design will support 2 scrambling symbols per symbol group, resulting in a length-10 scrambling sequence for the preamble repetition unit. Similar to   Figure 2, the same scrambling sequence could be reused across repetition units.

## Alternative 2: “750 Hz channel spacing”

In alternative 2, it is considered to introduce a frequency shift of the NPRACH sub-carrier frequency raster per cell of granularity 0.75 kHz. The current specifications support a base station receiver implementation based on symbol by symbol equalization requiring a FFT resolution of 3.75 kHz as described in [1]. The proposed shift of the NPRACH channels would increase the requirement on FFT resolution by a factor 5 and thereby significantly increase the base station computational complexity. For an implementation based on FFT sizes of 2N, the increase in complexity would potentially be higher, i.e., by a factor 23 = 8.

Reducing the channel spacing by a factor of at least 5 will imply a significant increase in computational complexity at the base station side.

Shifting the NPRACH sub-carrier frequency raster while maintaining the sub-carrier grid for the NPUSCH and LTE sub-carriers will potentially also be a source of sub-carrier power leakage. This possible loss of orthogonality between NPRACH, NPUSCH and LTE sub-carriers needs to be assessed.

The impact from shifting the NPRACH sub-carrier frequency raster on the potential loss of orthogonality between NPRACH, NPUSCH and LTE sub-carriers needs to be evaluated.

One may consider shifting the whole FFT grid to avoid the disadvantages highlighted in Observations 2 and 3. But such shift is not known to LTE UE (inband) and Rel-13/14 NB-IoT UE. These legacy UEs would not shift the FFT grid. As a result, the shift of the whole FFT grid may cause unpredictable UE impact.

Shifting the whole FFT grid may cause unpredictable impact to LTE UE (inband) and Rel-13/14 NB-IoT UE.

Before further considering this alternative, a closer look must be taken at the aspects highlighted in Observations 2, 3 and 4.

## Alternative 3: “New FH pattern”

In R1-1709190 and R1-1712627  [3], [2], “Improved FH randomization for NB-IoT Rel-14,” it was identified that the pseudo random frequency hop between repeated random access frequency hopping symbol groups is time invariant across reoccurring NPRACH resources. This may be a source of false alarm especially for the important configuration of two NPRACH repetitions, which caters for up to at least 144 dB coupling loss. This solution is applicable on top of the symbol level scrambling improvement, but since it was not adapted for Rel-14 non-anchor carrier access it will in Rel-15 increase the number of total hopping patters that needs to be hypothesized by a eNB. This may impact existing eNB implementations negatively, which should be avoided.

A new frequency hopping pattern in Release 15 will increase the total number of hopping patterns that needs to be hypothesized by an eNB receiver. This may negatively impact the eNB implementation complexity.

- For 1.25 kHz NPRACH, we may use the frequency hopping approach proposed in R1-1709190 [3] and R1-1712627 [2], “Improved FH randomization for NB-IoT Rel-14” to reduce false alarm rate.

Improved frequency hopping pattern can be introduced for 1.25 kHz NPRACH to enhance reliability.

# Way Forward

We believe that the approach outlined in alternative 1 can be used to improve NPRACH reliability. In the past, various forms of symbol or symbol group-level scrambling have been considered for NPRACH false-alarm reduction. This includes arbitrary symbol-level scrambling, pairwise symbol-level scrambling, and symbol-group level scrambling. We highlight the pros and cons of each solution.

Table 1 Arbitrary symbol-level scrambling vs. Pairwise symbol-level scrambling

|  | Receiver complexity | Timing performance | False alarm reduction |
| --- | --- | --- | --- |
| Arbitrary symbol-level scrambling | High | Better | Yes |
| Pairwise symbol-level scrambling | Low | Good | Yes |

Using arbitrary symbol-level scrambling with symbol-by-symbol processing will cause inter-subcarrier interference. We can eliminate the inter-subcarrier interference by a 5x increase in the receiver FFT window. This, however, comes at the expense of an increased receiver complexity.

Using pairwise (symbol-level) scrambling with symbol-by-symbol processing of NPRACH preamble does not cause inter-subcarrier interference.

To improve the false-alarm rate using scrambling, we propose the following way forward.

- Symbol-level pairwise scrambling is proposed for false alarm reduction.

As an example, let us discuss the structure of a basic codeword set (see Table 1) for one symbol group. Using a similar approach, we may construct a (longer) scrambling sequence for the entire repetition unit consisting of 4 symbol groups.

Codeword set X: It contains L codewords consisting of 6 symbols each (including CP) and is constructed by L length-3 sequences.

  - Structure: As an example, consider Codeword 1 {Xa1,Xa1,Xb1,Xb1,Xc1,Xc1} where two adjacent symbols (counting from the first symbol) are identical to implement pairwise scrambling.

  - Due to pairwise scrambling, we require a length-3 sequence to generate Codeword set X.

  - The choice of length-3 sequences is FFS.

    - Some potential options include Zadoff-Chu sequences, Gold sequences, LTE orthogonal codes, etc.

  - The cardinality L of codeword set X is FFS.

    - We require log2(L) bits to represent Codeword set X.

- It is possible to leverage a longer scrambling sequence to further improve the false alarm rate. For example, using length-12 scrambling sequences, we may construct a codebook of length 24 codewords for the entire repetition unit (4 symbol groups with 6 symbols per symbol group). Another option is to use a cover code on top of the basic codewords defined for a symbol group (Codeword set X).

Table 2: Proposed scrambling codebook for pairwise scrambling for one symbol group.

|  |  | Symbol 0 (CP) | Symbol 1 | Symbol 2 | Symbol 3 | Symbol 4 | Symbol 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Codeword 1 | Xa1 | Xa1 | Xb1 | Xb1 | Xc1 | Xc1 |
| Codeword set X | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ |
|  | Codeword L | XaL | XaL | XbL | XbL | XcL | XcL |

# 4 Conclusions

In this contribution, we have discussed the support for NPRACH false alarm reduction. We made the following observations:

- The NPRACH Release 15 scrambling design should seek to optimize the number of scrambling patterns and their properties in terms of auto- and cross correlation.

Reducing the channel spacing by a factor of at least 5 will imply a significant increase in computational complexity at the base station side.

A shift of the NPRACH sub-carrier frequency raster will potentially impact orthogonality between NPRACH, NPUSCH and LTE sub-carriers.

Shifting the whole FFT grid may cause unpredictable impact to LTE UE (inband) and Rel-13/14 NB-IoT UE.

A new frequency hopping pattern in Release 15 will increase the total number of hopping patterns that needs to be hypothesized by an eNB receiver. This may negatively impact the eNB implementation complexity.

Improved frequency hopping pattern can be introduced for 1.25 kHz NPRACH to enhance reliability.

Using arbitrary symbol-level scrambling with symbol-by-symbol processing will cause inter-subcarrier interference. We can eliminate the inter-subcarrier interference by increasing the receiver FFT window by 5 times. This, however, comes at the expense of an increased receiver complexity.

Using pairwise (symbol-level) scrambling with symbol-by-symbol processing of NPRACH preamble does not cause inter-subcarrier interference.

It is possible to leverage a longer scrambling sequence to further improve the false alarm rate. For example, using length-12 scrambling sequences, we may construct a codebook of length-24 codewords for the entire repetition unit (4 symbol groups with 6 symbols per symbol group). Another option is to use a cover code on top of the basic codewords defined for a symbol group (Codeword set X).

Based on the discussion we made the following proposals.

- The NPRACH Release 15 design should support the flexibility to use dedicated time-frequency resources as well as to reuse the resources configured for Release 13/14 NPRACH.

- The NPRACH Release 15 scrambling design should be applicable to all Release 13/14/15 NPRACH formats.

- Pairwise symbol-level scrambling is proposed for false alarm reduction.

# References

R1-1712627, “NPRACH range enhancement for NB-IoT”, RAN1#90, Ericsson.

R1-1712636, “Improved FH randomization for NB-IoT Rel-14”, RAN1#90, Ericsson

R1-1709190, “Improved FH randomization for NB-IoT Rel-14”, RAN1#90, Ericsson