3GPP TSG RAN WG1 Meeting 92	R1-1801573

Athens, Greece, 26nd Feb-2nd Mar 2018

Agenda Item:	7.2.5

Source:	Ericsson

Title:	URLLC downlink system level simulation results

Document for:	Information

# Introduction

In this paper we assume the following URLLC requirements:

Reliability: 10-5

Latency: 1ms

and evaluate the system level performance for periodic small packets in DL.

# Simulation setup

The simulation was performed on 700 MHz band, but otherwise we followed the assumptions in 38.802 [1]. The simulation setup is summarized in the table below.

| Numerology | 7000 TTIs/sec, 4 OFDM/TTI |
| --- | --- |
| Duplex | FDD |
| Propagation | SCM 3D urban macro, wrap-around |
| Carrier frequency/bandwidth | 700 MHz, 20 MHz (50 PRBs) |
| Deployment | Hexagonal Grid, One 3-sector site |
| BS TX power | 46 dBm |
| UE TX power | 23 dBm |
| ISD | 500 m |
| BS antenna configuration | 8 TX/RX, 2x2 X-pol antenna array, separation = 0.5 lambda, height = 25 m |
| UE antenna configuration | 1 TX, 4 RX, 1x2 X-pol, λ=0.5 |
| PDCCH | 1 symbol (symbol #0), 6 REGs/CCE, 1/3 DMRS overhead |
| PDSCH | 3 symbols (symbol #1-#3), DMRS: 6 RE per PRB |
| Short PUCCH | 1 symbol (symbol #3) |
| PUSCH | 3 symbols (symbol #0-#2), not frequency-multiplexed with PUCCH, DMRS: In symbol #0. |
| Traffic model | Periodic packets of size 32 B on application level. Both DL and UL traffic. TBS comprises overhead for PDCP, RLC UM and MAC |
| UE distribution | No eMBB traffic 20% outdoor: 30 km/h 80% indoor: 3 km/h 30 UEs in 3 sectors, i.e. on average 10 UE per sector |
| Mobility | Multi-path only (no macro mobility) |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| CSI | A-periodic CSI, targeting every 5th TTI, R-14 8-port codebook |
| Impairments | Ideal channel estimation |

The used timing relating to downlink is according to the table below.

| DL data arrive DL data assignment | 4 OFDM |
| --- | --- |
| DL data HARQ-ACK | 3 OFDM |
| NACK DL retransmission | 4 OFDM |

The HARQ RTT is hence n+3 and within 1ms delay budget there is time to perform one retransmission.

# Simulation results

The system performance in terms of the fraction of URLLC UEs that fulfil the URLLC requirements (10-5 reliablity and 1ms latency) is evaluated with respect to offered load (number of 32B packets per second and UE).  The simulation result is shown in Figure 1 below.

Figure 1: Downlink URLLC performance. The bars indicate 95% confidence interval.

We see that with a 5% outage limit the URLLC capacity is approximately 420 32B packets per second and UE. Since there are, on average, 10 UEs per sector (cell), approximately 1.08 Mbps offered URLLC cell load is achievable with fulfilled URLLC requirements assuming a 5% outage limit.

- On a 20 MHz carrier on 700 MHz band with 30 kHz SCS, approximately 1.08 Mbps offered URLLC cell load is achievable with fulfilled URLLC requirements assuming a 5% outage limit.

It should be noted that both DL and UL URLLC traffic is active. Hence, UL grants will compete with DL assignments for PDCCH resources. With pure DL traffic the achievable offered load may hence be higher.

Figure 2 below shows a histogram of number of CCEs per PDCCH.

Figure 2: Histogram of number of CCEs per PDCCH

We can see that with increased offered load on PDCCH the number of used CCEs increases. This will cause link adaption to select higher aggregation level due to increased interference on PDCCH. At 400 packets/sec/UE offered load, approximately 83% of the PDCCH transmissions use maximum aggregation level. Since the carrier bandwidth consists of 50 PRB and PDCCH REGs/CCE is 6, then if a UL grant or DL assignment uses 8 CCEs no more UL grants or DL assignments can be scheduled on PDCCH. Furthermore, since there are on average 10 UE per cell with both DL and UL traffic the 400 packets/sec/UE offered load means that 8000 assignments/grants per cell and second need to be allocated on PDCCH. Since there are 7000 TTI/sec we can conclude that the DL URLLC performance is likely limited by PDCCH. We hence make the following observation.

- PDCCH may be a bottleneck for DL URLLC performance.

# Conclusion

In Section 3 we made to following observation.

- On a 20 MHz carrier on 700 MHz band with 30 kHz SCS, approximately 1.08 Mbps offered URLLC cell load is achievable with fulfilled URLLC requirements assuming a 5% outage limit.

- PDCCH may be a bottleneck for DL URLLC performance.

# References

38.802 V14.2.0