3GPP TSG RAN WG1 Meeting #92                                         R1- 1801614

Athens, Greece, February 26th – March 2nd 2017

Source:               ZTE

Title:                    Details design of wake up signal for NB-IoT

Agenda item:      6.2.6.1.1.3

Document for:    Discussion and Decision

Introduction

In RAN #75 meeting, new WID RP-170852 on Even Further NB-IoT enhancements was agreed as working agreement [1]. One of the objectives is to reduce power consumption.

- Study and, if found beneficial, specify for idle mode paging and/or connected mode DRX, physical signal/channel that can be efficiently decoded or detected prior to decoding NPDCCH/NPDSCH. [RAN1,  RAN2,  RAN4]

In this contribution, we discuss the support of power consumption reduction for decoding the physical downlink control/data channel.

Discussion on designing wake up signal

In RAN1#91 meeting[2], the following agreement was achieved for designing wake-up signal:

WUS sequence is based on ZC-sequence

- When designing WUS sequence, negative impact on legacy NSSS detection should be avoided.

The definition of ZC sequence is shown as below:

There are two choices for L:

ALT-1: L is based on the available RE number inside one subframe

ALT-2: L is based on the total available RE number of N subframes

Compare with ALT-1, ALT-2 corresponds to longer sequence. With longer sequence, there are more root sequence and this is beneficial for cell interference randomization. However, this is based on the assumption that the corresponding subframes are continuous. In reality, for the downlink, considering the existing valid subframe and the collision with other messages[3], there is no guarantee that the subframes WUS mapping to are continuous. If the sequence between adjacent cells are not aligned, the correlation property of the sequences will be impacted. For ALT-1, if the subframe are aligned, then the correlation property of the sequence can be maintained. For ALT-2, if the N value is fixed, more resource will be wasted. If the N value is not fixed, then the sequence length of different cells could be different. Therefore, the cross-correlation property of the sequences cannot be maintained.  From the above analysis ALT-1 is preferred.

Proposal 1:  The length of the ZC sequence is based on the available RE number inside one subframe.

Based on the assumption that ZC sequence is used, there are two designs for the WUS sequence:

Alt 1:Based on existing NSSS sequence

To differentiate WUS sequence with legacy NSSS, the following alternatives can be considered:

Alt 1-1: Using different Hadamard sequence for WUS.  For example, WUS can use the following indexed Hadmard sequence:

Alt 1-2: Using different mapping for WUS [3]

For the legacy NSSS, the mapping is done in frequency domain and then in time domain. For WUS, it can use the mapping that is done first in time domain then in frequency domain.

Alt 2: ZC sequence*PN sequence

Similar to NSSS, the PN sequence is used to replace the Hadamard sequence in NSSS, where in b(m) is a cover code using a PN sequence, one example is Gold sequence[4].

When the root sequences of the ZC sequence corresponding to are same, the correlation property of the sequences  is same as pseudo random sequence.

Alt 3: ZC sequence

Considering the excellent correlation property of the ZC sequence, it is one of the alternative candidate for WUS.

The performance of the above sequence is simulated.

- The impact to legacy NSSS should be considered when choosing WUS sequence, especially when there is no repetition for WUS.  When there is repetition for WUS, the repetition is on contiguous subframes, while the repetition for NSSS is once per 20ms. UE can differentiate NSSS and WUS during energy detection. But if the WUS is configured without repetition, the impact cannot be neglected.  Simulation the of the impact of NSSS is shown in appendix.

Observation 1:For Alt 1 and Alt2, the impact  to NSSS can be neglected; Alt3 has relatively larger impact on NSSS.

- For WUS signal, the impacts of miss detection is more serious. The table below gives the miss detection rate with false detection rate =10%. The simulation parameters are in the appendix.

Table 1  Miss detection rate for different WUS sequence

|  |  | MCL=144dB | MCL=154dB | MCL=164dB |
| --- | --- | --- | --- | --- |
| Alt 1-1 | Without interference cell | 0.0022 | 0.0308 | 0.2546 |
|  | With subframe aligned interference cell | 0.0079 | 0.0688 | 0.4559 |
|  | Without subframe aligned interference cell | 0.0070 | 0.0627 | 0.4265 |
| Alt 1-2 | Without interference cell | 0.0023 | 0.0317 | 0.2546 |
|  | With subframe aligned interference cell | 0.0076 | 0.0688 | 0.4584 |
|  | Without subframe aligned interference cell | 0.0082 | 0.0720 | 0.4743 |
| Alt 2 | Without interference cell | 0.0021 | 0.031 | 0.2549 |
|  | With subframe aligned interference cell | 0.0070 | 0.0685 | 0.4539 |
|  | Without subframe aligned interference cell | 0.0069 | 0.0673 | 0.4454 |
| Alt 3 | Without interference cell | 0.0021 | 0.0304 | 0.2555 |
|  | With subframe aligned interference cell | 0.0074 | 0.0678 | 0.4570 |
|  | Without subframe aligned interference cell | 0.0066 | 0.0607 | 0.4210 |

Based on the above simulation, we have the following observation

Observation 2:In term of miss detection rate, Alt1- 1 and Alt 3 are  preferable.

Considering that ZC*Hadmard sequence is also used in legacy NSSS, UE implementation cost will not increase if WUS also adopts the same sequence structure. Together with the previous two observation, we have the following proposal:

Proposal 2:  ZC*Hadamard sequence is used for WUS.

Furthermore, current sequence hopping can also be introduced to further lower the inter-cell interferences. The detail hopping mechanism can be FFS.

When WUS sequence is used also for time synchronization and the WUS sequence is transmitted with repetition, if the sequence of each repetition is exactly same, timing error could happen. Therefore, subframe index information need to be carried in the WUS sequence. One way to do this is to use similar methods in NSS, where cyclic-shift is used to indicate subframe index. Or , if subframe based hopping is used, e.g., root sequence of ZC is different for different WUS subframes.

Proposal 3: It is preferable to introduce sequence hopping to further reduce inter-cell interferences.

Proposal 4: Subframe based hopping can be used to indicate WUS timing.

Conclusions

In this contribution, we have discussed the power consumption reduction for NB-IoT. We make the following observations and proposals:

Observation 1:For Alt 1 and Alt2, the impact to NSSS can be neglected; Alt3 has relatively larger impact on NSSS.

Observation 2:In term of miss detection rate, Alt1- 1 and Alt 3 are  preferable.

Proposal 1: The length of the ZC sequence is based on the available RE number inside one subframe.

Proposal 2: ZC*Hadamard sequence is used for WUS.

Proposal 3: It is preferable to introduce sequence hopping to further reduce inter-cell interferences.

Proposal 4: Subframe based hopping can be used to indicate WUS timing.

- References

- 3GPP, RP-170852, New WID on Further NB-IoT enhancements, Huawei, HiSilicon, Neul, RAN #75

- 3GPP, Draft Report of 3GPP TSG RAN WG1 #90bis, v0.1.0

- 3GPP, R1-1719879, Wake up signal design, LG Electronics, RAN1#91

- 3GPP, R1-1718142, Wake-up signal design, Qualcomm Incorporated, RAN1#90bis

- Appendix A

Table A.1 Simulation Assumption

| Parameter | Value |
| --- | --- |
| BS TX antenna configuration | 2 Tx for in-band/guard-band |
| BS power | 35 dBm for Inband/guardband; |
| System BW | 180kHz |
| Band | 900 MHz |
| Channel model | TU |
| Doppler spread | 1 Hz |
| UE RX antenna configuration | 1 Rx |
| UE NF | 5 dB |
| Coupling loss | 144dB, 154dB, 164 dB |
| Cell ID | Target Cell: 1 Interference Cell: 20 |

- Appendix B

Case A: NSSS impact to WUS

Table B.1 Miss detection rate of WUS

|  | MCL =144dB |  |
| --- | --- | --- |
| Alt 1-1 | With subframe aligned interference cell | 0.0057 |
|  | Without subframe aligned interference cell | 0.0072 |
| Alt 1-2 | With subframe aligned interference cell | 0.0075 |
|  | Without subframe aligned interference cell | 0.0078 |
| Alt 2 | With subframe aligned interference cell | 0.0069 |
|  | Without subframe aligned interference cell | 0.0068 |
| Alt 3 | With subframe aligned interference cell | 0.0075 |
|  | Without subframe aligned interference cell | 0.0065 |

Case B :WUS’s impact to NSSS

Table B.2 NSSS detection error rate

|  | MCL =144dB |  |
| --- | --- | --- |
| Alt 1-1 | With subframe aligned interference cell | 0.015 |
|  | Without subframe aligned interference cell | 0.0151 |
| Alt 1-2 | With subframe aligned interference cell | 0.0158 |
|  | Without subframe aligned interference cell | 0.014 |
| Alt 2 | With subframe aligned interference cell | 0.0192 |
|  | Without subframe aligned interference cell | 0.0127 |
| Alt 3 | With subframe aligned interference cell | 0.45 |
|  | Without subframe aligned interference cell | 0.0157 |