3GPP TSG RAN WG1 Meeting #92		R1-1801635

Athens, Greece, February 26th – March 2nd, 2018

Source: 				ZTE, Sanechips

Title: 					On short PUCCH enhancement for NR URLLC

Agenda item:		7.2.5

Document for:	Discussion and Decision

# Introduction

In the RAN plenary #78 meeting [1], it was approved that RAN1 shall further specify URLLC work for Rel-15 before June based on the scope defined in [2]. In this contribution, we mainly discuss short PUCCH enhancement for NR URLLC in case of simultaneous transmission of HARQ-ACK and SR.

Inthe RAN1#91meeting, the following agreements onNR short PUCCH were reached [3].

Agreements: - For simultaneous transmission of 1 or 2 bits HARQ-ACK and SR using PUCCH format 0: - In case of negative SR, the same PUCCH resources as for HARQ-ACK only transmission are used. - In case of positive SR, HARQ-ACK are transmitted on the PRB for HARQ-ACK only transmission. - The mapping of ACK and NACK to cyclic shifts is based on the index of initial cyclic shift of the HARQ-ACK only (CSinitial) and a fixed mapping pattern as given in Table 1 and Table 2 below corresponding to 1 and 2 bits HARQ-ACK, respectively. # Table 1: Mapping pattern for 1-bit HARQ-ACK and positive SR HARQ-ACK; NACK; ACK. Cyclicshift; (CSintitial+3)mod12; (CSintitial+9)mod12. # Table 2: Mapping pattern for 2-bit HARQ-ACK and positive SR HARQ-ACK; NACK, NACK; NACK, ACK; ACK, ACK; ACK, NACK. Cyclicshift; (CSintitial+1)mod12; (CSintitial+4)mod12; (CSintitial+7)mod12; (CSintitial+10)mod12. - Note: Maximum 12 SR per PRB can be configured with semi-static SR simultaneously. - One PRB can support simultaneous transmission of 2-bit HARQ-ACK with SR only for one UE. - The four remaining resources can be used for other purposes (e.g. 1-bit A/N with SR or 2-bit A/N only)

# Simultaneous transmission of HARQ-ACK and SR

For the simultaneous transmission of SR and HARQ-ACK in PUCCH format 0, a decision was made in the RAN1#91 meeting to meet the Phase 1 timeline. In that context, it was agreed the four cyclic shifts for HARQ-ACK only and the four cyclic shifts for HARQ-ACK and positive SR are in one PRB, and one SR resource is separately configured to the UE for SR only transmission. The agreed scheme is illustrated as Alt 1 in Figure 1.

However, always using 8 cyclic shifts in one PRB would be detrimental to the PUCCH performance, especially in case of a relatively large delay spread with a small SCS or a relatively small delay spread with a large SCS. This was proven in [3][4], and also verified by the simulation results for SR shown in the Appendix. On the other hand, URLLC UEs require an extremely high reliability e.g. 10-5 error probability. Therefore, an enhancement on PUCCH is crucial for URLLC.

Figure 1 Simultaneous transmission of SR and 2-bit HARQ-ACK in PUCCH format 0

One simple remedy is to allocate one more PRB for transmission of HARQ-ACK and positive SR, as Alt 2 shown in Figure 1. This is crucial especially for URLLC UE. In order to align with what already on the table as much as possible, the additional PRB, denoting as PRB #j could be the same as the PRB configured for SR only. The cyclic shifts within PRB #j could be also implicitly indicated by the initial cyclic shift CSinitial configured for HARQ-ACK only.

Given CSinitial is used for ‘NACK, NACK’ transmission, gNB could indicate a same CSinitial as the cyclic shift configured for SR only, as an implementation issue. Then, in such case one resource could be saved. Note, this also aligns with the rules defined in LTE, i.e., the modulation symbol of SR d(0)=1 is always the same as the modulation symbol for positive SR and ‘NACK’/’NACK,NACK’ in PUCCH format 1a/1b.

Proposal 1: For the simultaneous transmission of SR and 2-bit HARQ-ACK in PUCCH format 0, adopt a complementary scheme for URLLC,

- the PRB for transmission of HARQ-ACK and positive SR is the same as the PRB configured for SR only.

- the cyclic shifts for transmission of HARQ-ACK and positive SR are implicitly indicated by the initial cyclic shift configured for HARQ-ACK only.

# Conclusion

In this contribution, the enhancement of short PUCCH for URLLC is discussed and we have the following proposal:

Proposal 1: For the simultaneous transmission of SR and 2-bit HARQ-ACK in PUCCH format 0, adopt a complementary scheme for URLLC,

- the PRB for transmission of HARQ-ACK and positive SR is the same as the PRB configured for SR only.

- the cyclic shifts for transmission of HARQ-ACK and positive SR are implicitly indicated by the initial cyclic shift configured for HARQ-ACK only.

# References

- 3GPP TSG RAN Meeting RAN#78, NR workload management, RP-172844.

- 3GPP TSG RAN Meeting RAN#78,RP-172817, NR High-Reliability URLLC scope for RAN1/RAN2, Ericsson.

- 3GPP TSG RAN WG1 Meeting RAN1#91, Chairman’s Note.

- 3GPP RAN1#91.R1-1720326, Remaining Issues for Short PUCCH with UCI of 1 or 2 Bits, Samsung.

- 3GPP RAN1# AH_NR#3,2017. R1-1715522 On short PUCCH for up to 2 bits UCI, ZTE,Sanechips.

# Appendix

Table ASimulation parameters for link-level simulation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 60/120 Khz |
| System bandwidth | 40 MHz |
| Number of symbol | 1 symbol |
| Number of PRB | 1 PRB / 2 PRB |
| Channel model | TDL-C, Delay Spread(DS)= 100ns ,300ns |
| UE speed | 3km/h |
| Antenna configuration | 1Tx(UE), 2 Rx(gNB) |
| CP length | Normal |
| UCI bits | 2bits HARQ-ACK, SR |
| Detection method | Non-coherent detection |

Figure A-1 SR performance in short PUCCH with simultaneous transmission of 2 bits HARQ-ACK