3GPP TSG RAN WG1 Meeting #92                                               R1-1801689

Athens, Greece, 26th February – 2nd March 2018

Agenda item:            6.1.7

Source:	MediaTek Inc.

Title:	Discussion on Modulation Enhancement

Document for:          Discussion

1	Introduction

Less available REs degrades PDSCH performance, especially when CFI=3 and 32-port CSI-RS occur in a subframe. In RAN1#91, five solutions [1] are proposed:

- RAN1 to consider one of the following solutions to this issue in Rel-15 under TEI-15:

  - Solution 1: Select modulation scheme based on TBS and number of REs used for rate matching.

  - Solution 2: Introduce “Modulation overriding” field in DCI to change the modulation scheme.

  - Solution 3: Introduce an alternative table for 256QAM by RRC configuration.

  - Solution 4: Extend the MCS field in DCI to 6 bits, and introduce overlapped entries with different modulation scheme.

    - Study the possibility of a single MCS table covering from QPSK to 1024QAM.

  - Solution 5: N_RB scaling for TBS selection (similar to TBS selection for special subframes)

- No changes to Rel-12/13/14 64QAM/256 QAM TBS/MCS entries

In this paper, we discuss the possible solutions and suggest to adopt solution 5 for modulation enhancement. And, we further provide detailed design for solution 5.

2     	Discussion on Solutions

PDSCH performance is one of the design considerations for UE CSI. To reduce CSI complexity, the following two design considerations shall be evaluated in PDSCH performance enhancement:

- Consideration 1: For a given MCS, consistent PDSCH performance is preferred so as to avoid complicated UE-OLLA implementation.

- Consideration 2: Keep same number of MCS index. Larger number of MCS index also increases the UE-OLLA complexity.

Table 1 summarizes our evaluation. Regarding solution 1~3, modulation overriding and (table) selection lead to inconsistent PDSCH performance. As shown in Figure 1, for fixed MCS and fixed TBS, the required SNR to achieve 10% BLER varies from 2.4 dB to 4.1 dB because of introducing modulation order changes. Therefore solution 1~3 are not preferred due to the increased complexity of UE-OLLA. Regarding solution 4, MCS field in DCI is extended to 6bits. CSI implementation complexity could be doubled because UE-OLLA design shall consider at most 64 MCS indices (originally 32 MCS indices).

| Solution | Comments |
| --- | --- |
| Solution 1 | Consideration 1 is not met |
| Solution 2 | Consideration 1 is not met |
| Solution 3 | Consideration 1 is not met |
| Solution 4 | Consideration 2 is not met |
| Solution 5 | Both considerations are met |

Table 1: Comments on solutions

Figure 1: PDSCH performance with different modulation orders

Regarding solution 5, both mentioned design considerations are meet, and it is UE-implementation friendly.

Proposal 1: Adopt solution 5 for MCS enhancement.

3	Design on Solution 5

The detailed designs on N_RB scaling factor for TBS selection and trigger conditions are proposed for solution 5. Moreover, solution 5 can be applied for all MCS indices. The multi-stage architecture is used for consistent coderate maintenance. And the procedure can be broken down as following steps:

Step 1: Calculate available RE number for PDSCH data transmission, namely Avail_RE

- Base on PRB allocation in DCI

- Exclude CRS, control region, DMRS, CSI-RS, and EPDCCH

Step 2: Calculate all RE number for PDSCH data transmission, namely All_RE

  - Base on PRB allocation in DCI

  - Exclude CRS, control region with CFI of 1, and DMRS

Step 3: Derive r = ratio of Avail_RE over All_RE

Step 4: if (r < (4.5/8 = 0.5625)),           N_RB scaling factor = 4/8 = 0.5;

else if (r < (5.5/8 = 0.6875)),  N_RB scaling factor  = 5/8 = 0.625;

else if (r < (6.5/8 = 0.8125)),  N_RB scaling factor  = 6/8 = 0.75;

else if (r < (7.5/8 = 0.9375)),  N_RB scaling factor  = 7/8= 0.825;

else                                         ,  N_RB scaling factor  = 8/8 =1;

Table 2 lists two cases for N_RB scaling. It can be observed the coderate difference between two cases is 0.062 and it also demonstrates that solution 5 can provide more consistent PDSCH performance.

|  | Value |  |
| --- | --- | --- |
| Parameter | Case 1 | Case 2 |
| DL cell bandwidth | 10MHz |  |
| CRS port number | 2-CRS port |  |
| DMRS port number | 2-DMRS port |  |
| PDSCH PRB number | 100 (from DCI) |  |
| MCS | 16 (64QAM table) |  |
| ITBS | 15 |  |
| CFI | 3(from PCFICH) | 1 (from PCFICH) |
| CSI-RS port number | 32-CSI-RS port | N.A. |
| EPDCCH | 2 active PRBs | N.A. |
| N_RB scaling factor | 0.625 | 1 |
| TBS (bits) | 19080 | 30576 |
| TBS+CRCs (bits) | 19200 | 30720 |
| Coderate | 0.644 | 0.581 |

Table 2: Examples of N_RB scaling

N_PR scaling factor calculation for case 1:

Step 1: All_RE = 100 * (12* 14 -12(CFI =1) - 12 (CRS port) -12 (DMRS port)) = 13200 REs

Step 2: Avail_RE = (100 -2(EPDCCH))*(12*14- 36 (CFI from PCFICH) - 12 (CRS port) -12 (DMRS port) - 32 (CSI-RS port)) = 98*76 = 7448 REs

Step 3: r = 7448/13200 = 0.56422

Step 4: N_RB scaling factor = 0.625, PRB for TBS is floor(100*0.625) = 62 PRBs.

4	Summary

Based on the discussion in section 2, we have the following proposal:

Proposal 1: Adopt solution 5 for MCS enhancement.

5	Reference

[1] R1-1721268, ``WF on modulation enhancements,’’ Qualcomm, Intel, Verizon, KDDI, Samsung