3GPP TSG RAN WG1 Meeting #92		R1-1801708

Athens, Greece, February 26th – March 2nd, 2018

Source:	CATT

Title:	Remaining Issues on NR PBCH

Agenda Item:	7.1.1.2.1

Document for:	Discussion and Decision

# Introduction

In RAN1#91 meeting, the following agreements were made during the email discussion “[91-NR-04] RMSI presence indication”[1]:

Agreements: For an SSB on the sync raster, the indication of no associated RMSI is done using reserved value(s) in SSB-subcarrier-offset. If no RMSI present, RMSI-PDCCH-Config is used to signal the next sync raster that UE should search for cell-defining SSB.

In RAN1#AH1801, the above issue was discussed again and the following conclusion was made [2]

Conclusion:

Regarding signaling the valid search raster:

- No additional signaling entries in the PBCH (for example cellBarred, intraFreqReselection) are to be used to inform the UE of the valid raster

- The indication is only for the same frequency band

There was, however, no agreement on how to use RMSI-PDCCH-Config to indicate the next sync raster that UE should search for cell-defining SSB. In this contribution, we further discuss how an SSB without associated RMSI is indicated through reserved value(s) in SSB-subcarrier-offset, and also when no RMSI is present, how to use RMSI-PDCCH-Config to indicate the next sync raster where UE should search for cell-defining SSB. The paper is an updated version of our previous contribution R1-1800228.

# Discussion

In RAN1#AH1801, the following two proposals were discussed on how to how to use RMSI-PDCCH-Config to indicate the next sync raster [2]:

- Alt1: Signaling of the absolute raster position within the frequency band

- Alt2: Signaling of the raster position relative to the found raster position

In addition, there is a need to consider whether

- NR supports signaling the case when there is no indicated raster position, there is no RMSI present

- the ±5 kHz offset be indicated in FR1

## Indication of the sync raster for cell-defining SSB

The parameters in the NR-PBCH are listed in the following table:

Table 1: Contents and payload size of NR-PBCH

| NR-PBCH IEs | Above 6 GHz (bits) | Sub 6 GHz (bits) |
| --- | --- | --- |
| SFN | 10 | 10 |
| MSB of SS block time index | 3 | 0 |
| Half radio frame index | 1 | 1 |
| Indication of the first DL DMRS position | 1 | 1 |
| SCS indicator of RMSI/OSI/Msg.2/Msg.4 | 1 | 1 |
| PRB grid offset (i.e., ssb-subcarrierOffset in TS 38.331) | 4 | 5Note1 |
| RMSI-PDCCH-Config (i.e., pdcchConfigSIB1 in TS 38.331) | 8 | 8 |
| Identification that UE cannot camp on the cell | 1 | 1 |
| Indicates that intraFreqReselection is not allowed | 1 | 1 |
| Reserved (RAN2) | 1 | 1 |
| Spare | 1 | 3 |
| CRC | 24 | 24 |
| Total | 56 | 56 |

Note 1: For sub-6GHz, 5 bits are needed for the PRB grid offset. Because for sub-6GHz there is no need to indicate SS block time index, 1 of the 3 bits for SS block time index can be used for the indication of the PRB grid offset.

As shown in Table 1, if a cell defining SS/PBCH block (CD-SSB) has associated RMSI, 8-bits parameter RMSI-PDCCH-Config is used for the configuration of the RMSI CORESET. A CD-SSB must be placed at an SS Raster. But, it does not mean an SSB placed at an SS Raster has to be a CD-SSB. There are following two cases that a non CD-SSB is positioned at the SS Raster:

- SSB transmission periodicity is configured to be shorter than the RMSI transmission periodicity. In this case, some SSBs allocated at the same center frequency as the CD-SSB will not have the associated RMSI;

- The network transmits some SSBs purely for the RRM measurements. In this case these non CD-SSBs are positioned at the RB boundaries of the common RB grid. It happens that these SSB are placed at the frequency locations where RB boundaries of the common RB grid overlap with a SS Raster.

Because a non CD-SSB can be positioned at the SS Raster as described above, during cell search a UE may detect non CD-SSB before it finds a CD-SSB. It was agreed in RAN1#91 that

For an SSB on the SS Raster, the indication of no associated RMSI is done using reserved value(s) in SSB-subcarrier-offset. If no RMSI present, RMSI-PDCCH-Config is used to signal the next SS Raster that UE should search for cell-defining SSB.

The intention of above agreement is to inform a UE the exact location of the SS Raster with CD-SSBs during cell search to minimize UE cell search time, if it happens that a UE detects a non CD-SSB on the SS Raster before it detects a CD-SSB.

When the carrier frequency is lower than 6GHz, the parameter PRB-grid-offset has 5 bits to indicate maximum 24 possible subcarrier-offsets between the boundary of the SSB PRB and the RMSI PRB (i.e., for the case when {SSB SCS, RMSI SCS}={15kHz, 30kHz}). Since 5 bits can indicate up to 32 possible values, there are 8 remaining values that can be used together with 8 bits parameter RMSI-PDCCH-Config for the indication of no RMSI present and the next SS Raster with CD-SSBs. The maximum number of the SS Raster positions can be indicated is 8*256 = 2048.

When the carrier frequency is above 6GHz, the parameter PRB-grid-offset has 4 bits, which indicates the total 12 possible subcarrier-offsets between the SSB and the RMSI. Since 4 bits can indicate up to 16 possible values, there are 4 remaining values that can be used together with 8 bits RMSI-PDCCH-Config for the indication of no RMSI present and the next SS Raster with CD-SSBs. The maximum number of the SS Raster positions can be indicated is 4*256 = 1024.

In the LS from RAN4 [4], the following information is provided for the SS raster:

- SS Raster will indicate the position of RE=#0(subcarrier #0) of RB#10 of the SS block

- SS Raster Definition for the 0-2.65GHz range

  - SS Raster is defined as:

    - Raster entry=N*900kHz+ M*5kHz, N=1:[2944], M=-1:1

    - Raster entry numbering is in increasing order of frequency (first entry=895kHz, entry #2=900, entry #3=905, entry#4=1795kHz…)

    - Total number of entries [8832]

  - Raster applies to bands with 100kHz channel raster and 15kHz channel raster with 5MHz minimum bandwidth(bands might be defined in the future) in this frequency range

  - For 10MHz minimum channel bandwidth a down selection factor of [6](1 entry in [6]) is used

- SS Raster Definition for the 2.4-24.25 GHz range

  - SS Raster is defined as:

    - Raster entry= 2400MHz+N*1.44MHz,N= 0:[15173]

    - Raster entry numbering is in increasing order of frequency

    - Total number of entries [15174]

  - Raster applies to bands with 15kHz channel raster and 10MHz minimum bandwidth(n41, n77,n78, n79, etc) in this frequency range

  - For 10MHz minimum channel bandwidth and 15kHz SS block SCS a down selection factor of [3] is applied

  - For 40MHz minimum channel bandwidth and 30kHz SS block SCS a down selection factor of [21]

- SS Raster Definition for the 24.25-100GHz range

  - SS Raster is defined as:

    - Raster entry=[24250.08] MHz+N*[17.28]MHz, N= 0:[4383]

    - Raster entry numbering is in increasing order of frequency

    - Total number of entries [4384]

  - Raster applies to bands with 60kHz channel raster in this frequency range

  - For 100MHz minimum channel bandwidth and 240kHz SS block SCS a down selection factor of [2] can be used

The LS provides separate SS Raster Definitions for three frequency ranges (FRs) of [0-2.65GHz], [2.4-24.25 GHz] and [24.25-100GHz], since there is actually no NR band operating in FR [6 - 24.25] as shown in Table 2, we only need to consider how to indicate the next SS Raster in the following frequency ranges: [ 0-2.65GHz], [2.4-6GHz] and [24.25-100GHz].

Table 2 Definition of frequency ranges (Table 5.1-1 in TS 38.101-1)

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 450 MHz – 6000 MHz |
| FR2 | 24250 MHz – 52600 MHz |

Thus, we could have the following options on how to define the requirements for the indication of the next SS Raster positions:

- Alt.1: [0-2.65GHz], [2.4-6GHz] and [24.25-100GHz] separately;

- Alt.2: above 6GHz and sub-6GHz separately;

- Alt.3: not depend on the frequency ranges;

Which option is used will depend on the number of next SS Raster positions that need to be presented for each of the options.

As discussed above, the maximum number of the next SS Raster positions can be indicated by the use of remaining values in the parameter PRB grid offset and the 8 bits RMSI-PDCCH-Config is limited to 8*256 = 2048 SS Raster positions for sub-6GHz and 4*256 = 1024 SS Raster positions for above-6GHz. From the RAN4 LS [4], the total number of SS Raster positions in each FR is much larger than these numbers. Thus, it is clear that using remaining values in the parameter PRB grid offset  and the 8 bits RMSI-PDCCH-Config cannot indicate the all SS Raster positions in each of the FRs [0-2.65GHz], [2.4-6GHz] and [24.25-100GHz].

Since there is no enough bits to indicate the all SS Raster positions in each of the FRs [0-2.65GHz], [2.4-6GHz] and [24.25-100GHz], we will then check if it is possible to indicate the next SS Raster positions within a band for all bands. Table 3 and 4 gives the bandwidths for the bands agreed so far in TS 38.101-1 and TS 38.101-2.

Table 3: Bandwidth of NR operating bands in FR1 (Based on Table 5.2-1 in TS 38.101-1)

| NR Operating Band | Downlink (DL) operating band (FDL_low – FDL_high ) |  |  | Duplex Mode | Total bandwdith |
| --- | --- | --- | --- | --- | --- |
| n1 | 2110 MHz | – | 2170 MHz | FDD | 60 |
| n2 | 1930 MHz | – | 1990 MHz | FDD | 60 |
| n3 | 1805 MHz | – | 1880 MHz | FDD | 75 |
| n5 | 869 MHz | – | 894MHz | FDD | 25 |
| n7 | 2620 MHz | – | 2690 MHz | FDD | 70 |
| n8 | 925 MHz | – | 960 MHz | FDD | 35 |
| n20 | 791 MHz | – | 821 MHz | FDD | 30 |
| n28 | 758 MHz | – | 803 MHz | FDD | 45 |
| n38 | 2570 MHz | – | 2620 MHz | TDD | 50 |
| n41 | 2496 MHz | – | 2690 MHz | TDD | 194 |
| n50 | 1432 MHz | – | 1517 MHz | TDD | 85 |
| n51 | 1427 MHz | – | 1432 MHz | TDD | 5 |
| n66 | 2110 MHz | – | 2200 MHz | FDD | 90 |
| n70 | 1995 MHz | – | 2020 MHz | FDD | 25 |
| n71 | 617 MHz | – | 652 MHz | FDD | 35 |
| n74 | 1475 MHz | – | 1518 MHz | FDD | 43 |
| n75 | 1432 MHz | – | 1517 MHz | SDL | 85 |
| n76 | 1427 MHz | – | 1432 MHz | SDL | 5 |
| n78 | 3300 MHz | – | 3800 MHz | TDD | 500 |
| n77 | 3300 MHz | – | 4200 MHz | TDD | 900 |
| n79 | 4400 MHz | – | 5000 MHz | TDD | 600 |

Table 4: Bandwidth of NR operating bands in FR2 (Based on Table 5.2-1 in TS 38.101-2)

| NR Operating Band | Downlink (DL) operating band (FDL_low – FDL_high ) |  |  | Duplex Mode | Total bandwdith |
| --- | --- | --- | --- | --- | --- |
| n257 | 26500 MHz | – | 29500 MHz | TDD | 3000 |
| n258 | 24250 MHz | – | 27500 MHz | TDD | 3250 |
| n260 | 37000 MHz | – | 40000 MHz | TDD | 3000 |

From Table 3 and Table 4, the maximum bandwidths for NR bands operating in FR [0-2.65GHz] is 90 MHz; in FR [2.4-6GHz] is 900 MHz and [24.25-100GHz] is 3.25GHz. Thus, it is reasonable to use {90MHz, 900MHz, 3.25GHz} for the purpose of design of the indication of the sync-rater positions at this moment (Note: Larger bandwidths may be considered when the bands are introduced in the future). Assuming 90MHz, 900MHz, 3.25GHz} are used for the FRs {[0-2.65GHz], [2.4-6GHz], and [24.25-100GHz]}, we derive the maximum numbers of sync-raster positions in a band as follows:

- For a carrier in the FRs [0-2.65GHz]: 90MHz/900kHz*3=300

- For a carrier in the FRs [2.4-6GHz]: 900MHz/1.44MHz/3=208 (Note: Minimum BW is 10MHz for bands n78, n77, n79)

- For a carrier in the FRs [24.25-100GHz]: 3.25GHz/17.28MHz=188

Thus, it is possible to use the remaining values in the parameter PRB grid offset  and the 8 bits RMSI-PDCCH-Config to indicate the all SS Raster positions in each band in the FRs [0-2.65GHz], [2.4-6GHz] and [24.25-100GHz].

Proposal 1: When the reserved value(s) in SSB-subcarrier-offset and RMSI-PDCCH-Config of the detected non cell-defining SSB are used to indicate the next SS Raster that UE should search for cell-defining SSB, the maximum number of next SS Raster positions to be indicated is limited. The limitation can be set based on the maximum SS Raster position among the NR operating bands in the frequency range under consideration;

Proposal 2: Adopt one of the following options as the frequency range for defining the maximum number of next SS Raster positions

- Alt. 1: Based on three frequency ranges: [0-2.65GHz], [2.4-6GHz] and [24.25-100GHz] (Note: there is no NR operating bands within the FR [6GHz 24.25GHz]).

- Alt. 2: Based on two frequency ranges: sub-6GHz and above 6GHz;

- Alt. 3: Not depending on the frequency ranges;

Proposal 3: The maximum number of next SS Raster positions to be indicated should not be smaller than the value 2N-1, where N is the maximum number of SS Raster positions in a band in the adopted frequency range.

- For a sub-6GHz  band: N = 300

- For a above 6GHz band: N = 188

Proposal 4: Assume the maximum number of the next SS Raster positions is 2N-1. The offset for the indication of the next SS Raster can be defined by the pair of the values {PRB grid offset (g), RMSI-PDCCH-Config (m)} as shown in Table 5:

Table 5: Indication of the offset to the next sync-raster position

| PRB grid offset (=g) | RMSI-PDCCH-Config (=m) | Comments |
| --- | --- | --- |
| g=24 for sub-6GHz g=12 for above-6GHz | n=m |  |
| g=25 for sub-6GHz g=13 for above-6GHz | n=m+256 | Used when 128≤N |
| g=26 for sub-6GHz | n=m+512 | Used when 256≤N |
| Offset to the next sync-raster with cell-defining SSB Offset=n2 , if n is an even number –n-12, ifn is an odd number; n≤2N-1 |  |  |

# Conclusion

In this contribution, we discussed how to indicate a non cell-defining SSB without associated RMSI and the next sync raster that UE should search for cell-defining SSB by the use of the reserved value(s) in SSB-subcarrier-offset and the RMSI-PDCCH-Config.

Proposal 1: When the reserved value(s) in SSB-subcarrier-offset and RMSI-PDCCH-Config of the detected non cell-defining SSB are used to indicate the next SS Raster that UE should search for cell-defining SSB, the maximum number of next SS Raster positions to be indicated is limited. The limitation can be set based on the maximum SS Raster position among the NR operating bands in the frequency range under consideration;

Proposal 2: Adopt one of the following options as the frequency range for defining the maximum number of next SS Raster positions

- Alt. 1: Based on three frequency ranges: [0-2.65GHz], [2.4-6GHz] and [24.25-100GHz] (Note: there is no NR operating bands within the FR [6GHz 24.25GHz]).

- Alt. 2: Based on two frequency ranges: sub-6GHz and above 6GHz;

- Alt. 3: Not depending on the frequency ranges;

Proposal 3: The maximum number of next SS Raster positions to be indicated should not be smaller than the value 2N-1, where N is the maximum number of SS Raster positions in a band in the adopted frequency range.

- For a sub-6GHz  band: N = 300

- For a above 6GHz band: N = 188

Proposal 4: Assume the maximum number of the next SS Raster positions is 2N-1. The offset for the indication of the next SS Raster can be defined by the pair of the values {PRB grid offset (g), RMSI-PDCCH-Config (m)} as shown in Table 5:

Table 5: Indication of the offset to the next sync-raster position

| PRB grid offset (g) | RMSI-PDCCH-Config (m) | Comments |
| --- | --- | --- |
| g=24 for sub-6GHz g=12 for above-6GHz | n=m | If m=0, there is no offset. |
| g=25 for sub-6GHz g=13 for above-6GHz | n=m+256 | Used when 256≤2N |
| g=26 for sub-6GHz | n=m+512 | Used when 512≤2N |
| Offset to the next sync-raster with cell-defining SSB Offset=n2 , if n is an even number –n-12, ifn is an odd number; n≤2N-1 |  |  |

# References

  - Draft Report of 3GPP TSG RAN WG1 #91 v0.1.0", ETSI

  - Draft Report of 3GPP TSG RAN WG1 #AH_1801 v0.1.0, ETSI

  - RAN1 email discussion: “[91-NR-04] RMSI presence indication”.

  - R1-1721684, “WF on RMSI presence flag”, Qualcomm

  - R4-1714295, “LS on channel raster and SS raster,” RAN4

  - R1-1800228, “On Remaining Issues on NR PBCH,” CATT

  - R1-1801157, “Summary of 7.1.2.1 NR-PBCH,” Ericsson