3GPP TSG RAN WG1 Meeting #92	R1-1801781

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.2.3

Source:	Huawei, HiSilicon

Title:	PDCCH repetition for URLLC

Document for:	Discussion and decision

Introduction

The reliability requirement of PDCCH for URLLC was analyzed in [1]. The BLER target of PDCCH for URLLC should be lower than that in current LTE (1%) considering that one shot transmission needs to be supported for URLLC [2][3].

In general, the following options can be considered to improve PDCCH reliability.

Opt 1: Adopt a compact DCI format

It is simple and efficient to adopt a small DCI payload for URLLC in order to ensure the high PDCCH reliability. The details of DCI format design is presented in [3].

Opt 2: Allocate more time-frequency resources

It is straightforward to improve the PDCCH reliability by allocating more time-frequency resources.

For instance, higher CCE aggregation level can be used for URLLC, and AL=16 was already agreed to be supported. However, larger AL would bring PDCCH blocking issue especially in case of small carrier or BWP bandwidth or large subcarrier spacing. To alleviate the PDCCH blocking issue, we can consider PDCCH repetition in frequency domain or time domain, which will be discussed and evaluated in this contribution.

PDCCH repetition discussion

There are many methods to realize PDCCH repetition. As one alternative, PDCCH could be repeated in frequency domain, which may include PDCCH repeated in one or multiple CORESET(s). When PDCCH is repeated in one or multiple CORESET(s) in one carrier, it should be further study what the benefit is comparing with using high AL.

Also, PDCCH could also be repeated in time domain, especially repetition over multiple symbols in a slot is suitable for URLLC. Although it has been agreed that AL=16 is supported, which will occupy almost 100 RBs. Assuming BW=100RB, and each PDCCH occasions includes 1 OS, if PDCCH of one UE is AL=16, other eMBB and URLLC UEs could not be scheduled in this occasion, also the result may be same in the next PDCCH occasion, which incurs latency to some URLLC UEs and also decrease the UPT of eMBB UEs.

In order to guarantee the PDCCH reliability, AL=16 could be replaced by two PDCCH repetitions with AL=8 or four PDCCH repetitions with AL=4 in time domain, which will cause less blockage to other UE’s transmission. In order to prove this, the following two simulations are performed to evaluate eMBB UPT and URLLC latency and reliability, respectively. The detailed simulation assumptions could be found in the Appendix.

To evaluate impact to eMBB UPT, the following three schemes are considered. The SCS of the eMBB UE is 15kHz and the carrier bandwidth is 20MHz. The slot based scheduling granularity for the eMBB UE is assumed and half-slot based scheduling granularity for the URLLC UE is assumed.

- Scheme1: PDCCH AL=16, no repetition

- Scheme 2: PDCCH AL=8, repeat 2 times in time domain

- Scheme 3:PDCCH AL=4, repeat 4 times in time domain

Figure 1 PDCCH repetition schemes (time domain) for URLLC

The system simulation result is shown in Figure 2 and Table 1, and the gains of (scheme 2 vs scheme 1) and (scheme 3 vs scheme 1) could be seen in Table 1. It could be seen that comparing with AL=16, PDCCH repetition, such as two times of PDCCH repetitions with AL=8 and four times of PDCCH repetitions with AL=4, can provide significant UPT gain for eMBB UEs and the gain is increased with the increasing of repetition times.

Figure 2 Simulation result for eMBB UPT

Table 1 Simulation result for eMBB UPT

| Schemes | eMBB UPT（Mbps） |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | Average |
| Scheme 1 | 0.64641 | 2.3641 | 15.1515 | 4.3374 |
| Scheme 2 | 0.86207 | 3.9139 | 27.3973 | 7.4804 |
| Scheme 3 | 1.001 | 5.3619 | 31.746 | 9.4519 |
| Gain (Scheme 2 vs Scheme 1 ) | 33.36% | 65.56% | 80.82% | 72.46% |
| Gain (Scheme 3 vs Scheme 1 ) | 54.86% | 126.8% | 109.52% | 117.92% |

Observation 1: PDCCH repetition in time domain can alleviate the PDCCH blocking for eMBB UEs and increase the eMBB UPT.

To evaluate impact of PDCCH blocking among URLLC UEs, the above scheme 1 and scheme 2 are considered. The SCS of the URLLC is 60 KHz and the carrier bandwidth is 80MHz. Half-slot based scheduling granularity for the URLLC UE is assumed. The metric is the number of successful UEs to meet the requirement of up to 1e-3 within 1ms.

Table 2 Simulation result for URLLC latency and reliability

|  | 90% successful UE ratio | 99% successful UE ratio | 99.9% successful UE ratio |
| --- | --- | --- | --- |
| Scheme 1 | 99.5% | 76.2% | 6.7% |
| Scheme 2 | 99.5% | 96.7% | 82.9% |

The system simulation result is shown in Table 2. It could be seen from the table that comparing with AL=16, PDCCH repetition, such as two times of PDCCH repetitions with AL=8, more URLLC UEs could meet the latency and reliability requirement. So considering URLLC latency and URLLC reliability, PDCCH repetition in time domain should be supported.

Observation 2: PDCCH repetition in time domain can alleviate PDCCH blocking among URLLC UEs.

Moreover, between two PDCCH repetition occasions spanning non-continuous slots, there could be some prompt feedback, such as A-CSI, PDCCH-ACK/NACK and so on. For example, if PDCCH-ACK is received between two PDCCHs repetition occasions, the gNB can omit the transmission of the second PDCCH and the PDCCH overhead could be reduced. As another example, a fast A-CSI report triggered by the PDCCH can also be viewed as implicit ACK for the PDCCH. Upon receiving the A-CSI at the gNB, the gNB can also stop the PDCCH repetition. From this perspective, PDCCH repetitions over multiple PDCCH monitoring occasions is also beneficial.

Observation 3: A fast feedback scheme for the PDCCH between PDCCH repetitions in time domain can save PDCCH overhead.

Proposal 1: PDCCH repetitions in time domain should be supported and PDCCH repetition in frequency domain can also be considered.

Conclusion

PDCCH repetition in frequency domain or time domain is discussed in this contribution, and evaluations are performed to prove the alleviation of PDCCH blocking impact to eMBB UPT and URLLC latency and reliability. Finally, the following observations and proposals are provided:

Observation 1: PDCCH repetition in time domain can alleviate the PDCCH blocking for eMBB UEs and increase the eMBB UPT.

Observation 2: PDCCH repetition in time domain can alleviate PDCCH blocking among URLLC UEs.

Observation 3: A fast feedback scheme for the PDCCH between PDCCH repetitions in time domain can save PDCCH overhead.

Proposal 1: PDCCH repetitions in time domain should be supported and PDCCH repetition in frequency domain can also be considered.

- References

R1-1611221, “Control channel design for URLLC,” Huawei, HiSilicon, Reno, Nevada, USA, Nov. 13 - 18, 2016.

R1-1611659, “HARQ design for DL URLLC,” Huawei, HiSilicon, Reno, Nevada, USA, Nov. 13 - 18, 2016.

R1-1801355, “Compact DCI design for URLLC”, Huawei, HiSilicon

- Appendix

Table A1 Simulation assumptions

| Parameters | Description |
| --- | --- |
| Deployment scenarios | Homogeneous network (7*3 site) |
| Inter-BS distance | 500 m |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 15kHz and 60kHz |
| Scheduled PDSCH time-domain | 14 symbols |
| Simulation bandwidth | 20 MHz and 80 MHz |
| Channel model | 3D Uma |
| BS Tx power | 46 dBm |
| BS antenna configurations | 2TX |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 8dBi |
| UE antenna configurations | 2RX |
| UE antenna height | Follow the modelling of TR 36.873 |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9 dB |
| Traffic model | URLLC: FTP Model 3 with MAC packet size 32bytes eMBB: FTP Model 3 with APP packet size 0.5Mbytes |
| UE distribution | 20% Outdoor in cars: 30 km/h, 80% Indoor: 3 km/h URLLC: 10 UE/sector eMBB: 10 UE/sector |
| UE receiver | MMSE-IRC |