3GPP TSG RAN WG1 Meeting #92	R1-1801946

# Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	7.1.1.2.1

Source:	Samsung

Title:	Corrections on NR-PBCH

Document for:	Discussion and decision

# Introduction

This contribution discusses the following remaining issue for NR-PBCH:

- Frequency location indication of next cell-defined SS/PBCH block if no RMSI presents

# Frequency Location Indication of Next SS/PBCH Block

Agreements:

- For an SSB on the sync raster, the indication of no associated RMSI is done using reserved value(s) in SSB-subcarrier-offset. If no RMSI present, RMSI-PDCCH-Config is used to signal the next sync raster that UE should search for cell-defining SSB.

In the email discussion [91-NR-04] after RAN1#91, it was agreed that For an SSB on the sync raster, the indication of no associated RMSI is done using reserved value(s) in ssb-subcarrierOffset. If no RMSI present, RMSI-PDCCH-Config is used to indicate the next sync raster that UE should search for cell-defining SS/PBCH block.

Conclusion:

Regarding signaling the valid search raster:

- No additional signaling entries in the PBCH (for example cellBarred, intraFreqReselection) are to be used to inform the UE of the valid raster

- The indication is only for the same frequency band

In RAN1 AH1801 [1], the following conclusion was made, and the remaining details of the indication method are proposed in this contribution.

On straightforward way is using 8 bits RMSI-PDCCH-Config combined with the 4 reserved codepoints in ssb-subcarrierOffset to indicate the GSCN value of the sync raster that UE should search for the cell-defining SS/PBCH block within the searching band. Note that the indication capability of this method is 256*4=1024, which is much larger than the maximum number of sync rasters for a given band supported in NR, wherein the maximum number specified in RAN4 is 620 (for band n77 as in the latest version of Table 5.4.3.3-1 in TS 38.101 captured in R4-1801318 [2], and note that the number of sync rasters for FR2 is much smaller than 620, although not listed in Table 5.4.3.3-1). Hence, at least for now, 620 out of the 1024 codepoints are sufficient to indicate the GSCN values for a given NR band, and the remaining codepoints can be reserved for the future extension, if a new band with a larger number of sync rasters is introduced to NR later. Meanwhile, another separate codepoint should be utilized to indicate that there is no such cell-defined SS/PBCH block within the searching band.

A TP for TS 38.213 summarizes our proposal as follow:

Proposal: Adopt the following TP on cell search procedure in session 4.1 of TS 38.213

======================== Begin of Text Proposal for Section 4.1 in TS38.213 =======================

-------------------------------------------------------- Unchanged Text Omitted --------------------------------------------------------

In initial cell selection, a UE shall assume no associated RMSI presents if the higher layer parameter ssb-subcarrierOffset takes the value from {RMSI-notPresent0, RMSI-notPresent1, RMSI-notPresent2, RMSI-notPresent3}, and the UE shall assume the mapping of ssb-subcarrierOffset and RMSI-PDCCH-Config to the GSCN of the synchronization raster that the UE should search for cell-defining SS/PBCH block within the searching band is according to Table 4.1-1, where GSCN-first and GSCN-step-size are the first and step size of GSCN of synchronization raster for the searching band, respectively [8, TS 38.101].

Table 4.1-1: Mapping of ssb-subcarrierOffset and RMSI-PDCCH-Config to the GSCN of the synchronization raster that the UE should search for cell-defining SS/PBCH block within the searching band.

| ssb-subcarrierOffset | RMSI-PDCCH-Config | GSCN of the synchronization raster that the UE should search for cell-defining SS/PBCH block within the searching band |
| --- | --- | --- |
| RMSI-notPresent0 | 0 | No cell-defined SS/PBCH block in the searching band |
| RMSI-notPresent0 | 1 | GSCN-first |
| RMSI-notPresent0 | 2 | GSCN-first + GSCN-step-size |
| … | … | … |
| RMSI-notPresent0 | 255 | GSCN-first + 254*GSCN-step-size |
| RMSI-notPresent1 | 0 | GSCN-first + 255*GSCN-step-size |
| … | … | … |
| RMSI-notPresent1 | 255 | GSCN-first + 510*GSCN-step-size |
| RMSI-notPresent2 | 0 | GSCN-first + 511*GSCN-step-size |
| … | … | … |
| RMSI-notPresent2 | 108 | GSCN-first + 619*GSCN-step-size |
| RMSI-notPresent2 | 109 | Reserved |
| … | … | … |
| RMSI-notPresent2 | 255 | Reserved |
| RMSI-notPresent3 | 0 | Reserved |
| … | … | … |
| RMSI-notPresent3 | 255 | Reserved |

========================== End of Text Proposal for Section 4.1 in TS38.213 =======================

# Conclusion

The proposal made in this contribution is as below:

Proposal: Adopt the following TP on cell search procedure in session 4.1 of TS 38.213

======================== Begin of Text Proposal for Section 4.1 in TS38.213 =======================

-------------------------------------------------------- Unchanged Text Omitted --------------------------------------------------------

In initial cell selection, a UE shall assume no associated RMSI presents if the higher layer parameter ssb-subcarrierOffset takes the value from {RMSI-notPresent0, RMSI-notPresent1, RMSI-notPresent2, RMSI-notPresent3}, and the UE shall assume the mapping of ssb-subcarrierOffset and RMSI-PDCCH-Config to the GSCN of the synchronization raster that the UE should search for cell-defining SS/PBCH block within the searching band is according to Table 4.1-1, where GSCN-first and GSCN-step-size are the first and step size of GSCN of synchronization raster for the searching band, respectively [8, TS 38.101].

Table 4.1-1: Mapping of ssb-subcarrierOffset and RMSI-PDCCH-Config to the GSCN of the synchronization raster that the UE should search for cell-defining SS/PBCH block within the searching band.

| ssb-subcarrierOffset | RMSI-PDCCH-Config | GSCN of the synchronization raster that the UE should search for cell-defining SS/PBCH block within the searching band |
| --- | --- | --- |
| RMSI-notPresent0 | 0 | No cell-defined SS/PBCH block in the searching band |
| RMSI-notPresent0 | 1 | GSCN-first |
| RMSI-notPresent0 | 2 | GSCN-first + GSCN-step-size |
| … | … | … |
| RMSI-notPresent0 | 255 | GSCN-first + 254*GSCN-step-size |
| RMSI-notPresent1 | 0 | GSCN-first + 255*GSCN-step-size |
| … | … | … |
| RMSI-notPresent1 | 255 | GSCN-first + 510*GSCN-step-size |
| RMSI-notPresent2 | 0 | GSCN-first + 511*GSCN-step-size |
| … | … | … |
| RMSI-notPresent2 | 108 | GSCN-first + 619*GSCN-step-size |
| RMSI-notPresent2 | 109 | Reserved |
| … | … | … |
| RMSI-notPresent2 | 255 | Reserved |
| RMSI-notPresent3 | 0 | Reserved |
| … | … | … |
| RMSI-notPresent3 | 255 | Reserved |

========================== End of Text Proposal for Section 4.1 in TS38.213 =======================

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting AH1801, Vancouver, Canada, January 22nd – 26th, 2018.

[2] Draft CR on synchronization raster, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting AH-1801, San Diego, US, 22 - 26 Jan, 2018

# Appendix

A copy of updated Table 5.4.3.3-1 in TS 38.101 is attached as follows (captured in R4-1801318 [2]).

# Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1)

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 7039 – <1> – 7224 |
| n2 | 15 kHz | Case A | 6439 – <1> – 6624 |
| n3 | 15 kHz | Case A | 6022 – <1> – 6258 |
| n5 | 15 kHz | Case A | 2902 – <1> – 2973 |
|  | 30 kHz | Case B | 2911 – <1> – 2964 |
| n7 | 15 kHz | Case A | [8740 – <1> – 8958] |
| n8 | 15 kHz | Case A | 3091 – <1> – 3192 |
| n20 | 15 kHz | Case A | 2644 – <1> – 2727 |
| n28 | 15 kHz | Case A | 2533 – <1> – 2667 |
| n38 | 15 kHz | Case A | [8572 – <1> – 8958] |
| n41 | 15 kHz | Case A | [9069] – <TBD> – [9199] |
|  | 30 kHz | Case C | 9070 – <1> – 9198 |
| n50 | 15 kHz | Case A | 4780 – <1> – 5049 |
| n51 | 15 kHz | Case A | 4762 – <1> – 4764 |
| n66 | 15 kHz | Case A | 7039 – <1> – [7326] |
|  | 30 kHz | Case B | 7048 – <1> – [7317] |
| n70 | 15 kHz | Case A | 6655 – <1> – [6726] |
| n71 | 15 kHz | Case A | 2062 – <1> – 2166 |
| n74 | 15 kHz | Case A | 4924 – <1> – 5052 |
| n75 | 15 kHz | Case A | [4780 – <1> – 5049] |
| n76 | 15 kHz | Case A | [4762 – <1> – 4764] |
| n77 | 30 kHz | Case C | 9628 – <1> – 10247 |
| n78 | 30 kHz | Case C | 9628 – <1> – 9969 |
| n79 | 30 kHz | Case C | [10393] – <TBD> – [10802] |
| NOTE 1: SS Block pattern is defined in section 4.1 in [TS 38.213]. |  |  |  |