3GPP TSG RAN WG1 Meeting #92                          R1-1802187

Athens, Greece, Feb. 26nd – Mar. 2th, 2018

Agenda Item:	7.1.1.5.2

Source: 	LG Electronics

Title: 	Remaining details on remaining issues for RLM in NR

Document for:	Discussion and decision

- Introduction

In last meeting, the agreements were made for the most of issues related to RLM [1], and there are some issues remaining to be discussed. In this contribution, we discuss about the relationship between RLM and beam management (e.g. aperiodic IS/OOS) among remaining issues in order to finalize RLM operation in NR system.

- Relation with beam failure recovery

In NR system, the resource of RLM-RS and RS for beam management are separately configured while the RLM-RS resources can be partially overlapped with beam management RS resource set. In case the configured RS set for new candidate beams does not fully include the configured RLM-RS set, in-sync might be indicated to higher layer for RLM status even if new candidate beam(s) is/are not detected for beam failure recovery after beam failure event. Therefore, in order to reduce the redundant measurement operation by avoiding the mis-aligned status between RLM and BM, and re-establish the radio link to the suitable beam or cell as fast as possible, the failure of beam recovery procedure needs to be indicated to higher layer, at least for the case that the new candidate beams are not detected. Based on the discussion, it is proposed that the following indication scheme is defined for radio link monitoring in NR system.

- Radio link problem is indicated to higher layer when the beam recovery procedure is not successfully completed (e.g. new beam is not detected, the channel quality of the new best beam is worse than the pre-defined or pre-configured threshold after beam failure or network does not respond to beam recovery operation). The condition is used in only multi-beam scenario, and RRC layer might need to re-establish RRC connection as soon as the event is indicated by Layer 1 or 2 (detail procedure will be discussed by RAN2)

Proposal 1. Additional condition for indication of radio link problem to RRC layer is defined : When the beam recovery procedure is failed (e.g. new beam is not detected, the channel quality of the new best beam is worse than the pre-defined or pre-configured threshold after beam failure or network does not respond to beam recovery operation).

- Conclusion

In this contribution, we discuss about RLM operation and provide the following proposals.

Proposal 1. Additional condition for indication of radio link problem to RRC layer is defined : When the beam recovery procedure is failed (e.g. new beam is not detected, the channel quality of the new best beam is worse than the pre-defined or pre-configured threshold after beam failure or network does not respond to beam recovery operation).

- Reference

RAN1 NR-AH #1801 Chairman Notes