3GPP TSG RAN WG1 Meeting #92    	    R1-1802579

Athens, Greece, February 26th – March 2nd, 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Mode 4 support for V2X carrier aggregation

Agenda item:	6.2.3.1.1

Document for:	Discussion and Decision

# Introduction

One of the objectives of the “V2X phase 2 based on LTE” work item ‎[1] is carrier aggregation:

- Specify solutions for the following PC5 functionalities, which can co-exist in the same resource pools as Rel-14 functionality and use the same scheduling assignment format (which can be decoded by Rel-14 UEs), without causing significant degradation to Rel-14 PC5 operation compared to that of Rel-14 UEs: [RAN1, RAN2, RAN4]

  - Carrier aggregation (up to 8 PC5 carriers);

In this contribution we discuss resource selection and carrier selection to support mode-4 transmission with PC5 carrier aggregation.

We start by listing the relevant existing agreements.

At RAN1 #90bis the following was agreed ‎[2]:

Agreement: Any sensing and resource (re)selection procedure uses the Rel-14 PHY UE procedure of determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink transmission mode 4. Additional rules for resource exclusion of resources is not precluded after the procedure

Note: T2 values may be discussed, and potentially modified, when discussing latency reduction

Conclusion: Continue discussion on whether address the following issue for resource selection for mode-4 CA:

  - UE’s limited TX capability

    - TX chain switching time

  - Half duplex problem

  - TX power budget constraint

At RAN1#91 the following was agreed ‎[3] :

# Discussion

## UEs with Limited TX Capabilities

Among the options listed in the RAN1#91 agreement on UEs with limited TX capabilities, option 2 has the benefit of being very simple and minimizing specification impact; on the other hand, it will result in degraded performance and inability to reap the expected benefits of carrier aggregation. Turning then to option 1-1 and option 1-2, the resulting resource selection seems to be very similar between these two options; the difference seems to mainly be in how to describe the behaviour and which specifications would be impacted. Assuming that the performance would be the same, how to decide between them is a matter of ease of specification. In that respect, option 1-2 seems slightly more straightforward. Option 1-2 can handle all 4 types of TX capability limitation listed in the agreement, so the simplest alternative is to use option 1-2 for all cases.

Proposal 1: For mode-4 resource selection of UEs with limited TX capabilities, select the alternative “Option 1-2 for (a), (b), (c), and (d)”.

One TX capability limitation not listed in the RAN1#91 agreement is the case of limited TX power budget. It seems desirable to treat all TX capability limitations in a consistent way, so we propose

Proposal 2: Add “total TX power exceeds Pcmax” as an additional TX capability limitation to be handled.

Further aspects of TX power sharing are discussed in our companion contribution ‎[4].

# Conclusions

In this contribution, we discussed mode-4 transmission with PC5 carrier aggregation for UEs with limited TX capabilities and make the following proposals:

Proposal 1: For mode-4 resource selection of UEs with limited TX capabilities, select the alternative “Option 1-2 for (a), (b), (c), and (d)”.

Proposal 2: Add “total TX power exceeds Pcmax” as an additional TX capability limitation to be handled.

# References

- RP-171740, WID “V2X phase 2 based on LTE”

- Chairman notes RAN1 #90bis

- Chairman notes RAN1 #91

- R1-1802581, Tx power allocation in SL CA, Nokia, Nokia Shanghai Bell