3GPP TSG RAN WG1 Meeting #92			R1-1802886

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.4.4

Source: 	CATR

Title:	Initial link level simulation results of non-orthogonal multiple-access schemes

Document for:	Discussion and Decision

Introduction

A study item of non-orthogonal multiple access for NR is agreed in RAN#76[1]. The study will furtherprogress on the evaluation of non-orthogonal multiple access schemesfocusing on uplink, and provide recommendation on the non-orthogonal multiple access scheme(s) to be specified later.Agreements, observations and evaluation assumption in Rel-14 study shall be the starting point.Link and system level performance evaluation or analysis for non-orthogonal multiple access will be further present with the new evaluation assumptions.

In this contribution, we present initial link level simulation results of some non-orthogonal multiple accessschemes, taking SCMA[2] and PDMA[3] as example same as our previous work in Rel-14 study [4][5] with the new simulation parameters suggested in the associated email discussions. Observations are given based on the simulation results.

Simulation Setup

Some simulation parameters have been suggested in the associated email discussions. This section further clarifies the parameters setting up.

Table 1 Evaluation parameters – LLS for UL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | CP-OFDM |
| Channel coding | LTE Turbo |
| Numerology (data part) | SCS = 15 kHz #OS = 14 Normal CP |
| System Bandwidth | 10 MHz |
| Total allocated bandwidth for transmission | 4PRB |
| Overhead | 2 DMRS symbols, no SRS, i.e., 144 available RE per RB for data transmission, or equivalent overhead |
| Target spectral efficiency | SE =0.25 |
| TB size without CRC | 120bits |
| MCS | For OFDMA case: QPSK, code rate 3/4 For SCMA case: code rate 1/2 For PDMA case: QPSK 1/2 Same for all UEs |
| Number of UEs multiplexed in the same allocated bandwidth | 6 |
| BS antenna configuration | 2Rx |
| UE antenna configuration | 1Tx |
| Propagation channel & UE velocity | TDL-C 300ns in TR38.901, 3km/h |
| Max number of HARQ transmission | 1 |
| Channel estimation | Ideal Channel Estimation |
| Distribution of avg. SNR | Equal average SNR |
| MA signature allocation | Fixed |
| Receiver algorithm | For OFDMA case: MMSE-IRC For SCMA case: Turbo-MPA (Inneriteration:3, Outer iteration:3) For PDMA case: Turbo-MPA (Inner iteration:3, Outer iteration:3) |

Evaluation results and analysis

In this section, the uplink link-level performance of SCMA and PDMA are evaluated and OFDMA is taken as the baseline. Moreover, the localized resource allocation method referring to [6]is adopted in OFDM. And equal power consumption is assumed fornon-orthogonal multiple access schemes and OFDMAreferring to [7].

The BLER performance comparison between SCMA and OFDMA in uplink link level simulation with TDL-C(DS=300ns) channel model isshown in Figure 1. From this figure, we can see that SCMA could get better performance than OFDMA.

Figure 1 performance comparison between OFDMA and SCMA

The BLER performance comparisons between PDMA and OFDMA in uplink link level simulation with TDL-C(DS=300ns) channel model isshown in Figure 2. From this figure, we can see that PDMA could get better performance than OFDMA.

Figure 2performance comparison between OFDMA and PDMA

The performance gain of SCMA and PDMAboth come from the code domain signal superposition, whichallows superposition of multiple symbols from different users on each resource element (RE),andthe advanced receiver can be adopted to differentiate multiple users on same RE.So based on ourobservation that code domain non orthogonal signal superposition could get performance gain compared with OFDMA, we propose that code domain spreading non orthogonal multiple access scheme should be supported as new multiple access scheme.

Conclusion

According tothe evaluated results, we have observed that code domain spreading non-orthogonal multiple access could get performance gain than OFDMA.We propose code domain spreading non-orthogonal multiple accessto be supported as new multiple access scheme.

References

- RP-171043Revision of Study on 5G Non-orthogonal Multiple Access, ZTE, RAN#76 meeting, 2017.7

- R1-162155Sparse Code Multiple Access (SCMA) for 5G Radio Transmission Huawei, HiSilicon

- R1-163383Candidate Solution for New Multiple Access  CATT

- R1-165287Initial link level simulation results of new multiple access scheme, CATR, RAN1#85 meeting, 2016.5

- R1-167602Link level simulation results of new multiple access scheme, CATR, RAN1#86 meeting, 2016.8

- R1-164037  LLS results for uplink multiple access, Huawei, HiSilicon, RAN1#85 meeting, 2016.5

- R1-166468Remaining issues on evaluation assumption and methodology of MA, CATT, RAN1#86 meeting, 2016.8