3GPP TSG RAN WG1 Meeting #92	                                                                        R1-1803241

Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	7.1.1.1

Source: 	Qualcomm Incorporated

Title: 	Remaining details on synchronization signal design

Document for:	Discussion/Decision

# Introduction

This contribution discusses remaining aspects on signaling power spectral density (PSD) offset between SSB and PDCCH, and OFDM symbol generation.

# Power Spectral Density Offset between SSB and PDCCH

In NR, the sync signals are the only downlink signals that are always-ON and broadcast periodically. UE uses the sync signals for Rx-AGC loops when receiving PDCCH/PDSCH. Thus, UE should know the power spectral density (PSD) offset between SSB and PDCCH/PDSCH. In the absence of such offset signalling, UE assumes the nominal power offset of 0dB and there is uncertainty on the measured per-tone energy, which results in a wrong scaling applied in the Rx-AGC loops and hence either reducing SQNR or reducing headroom for signal power variations.

Assuming the PSD offset between PDCCH and PDSCH is not very large e.g., within the range [-6dB, +4dB], it might be better to specify the power offset with respect to PDCCH since typically PDCCH is expected to come first. Below are possible alternatives on signalling the PSD offset between SSB and PDCCH to UE:

- Alt.1: gNodeB signals PSD offset between SSB and PDCCH in RMSI.

- Alt.2: gNodeB signals PSD offset between SSB and PDCCH in PBCH.

- Alt.3: UE assumes a fixed PSD offset between SSB and PDCCH.

Alt.3 might limit the network deployment flexibility. In fact, it is the best to provide this offset signalling in PBCH (Alt.2). However, since there are very limited number of reserved bits in PBCH, Alt.2 might not be a good candidate neither. From this analysis, signaling the power offset between SSB and PDCCH in RMSI (Alt.1) is a reasonable tradeoff between Alt.2 and Alt.3. The values for power offset between SSB and PDCCH might be signalled in ranges e.g., [-5dB, +5dB], [+5dB, +10dB], [+10dB, +15dB], etc.

Proposal 1: The power spectral density offset between SSS and PDCCH is signalled in RMSI. Furthermore, the power offset could be signalled in ranges. UE assumes power spectral density offset between SSS and RMSI PDCCH is within 6dB.

# Rate matching around SSB

RAN1 #91 made the following agreements:

Agreements: - The SS block resource elements are: - Subcarriers 0 to 239 in all four symbols - Subcarriers 48 to 55 and 183 to 191(according to the numbering in the definition of SS/PBCH block in 38.211) in symbols allocated to PSS and SSS are transmitted with zero power - The REs not used for SS/PBCH block of any data PRB that partially or fully contain SS block resource elements are transmitted with zero power and other physical channels are rate matched around such PRBs - Note: The SS/PBCH block PRB grid offset is applied to the whole SS/PBCH block and the above subcarrier numbering is before the shift

The agreements stated that “The REs not used for SS/PBCH block of any data PRB that partially or fully contain SS block resource elements are transmitted with zero power and other physical channels are rate matched around such PRBs.” However, the current text in Section 4.1 of TS 38.213 expresses the rate-matching behaviour at UE in the RE level. To make it consistent with the agreements, we are proposing the following TP:

Proposal 2: RAN1 adopts the following TP on Section 4.1 of TS 38.213:

------------------------- Start TP for Section 4.1 of TS 38.213 ---------------------

A UE can be configured by higher layer parameter SSB-transmitted-SIB1, indexes of SS/PBCH blocks for which the UE shall not receive other signals or channels in PRBs that overlap with PRBs containing the SS/PBCH blocks. A UE can also be configured per serving cell, by higher layer parameter SSB-transmitted, indexes of SS/PBCH blocks for which the UE shall not receive other signals or channels in PRBs that overlap with PRBs containing the SS/PBCH blocks. A configuration by SSB-transmitted overrides a configuration by SSB-transmitted-SIB1. A UE can be configured per serving cell by higher layer parameter SSB-timing a periodicity of the half frames for reception of SS/PBCH blocks per serving cell. If the UE is not configured a periodicity of the half frames for receptions of SS/PBCH blocks, the UE shall assume a periodicity of a half frame. A UE shall assume that the periodicity is same for all SS/PBCH blocks in the serving cell.

------------------------- Start TP for Section 4.1 of TS 38.213 ---------------------

# OFDM symbol generation

RAN1 201801 AH made the following agreement.

R1-1801205 Summary of 7.1.1 Synchronization signal Ericsson Agreements: - Send an LS to RAN4 regarding the following working assumption. Note that there are also other alternative(s) discussed in RAN1. - RAN1 asks RAN4 would especially appreciate if RAN4 can progress on the frequency offset associated with the value of M. RAN1 aims to make a decision in the early week of the next Feb. meeting. - (Working assumption) For signal generation: - Agree to option 3a (unquantized) - Baseband signal generation remains unchanged - Change upconversion formula for all channels/signals expect PRACH to: - where - Upconversion formula for PRACH remains unchanged - Draft LS to be prepared in R1-1801245, which is approved and final LS in R1-1801279

We first note that the working assumption proposes to modify the upconversion formula of OFDM symbol generation while keeping the baseband signal generation formula unchanged. Although this would be mathematically the same as changing the baseband formula by adding the phase compensation term (e-j2πf0tstart,l+NCP,luTc), in practice the transmitter’s frequency for upconversion is not quite accurate and has some frequency offset – i.e., f0=f0+ϵ0. Therefore, if phase compensation is to be indeed implemented as suggested by this working assumption, it will not be quite equivalent to a modified baseband processing formula. We also note that it is not practically feasible to reset the phase for each symbol at the RF. Hence, we propose to adopt a modified baseband formula that includes the phase compensation term.

Observation 1: RAN1 should adopt a change in the baseband signal generation formula to include the phase compensation term, and leave the upconversion formula unchanged.

In what follows, we compare unquantized and quantized version of OFDM symbol generation by focusing on the compensation methods in baseband.

## Phase Compensation Model at Transmitter and/or Receiver

Figure 1: Phase compensation model at transmitter and receiver where transmitter and receiver assume different center frequencies

Figure 1 shows phase compensation at transmitter and receiver where transmitter and receiver assume different center frequencies, i.e., zero tone locations. We don’t assume any noise and fading in this scenario.

The received signal before the FFT block, y(t), can be written as:

yt=lkak,lejφlej2πk+k0μ- Ngridsize,μNSCRB2∆freft-tstart,lμ-NCP,lμTcw(t-tstart,lμ)ej2πf0-f1te-jθl

yt=lkak,lejφl-θl+2πf0-f1tstart,lμ+NCP,lμTcej2πk+k0μ- Ngridsize,μNSCRB2∆fref+f0-f1t-tstart,lμ-NCP,lμTcw(t-tstart,lμ

Where,

wt=1        0≤t≤(Nuμ+NCP,lμ)Tc0                                otherwise

Now, received signal at the l-th symbol can be expressed as,

ylt+tstart,lμ=kak,lejφl-θl+2πf0-f1tstart,lμ+NCP,lμTc ej2πk+k0μ- Ngridsize,μNSCRB2∆fref+f0-f1t-NCP,lμTc where 0≤t≤Nuμ+NCP,lμTC                  							(1).

## Comparison between Quantized and Unquantized phase compensation methods during initial access

In this section, we use the analysis of [8] and use that to compare quantized and unquantized phase compensation methods.

Let us focus on the following scenario [8],

- Transmitter frequency, f0=kTX∆fref+δ0

- Quantized Transmitter frequency, q0=kTX∆fref

- Receiver frequency, f1=kRX∆fref+δ1

- Quantized Receiver frequency, q1=kRX∆fref

- Initial frequency offset, ε

- Receiver frequency with initial frequency offset, = kRX∆fref+δ1+ε

Similar to the analysis of [8], let us consider the frequency range 0-2.65 GHz, where the synchronization signals are located at [5], fSS=N*900 kHz+M*5 kHz. Following [8], we also assume that the receiver does not use the exact value of fSS for down-conversion. Since the 3 synchronization signal raster points are very close to each other, the receiver uses,

f1=N*900 kHz= q1=kRX∆fref								(2)

In the presence of initial frequency offset, the actual receiver frequency is,

f1=N*900 kHz+ε= q1+ε=kRX∆fref+ε							(3)

The transmitter, on the other hand, uses,

f0=N*900 kHz+M*5 kHz= q0 + δ0=kTX∆fref+δ0, where δ0=M*5 kHz  			(4)

The phase compensation terms for quantized and unquantized methods can be written as [9]:

Unquantized method:

φl=-2πf0tstart,lμ+NCP,lμTc									(5)

θl=-2πq1tstart,lμ+NCP,lμTc									(6)

Quantized method:

φl=-2πq0tstart,lμ+NCP,lμTc									(7)

θl=-2πq1tstart,lμ+NCP,lμTc									(8)

### Received signal with quantized phase compensation

Following the analysis of [8], we, at first, apply (7) and (8) in (1) and investigate the received signal with quantized phase compensation. Note that, we plug receiver frequency plus initial frequency offset f1, instead of the receiver frequency f1, in (1) since we are focusing on initial access.

ylt+tstart,lμ=kak,lejφl-θl+2πf0-f1tstart,lμ+NCP,lμTc ej2πk+k0μ- Ngridsize,μNSCRB2∆fref+f0-f1t-NCP,lμTc

ylt+tstart,lμ=kak,lej2πδ0-εtstart,lμ+NCP,lμTc ej2πδ0-εt-NCP,lμTc  ej2πk+k0μ- Ngridsize,μNSCRB2∆fref+qTX-qRXt-NCP,lμTc (9) where 0≤t≤Nuμ+NCP,lμTC

The above equation suggests that the quantized signal method sees a continuous phase ramp across symbols in time domain due to frequency offset of δ0-ε between transmitted and received signals.

Observation 2: In 0-2.65 GHz with GSCN 1- [8832], where synchronization signal raster points are located at N*900 kHz+M*5 kHz, during initial access based on quantized phase compensation method, received signal experiences a continuous phase ramp across symbols in time domain due to frequency offset of δ0-ε, where ε is the initial frequency offset and δ0 is equal to M * 5 kHz.

Note that, if transmit and receive frequencies were equal and if no phase compensation methods were applied, received signal would have still experienced a continuous phase ramp across symbols in time domain due to initial frequency offset ε. UE could have estimated a portion of this initial frequency offset by looking at different phase ramp values within the PSS symbol. The remaining part of the initial frequency offset could have been compensated by looking at the rotation across PBCH symbols, e.g., using DMRS of PBCH symbols.

The same procedure could be used to estimate δ0-ε with quantized phase compensation method.

The phase compensation terms to decode PDCCH or PDSCH of RMSI do not depend on δ0 in quantized phase compensation methods. Hence, UE does not need to estimate δ0 in quantized phase compensation methods.

Observation 3: In quantized phase compensation method, UE can estimate the frequency offset  δ0-ε through the combination of estimation within OFDM symbol, e.g., PSS, and estimation across two OFDM symbols, e.g., using PBCH DMRS.

Observation 4: The phase compensation terms to decode PDCCH or PDSCH of RMSI do not depend on δ0 in quantized phase compensation methods. Hence, UE does not need to estimate δ0 in quantized phase compensation methods.

### Received signal with unquantized phase compensation

We now apply (5) and (6) in (1) and investigate the received signal with unquantized phase compensation. Note that, similar to the last section, we plug receiver frequency plus initial frequency offset f1, instead of the receiver frequency f1, in (1) since we are focusing on initial access.

ylt+tstart,lμ=kak,lejφl-θl+2πf0-f1tstart,lμ+NCP,lμTc ej2πk+k0μ- Ngridsize,μNSCRB2∆fref+f0-f1t-NCP,lμTc

ylt+tstart,lμ=kak,le-j2πεtstart,lμ+NCP,lμTc ej2πδ0-εt-NCP,lμTc  ej2πk+k0μ- Ngridsize,μNSCRB2∆fref+q0-q1t-NCP,lμTc (10) where 0≤t≤Nuμ+NCP,lμTC

Above equation suggests that UE observes a continuous phase ramp due to frequency offset of δ0-ε within each symbol and a discrete phase jump due to frequency offset of ε across symbols.

Note that, phase compensation at the receiver during RMSI PDCCH and PDSCH decoding depend on δ0 in unquantized phase compensation methods. Hence, UE needs to estimate δ0 in unquantized phase compensation method.

Observation 5: In 0-2.65 GHz with GSCN 1-[8832], where synchronization signal raster points are located at N*900 kHz+M*5 kHz, during initial access based on unquantized phase compensation method, received signal experiences a continuous phase ramp due to frequency offset of δ0-ε within each symbol and a discrete phase discontinuity due to frequency offset of ε at symbol boundaries, where ε is the initial frequency offset and δ0 is equal to M * 5 kHz.

Observation 6: The phase compensation terms to decode PDCCH or PDSCH of RMSI depend on δ0 in unquantized phase compensation methods. Hence, UE needs to estimate δ0 in unquantized phase compensation method.

The authors of [8] claim that the frequency offset of ω≔δ0-ε within each symbol, or at least a portion of it, can be estimated as any regular initial frequency offset, and the frequency offset ε, which leads to a discrete phase discontinuity across symbols, can be estimated using PBCH DMRS symbols.

Let ω=ω+e1=δ0-ε+e1 and ε=ε+e2 be respectively the estimates of ω and ε; and e1and e2 are the associated estimation errors. δ0 can be estimated as δ0=M*5kHz, where

M≔ arg minm∈o,±1ω+ε-m*5kHz= arg minm∈o,±1M*5kHz+e1+e2-m*5kHz.

One can observe the estimation of raster offset δ0 is impaired by the error term e1+e2, and in case e1+e2≥2.5kHz, δ0will be wrongly estimated. In what follows, we examine the estimation errors e1and e2and show the effective error e1+e2 can be sufficiently large to lead to a wrong δ0 estimation.

e1 -- error of frequency offset estimation within a symbol:

The accuracy of frequency offset estimation within a symbol (e.g. PSS) depends on the operating SNR. In other words, SNR determines the granularity of frequency offset that we can estimate (i.e. distinguish). A noiseless estimation can virtually provide infinitely small granularity. However, at lower SNR we lose granularity and accuracy.

To investigate frequency estimation granularity, we can check the sensitivity of correlation (e.g. PSS correlation) to the frequency offset (note: the maximum likelihood (ML) estimator finds a frequency offset hypothesis that gives the highest correlation).

Consider a time domain (TD) signal xn=rnej2πωnNΔfref  ,  for n=0,1,…,N-1, where rn is the base signal (e.g. TD PSS signal) and ej2πωnNΔfref  is the phase ramp due to the frequency offset ω. We would like to investigate the sensitivity of correlation value Cω to frequency offset ω:

Cω≔1Nn=0N-1rnxn*rn2=1Nn=0N-1e-j2πωnNΔfref  =1N*sinπα sinπαN

for α≔ωΔfref. Figure 2 shows 20*log10C for various values of α (assuming N=256).

Figure 2: sensitivity of frequency offset estimation within a symbol

We observe the correlation metric function is very flat around zero. In fact, in the presence of 2.5kHz frequency offset and assuming subcarrier spacing Δfref=15kHz, α=2.515=0.166 and the correlation metric degrades by only less than 0.42 dB. It shows the insensitivity of correlation to small frequency offsets (in the order of 0.1 to 0.2 of subcarrier spacing). Now it should be clear that at low operating SNR, the magnitude of frequency offset estimation error (e1) can be simply in the order of a couple of kilo Hertz.

Observation 7: the estimation of the frequency offset within a symbol can be prone to an error in the order of a few kHz (e.g. 0.1 to 0.2 of subcarrier spacing).

e2 -- error of frequency offset estimation across PBCH symbols:

We use the same methodology, as above, to evaluate the accuracy of frequency offset estimation across multiple adjacent symbols through investigating the sensitivity of the correlation metric to frequency offset.

To formulate the correlation metric, we assume DMRS signal is already successfully detected (1 out of 8 candidates), and we also use the detected SSS tones, along with DMRS, to improve the estimation accuracy. Therefore, our reference sequence comprises the DMRS tones (spread across 3 PBCH symbols) and SSS tones in the middle symbol. Let d11:60, d21:24 and d31:60 indicate the DMRS sequence in the 1st, 2nd and 3rd symbol respectively, and s1:127 indicate the SSS sequence. The reference sequence r1:271 can be defined as

rk=d1k                       k=1,…,60 d2k-60           k=61,…,84 sk-84            k=85,…,211 d3k-211    k=212,…,271

We discussed initial frequency offset ε results in a phase jump (e-j2πεtstart,lμ+NCP,lμTc) at the beginning of each symbol l. Assuming normal CP, the amount of phase jump over two consecutive symbols can be shown to be equal to e-j2.14πγ, where γ≔εΔfref. With this phase offset, the received (noiseless) reference sequence will be

xk=d1k                                        k=1,…,60 e-j2.14πγd2k-60           k=61,…,84 e-j2.14πγsk-84            k=85,…,211 e-j4.28πγd3k-211    k=212,…,271

Note that for simplicity, we assumed zero phase offset for the first symbol. The correlation metric C'γ can now be defined as

C'γ≔1271k=1271rkxk*rk2=127160+151ej2.14πγ+60ej4.28πγ

Figure 3 shows the sensitivity of this correlation to frequency offset – i.e., 20*log10C'γ versus various values of γ:

Figure 3: sensitivity of frequency offset estimation, across 3 PBCH symbols, using DMRS and SSS

We can observe although the estimation (of the phase offset) across [PBCH] symbols is more sensitive to the frequency offset ε (more accurately to the resulting phase offset γ) – i.e. C'γ function is less flat around 0 than Cω – the accuracy of this estimation is still limited. For example, 1kHz offset (1kHz/15kHz = 0.067) degrades the correlation by only about 0.4dB  at low operating SNR the magnitude of frequency offset estimation error (e2) can be simply in the order of 1 kHz.

Observation 8: the estimation of the phase offset (resulting from the initial frequency offset) across consecutive symbols can be prone to an error in the order of 1 kHz (e.g. up to 0.1 of subcarrier spacing).

Based on the previous two observations, we conclude the raster offset δ0 may not be reliably estimated using the SS block. In the following sub-section, we focus on the probability of miss-detection of  δ0.

#### Probability of miss-detection of M in unquantized phase compensation

We focus on the joint detection of SSB index (i.e. DMRS sequence) and M to investigate the performance of option 3a. That means, we use the algorithm 3 of [8], where detection metric is computed for each SSB raster shift (0 kHz,±5 kHz) and DMRS sequence pair hypothesis. Overall, we take the following approach.

- Detect PSS

- Estimate continuous phase ramp, arising from δ0-ε, within the PSS symbol.

- De-rotate received signal, i.e., PSS and PBCH DMRS, with the estimated value of δ0-ε and perform joint detection of PBCH DMRS sequence and SSB raster shift (M) using the algorithm of [8].

  - For each SSB raster shift and DMRS sequence pair hypothesis, UE computes detection metric

  - Total 24 detection hypothesis pairs are performed.

  - Select the pair with the strongest metric

The simulation is conducted with flat Rayleigh fading channel with 15 kHz SCS for SSB. An initial carrier frequency offset of 5 ppm was injected on top of the unknown 0 kHz or ±5 kHz shifts of the SSB to perform this simulation.

Figure 7: Probability of error for M detection and SSB index detection

Note that, the authors of [8] used a CDL-C channel to generate the SSB detection curve, whereas, we used a flat Rayleigh fading channel in our simulations. We think that this is probably why the SSB index detection error curves are different between our simulations and that of [8].

#### Summary of comparison between unquantized and quantized phase compensation methods

Miss-detection of raster offset will cause performance and complexity issues with RMSI processing for unquantized phase compensation method.

We note that the above argument only focused on a fundamental issue with the accuracy of raster offset estimation, while there are yet other problems with the unquantized phase compensation method including:

- Frequency pulling range across consecutive symbols: as shown in Figure 3, the detection (pulling) range of frequency offset across two adjacent symbols is -Δfref2.14,+Δfref2.14 = -7kHz,+7kHz (for Δfref=15kHz), while the initial frequency offset can be in the range of ±5 ppm, which leads to ±13.25 kHz at 2.65 GHz  ε cannot be immediately estimated via the suggested method, it will require extra processing at the cost of more complexity and higher chance of wrong estimation.

While the unquantized phase compensation method does not provide any specific benefit compared to the quantized method, the latter does not have any of the mentioned performance or complexity issues.

Observation 9: The unquantized phase compensation method is subject to a number of issues:

- UE may not be able to reliably estimate δ0.

  - RMSI processing: processing the PDCCH and PDSCH of RMSI will be more complex and the performance will be degraded, when δ0 is not reliably estimated.

- This method does not have any benefit compared to the quantized phase compensation method, while the latter is not subject to any of the above issues.

We notice that in the quantized phase compensation method, the UE cannot detect the raster offset δ0, using the SS block and before acquiring RMSI. In fact, the UE initially estimates a frequency offset that comprises two components: the raster offset (if any) and its initial frequency offset (of its local oscillator), and cannot differentiate between the two. This may be thought of as a potential issue, that is the UE cannot accurately adjust its oscillator frequency based on the estimated frequency offset since there may be -/+ 5kHz ambiguity. This would imply an inaccurate sampling frequency (assuming the same oscillator is used for synthesizing the sampling frequency) that could eventually result in some unknown timing offset at RMSI processing. However, considering the SS burst set periodicity of 20msec, the time separation between the RMSI and a corresponding SS block is at most 20 msec. At the carrier frequency of 1GHz, 5kHz frequency ambiguity is 5ppm, which in turn corresponds to 0.1 usec timing drift over 20 msec interval. This maximum timing offset is well within the CP (NCP for 30kHz subcarrier spacing is 2.3 usec), and should not cause any issue for RMSI processing.

Observation 10: With quantized phase compensation method, in 0-2.65 GHz band, UE cannot accurately adjust its oscillator frequency based on the estimated frequency offset since there may be -/+ 5kHz ambiguity. At the carrier frequency of 1GHz, however, 5kHz frequency ambiguity is 5ppm, which in turn corresponds to 0.1 usec timing drift over 20 msec interval. This maximum timing offset is well within the CP (NCP for 30kHz subcarrier spacing is 2.3 usec), and should not cause any issue for RMSI processing.

Proposal 3: RAN1 adopts the quantized phase compensation method for OFDM symbol generation.

- Signal generation formula in baseband is changed to

  - slp,μt=λlμk=0Ngridsize,μNSCRB-1ak,l(p,μ)∙ej2π(k+k0μ-Ngridsize,μNSCRB2)∆f(t-NCP,lμTc)

  - where, λlμ=e-j2πpμ∆fref(NCP,lTCTC+tstart,lμ),  ∆fref=15 kHz, and pμ=argminβ∈Z+|f0-β∆fref|

- Up-conversion signal generation formula is unchanged.

# 5. Conclusion

This contribution has made the following observations and proposals:

Observation 1: RAN1 should adopt a change in the baseband signal generation formula to include the phase compensation term, and leave the upconversion formula unchanged.

Observation 2: In 0-2.65 GHz with GSCN 1-[8832], where synchronization signal raster points are located at N*900 kHz+M*5 kHz, during initial access based on quantized phase compensation method, received signal experiences a continuous phase ramp across symbols in time domain due to frequency offset of δ0-ε, where ε is the initial frequency offset and δ0 is equal to M * 5 kHz.

Observation 3: In quantized phase compensation method, UE can estimate the frequency offset  δ0-ε through the combination of estimation within OFDM symbol, e.g., PSS, and estimation across two OFDM symbols, e.g., using PBCH DMRS.

Observation 4: The phase compensation terms to decode PDCCH or PDSCH of RMSI do not depend on δ0 in quantized phase compensation methods. Hence, UE does not need to estimate δ0 in quantized phase compensation methods.

Observation 5: In 0-2.65 GHz with GSCN 1-[8832], where synchronization signal raster points are located at N*900 kHz+M*5 kHz, during initial access based on unquantized phase compensation method, received signal experiences a continuous phase ramp due to frequency offset of δ0-ε within each symbol and a discrete phase discontinuity due to frequency offset of ε at symbol boundaries, where ε is the initial frequency offset and δ0 is equal to M * 5 kHz.

Observation 6: The phase compensation terms to decode PDCCH or PDSCH of RMSI depend on δ0 in unquantized phase compensation methods. Hence, UE needs to estimate δ0 in unquantized phase compensation method.

Observation 7: the estimation of the frequency offset within a symbol can be prone to an error in the order of a few kHz (e.g. 0.1 to 0.2 of subcarrier spacing).

Observation 8: the estimation of the phase offset (resulting from the initial frequency offset) across consecutive symbols can be prone to an error in the order of 1 kHz (e.g. up to 0.1 of subcarrier spacing).

Observation 9: The unquantized phase compensation method is subject to a number of issues:

- DMRS detection: DMRS detection performance will be degraded in the presence of an unknown δ0.

- UE may not be able to reliably estimate δ0.

  - RMSI processing: processing the PDCCH and PDSCH of RMSI will be more complex and the performance will be degraded, when δ0 is not reliably estimated.

  - PBCH processing: PBCH processing will be more complex or the performance will be degraded, when δ0 is not reliably estimated.

- This method does not have any benefit compared to the quantized phase compensation method, while the latter is not subject to any of the above issues.

Observation 10: With quantized phase compensation method, in 0-2.65 GHz band, UE cannot accurately adjust its oscillator frequency based on the estimated frequency offset since there may be -/+ 5kHz ambiguity. At the carrier frequency of 1GHz, however, 5kHz frequency ambiguity is 5ppm, which in turn corresponds to 0.1 usec timing drift over 20 msec interval. This maximum timing offset is well within the CP (NCP for 30kHz subcarrier spacing is 2.3 usec), and should not cause any issue for RMSI processing.

Proposal 1: The power spectral density offset between SSS and PDCCH is signalled in RMSI. Furthermore, the power offset could be signalled in ranges. UE assumes power spectral density offset between SSS and RMSI PDCCH is within 6dB.

Proposal 2: RAN1 adopts the following TP on Section 4.1 of TS 38.213:

------------------------- Start TP for Section 4.1 of TS 38.213 ---------------------

A UE can be configured by higher layer parameter SSB-transmitted-SIB1, indexes of SS/PBCH blocks for which the UE shall not receive other signals or channels in PRBs that overlap with PRBs containing the SS/PBCH blocks. A UE can also be configured per serving cell, by higher layer parameter SSB-transmitted, indexes of SS/PBCH blocks for which the UE shall not receive other signals or channels in PRBs that overlap with PRBs containing the SS/PBCH blocks. A configuration by SSB-transmitted overrides a configuration by SSB-transmitted-SIB1. A UE can be configured per serving cell by higher layer parameter SSB-timing a periodicity of the half frames for reception of SS/PBCH blocks per serving cell. If the UE is not configured a periodicity of the half frames for receptions of SS/PBCH blocks, the UE shall assume a periodicity of a half frame. A UE shall assume that the periodicity is same for all SS/PBCH blocks in the serving cell.

------------------------- Start TP for Section 4.1 of TS 38.213 ---------------------

Proposal 3: RAN1 adopts the quantized phase compensation method for OFDM symbol generation.

- Signal generation formula in baseband is changed to

  - slp,μt=λlμk=0Ngridsize,μNSCRB-1ak,l(p,μ)∙ej2π(k+k0μ-Ngridsize,μNSCRB2)∆f(t-NCP,lμTc)

  - where, λlμ=e-j2πpμ∆fref(NCP,lTCTC+tstart,lμ),  ∆fref=15 kHz, and pμ=argminβ∈Z+|f0-β∆fref|

- Up-conversion signal generation formula is unchanged.

# 6. References

[1] R4-1710571, “Discussion on SS block transmission frequency locations”, Huawei, HiSilicon

[2] R1-1720272, “Remaining details on sync signals”, Samsung

[3] R1-1721601, “Correcting NR OFDM Symbol Generation”, Intel, 3GPP TSG RAN WG1 Meeting #91.

[4] R1-18xxxxxx, “NR OFDM Symbol Generation Option Analysis”, Intel, 3GPP TSG RAN WG1 AH#1 meeting.

[5] Section 5.4.3.1, 38.104.

[6] Final Chairman’s notes, 3GPP TSG RAN WG1 Meeting 91

[7] R1-1711839 “LS on subcarrier alignment and channel raster” 3GPP TSG RAN WG1 NR Ad-Hoc#2

[8] R1-1802381, “Remaining Details of SS/PBCH Block”, Intel, 3GPP TSG RAN WG1 92 meeting.

[9] R1-18xxxxx, “Remaining details on synchronization signal design”, Qualcomm Incorporated, 3GPP TSG-RAN WG1 Meeting #91AH-01