3GPP TSG RAN WG1 Meeting #92	R1-1803182

Athens, Greece, February 26 – March 2, 2018

Source:	Huawei

Title:	List of RAN1 agreements of Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE

Agenda Item:	6.2.4

Document for:	Information

# List in the chronological order

## RAN1#88bis:

R1-1706842 LS regarding 1024QAM Source: RAN1 To: RAN4

R1-1706493	Skeleton of TR 36.783 V0.0.1 Introduction of 1024QAM in LTE downlink	Qualcomm, Huawei

Decision:Endorsed

Agreement:

| Channel model | AWGN, TDL with delay spread of {10, 100}ns |
| --- | --- |
| Doppler | 5Hz |
| Bandwidth | 20 MHz |
| Tx EVM | Between 0-[3]% |
| Rx EVM | Between 0-[3]% |
| Number of tx/rx antennas | 2T2R, 2T4R, 2T8R, 8T8R (optional) |
| Transmission modes | TM3 for open loop TM4 for closed loop TM9/10 |
| Format of reported results | - Crossover SNR between 256QAM and 1024QAM - Throughput gain at [30]dB and [35]dB SNR |
| Modulation mapping | Gray mapping (described in R1-1705007) |
| Link adaptation scheme | AMC (companies to provide details on the selected scheme) |
| Channel estimation | Realistic |
| Antenna correlation (Tx and Rx) | Uncorrelated |

Agreement:

- Define a new DMRS table or introduce new entries in existing DMRS table

  - DCI payload size is the same as legacy

- Additional DMRS overhead reduction scheme for rank 3/4 transmission is FFS

## RAN1#89:

Agreements: Observations based on the results submitted for RAN1#89:

- With increased number of receiving antennas, the crossover SNR is decreased.

- With increased TX/RX EVM, the crossover SNR is increased.

- The gains provided by 1024QAM are higher in scenarios with LOS component.

- The crossover SNR is lower for scenarios with LOS component.

- For 2T2R under TDL-A/B channel, 1024QAM provides performance gain of 3%~10% at 40dB for TX EVM less than or equal to 2%.

- For 2T4R under TDL-A/B channel, 1024QAM provides performance gain of 10%~22% at 35dB for TX EVM less than or equal to 2%.

- For 2T8R under TDL-B/D channel, 1024QAM starts to provide performance gain at 24~28dB for TX EVM less than or equal to 2%.

- For 2T2R under TDL-D/E channel with correlated LOS, 1024QAM provides performance gain of 0-11% at 30dB SNR for TX EVM less than or equal to 2%

- For 2T4R under TDL-D/E channel with correlated LOS, 1024QAM provides performance gains of 11-16% at 30dB SNR for TX EVM less than or equal to 2%

- For 2T2R under TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gain of 0-19% at 30dB SNR for Tx EVM less than or equal to 2%

- For 2T4R under TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gain of 8-22% at 30dB SNR  for Tx EVM less than or equal to 2%

- For 2T2R and 2T4R TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gains of 16-22% gain at 35dB SNR

- For 2T8R TDL-D channel, 1024QAM provides performance gains of 12.8%~21% at 35dB SNR for TX EVM less than or equal to 2%

- For 2T2R TDL-A, 1024QAM doesn’t provide performance gains over 256QAM when 2 MIMO layer for Tx/Rx EVMs of {3,1.5}%

- For 2T2R TDL-A, 1024QAM doesn’t provide performance gains over 256QAM when 2 MIMO layer for Tx/Rx EVMs of {3,3}%

- For 2T2R TDL-A, 1024QAM, the peak spectral efficiency of 1024QAM has not been observed for {3,1.5}% Tx/Rx

Conclusions:

- Capture the observations in the TR

- Capture in the TR the evaluation results from contributions for RAN1#88bis and #89. Editor to provide an updated TR before RAN1#90.

  - May exclude results that are not aligned with the agreed assumptions.

Agreements:

- New entries in DMRS table to support DMRS density reduction

  - At least including the following entries in DMRS table at least for two enabled CWs.

    - 3 layers, ports 7,8,11 (OCC=4)

    - 4 layers, ports 7,8,11,13 (OCC=4)

    - FFS: also for one enabled CW case

    - This applied to both TM9 and 10

    - FFS: new DMRS table or modification based on legacy table

  - FFS: introducing n_scid for MU-MIMO

- FFS: Additional DMRS overhead reduction scheme for rank 3/4 transmission

## After RAN1#89:

R1-1709858	TP on Summary of evaluation for DL 1024 QAM	Huawei, HiSilicon	(revision of R1-1709411	TP on Summary of evaluation for DL 1024 QAM	Huawei, HiSilicon, Qualcomm)

## RAN1#90:

R1-1712798	Updated TR for 1024QAM	Qualcomm Incorporated

Decision: The TR is endorsed.

Agreements:

- Followings are conclusions of TR

  - RAN1 has observed different degrees of performance improvement due to support of 1024QAM in some scenarios based on link level evaluations.

  - RAN1 has not conducted system level simulation and thus did not confirm the benefits of 1024QAM on system level.

  - RAN1 has concluded that 1024QAM is beneficial to achieve higher peak data rates than 256QAM and recommends to specify 1024QAM at least for some types of UEs (e.g. CPE)

Agreements:

- Introduce new entries, i.e., 3/4-layer(port 7, 8 and 11 for 3 layers, port 7,8,11 and 13 for 4 layers) OCC=4 for two enable CWs, to existing 4-bit DMRS table

  - FFS: Support OCC4 for rank 3 and 4 in one enabled CW case

## After RAN1#90:

The agreements made in email discussion after RAN1#90:

R1-1714994	TR 36.783 v1.0.0 E-UTRA; Introduction of 1024QAM in LTE downlink (Release 15)	Qualcomm

## RAN1#90bis:

R1-1719217	LS on HCS	Source: RAN1, To: RAN2, RAN4

Agreement:

1024QAM supports peak data rates of at least 120Mbps per layer per 20 MHz CC

Agreements:

- UE capability for support of 1024QAM is reported per band/band combination.

- Introduce new DL UE categories based on a subset of LTE DL Categories 11~20

  - FFS on which DL category/categories.

  - Note: other new UE DL category/categories are not precluded.

- Joint RRC configuration of CQI/MCS table to support 1024QAM is supported for UE.

  - Per CC and per CSI subframe set if configured

  - FFS: Per codeword in addition

Agreements:

- Adopt the following modulation definition for 1024QAM at least for initial transmissions:
x=16821-2bi16-1-2bi+28-1-2bi+44-1-2bi+62-1-2bi+8

+ j16821-2bi+116-1-2bi+38-1-2bi+54-1-2bi+72-1-2bi+9

- FFS: whether same is used for re-transmission

Agreement:

- Introduce at least 2 new entries in CQI table for 1024 QAM

- For introduction of 1024QAM CQI table:

  - Remove N entries from the 256QAM table.

  - Add N entries for 1024QAM.

- For introduction of 1024QAM MCS table:

  - Remove M entries from the 256QAM table while maintaining (close to) uniformly spaced SE, while keeping the lowest MCS

  - Add M new entries for 1024QAM, with (close to) uniformly spaced SE

    - Including 1 entry to support re-transmission with 1024 QAM

R1-1718978	WF on DMRS density reduction	Huawei, HiSilicon, Qualcomm, China Telecom, Nokia, NSB

- Note: Applicable to PDSCH with C-RNTI

- When the OCC4 is used for rank 3 and 4 transmission, nSCID is fixed to zero

- The OCC4 for rank 3 and 4 is not supported for one enabled CW case

- No additional DMRS overhead reduction scheme for rank 3/4 transmission will be specified in R15

- Note: Applicable to PDSCH with C-RNTI

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |
| --- | --- | --- | --- |
| Value | Message | Value | Message |
| 0 | 1 layer, port 7, nSCID=0 (OCC=2) | 0 | 2 layer, port 7-8, nSCID=0 (OCC=2) |
| 1 | 1 layer, port 7, nSCID=1 (OCC=2) | 1 | 2 layer, port 7-8, nSCID=1 (OCC=2) |
| 2 | 1 layer, port 8, nSCID=0 (OCC=2) | 2 | 2 layer, port 7-8, nSCID=0 (OCC=4) |
| 3 | 1 layer, port 8, nSCID=1 (OCC=2) | 3 | 2 layer, port 7-8, nSCID=1 (OCC=4) |
| 4 | 1 layer, port 7, nSCID=0 (OCC=4) | 4 | 2 layer, port 11,13, nSCID=0 (OCC=4) |
| 5 | 1 layer, port 7, nSCID=1 (OCC=4) | 5 | 2 layer, port 11,13, nSCID=1 (OCC=4) |
| 6 | 1 layer, port 8, nSCID=0 (OCC=4) | 6 | 3 layer, port 7-9 |
| 7 | 1 layer, port 8, nSCID=1 (OCC=4) | 7 | 4 layer, port 7-10 |
| 8 | 1 layer, port 11, nSCID=0 (OCC=4) | 8 | 5 layer, port 7-11 |
| 9 | 1 layer, port 11, nSCID=1 (OCC=4) | 9 | 6 layer, port 7-12 |
| 10 | 1 layer, port 13, nSCID=0 (OCC=4) | 10 | 7 layers, ports 7-13 |
| 11 | 1 layer, port 13, nSCID=1 (OCC=4) | 11 | 8 layers, ports 7-14 |
| 12 | 2 layers, ports 7-8 | 12 | 3 layers, ports 7, 8,11 (OCC=4) |
| 13 | 3 layers, ports 7-9 | 13 | 4 layers, ports 7, 8,11,13 (OCC=4) |
| 14 | 4 layers, ports 7-10 | 14 | Reserved |
| 15 | Reserved | 15 | Reserved |

## RAN1#91:

Agreement: All code blocks in any newly defined TBS have the same size and zero filler bits

Agreement: The target peak data rate is 1 Gbps for a UE with 4 layers per component carrier and two component carriers.

- Note: This target is only for determining the maximum TBS size and does not have any implications on the definition of UE categories.

Agreement: The largest TBS size for a single layer and for two layers are chosen to be able to meet the target peak data rate.

- FFS: Whether the maximum code rate of 0.931 has to be revisited

Agreement: The largest TBS for a single layer is 125808 and for two layers is 251640.

Agreement:  One RRC configuration of CQI/MCS table is used for 1024QAM for both codewords.

Agreement: Introduce two maximum  entries for 1024QAM, with an RRC parameter selecting between both.

Agreement: The same modulation (including constellation mapping) definition is used for initial transmission and retransmissions.

Agreement: Order CQI and MCS indices according to spectral efficiency.

## After RAN1#91:

Agreements made in email discussion after RAN1#91:

R1-1721731	LS on RRC parameters for high capacity stationary wireless link and introduction of 1024 QAM for LTE	RAN1, Huawei

Agreements:

- 5 new MCS entries are introduced (4 + 1 for retransmission)

- The removed set of entries from the MCS table are approximately every other entry starting from MCS 5 or 6

- Two new entries are added in the CQI table, as follows:

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

- The removed CQI entries are entries 5 and 7 from 256QAM table.

- The new TBS entries for 100 PRBs, 1 layer are {105528, 110136, 115040, 119816/125808}

- For each of the maximum TBS entries above, the complete TBS table is agreed as in the attached .xls (also copied below)

| NRB | 105528 | 110136 | 115040 | 119816 | 125808 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1064 | 1096 | 1160 | 1192 | 1224 |
| 2 | 2088 | 2216 | 2280 | 2408 | 2472 |
| 3 | 3112 | 3240 | 3496 | 3624 | 3752 |
| 4 | 4264 | 4392 | 4584 | 4776 | 4968 |
| 5 | 5352 | 5544 | 5736 | 5992 | 6200 |
| 6 | 6456 | 6456 | 6968 | 7224 | 7480 |
| 7 | 7480 | 7736 | 7992 | 8504 | 8760 |
| 8 | 8504 | 8760 | 9144 | 9528 | 9912 |
| 9 | 9528 | 9912 | 10296 | 10680 | 11064 |
| 10 | 10680 | 11064 | 11448 | 11832 | 12384 |
| 11 | 11448 | 12216 | 12576 | 12960 | 13536 |
| 12 | 12576 | 12960 | 13536 | 14112 | 14688 |
| 13 | 13536 | 14112 | 14688 | 15840 | 15840 |
| 14 | 14688 | 15264 | 15840 | 16992 | 17568 |
| 15 | 15840 | 16416 | 16992 | 18336 | 18336 |
| 16 | 16992 | 17568 | 18336 | 19080 | 19848 |
| 17 | 17568 | 18336 | 19848 | 20616 | 21384 |
| 18 | 19080 | 19848 | 20616 | 21384 | 22152 |
| 19 | 19848 | 20616 | 22152 | 22920 | 23688 |
| 20 | 21384 | 22152 | 22920 | 23688 | 24496 |
| 21 | 22152 | 22920 | 24496 | 25456 | 26416 |
| 22 | 22920 | 24496 | 25456 | 26416 | 27376 |
| 23 | 24496 | 25456 | 26416 | 27376 | 28336 |
| 24 | 25456 | 26416 | 27376 | 28336 | 29296 |
| 25 | 26416 | 27376 | 28336 | 30576 | 30576 |
| 26 | 27376 | 28336 | 29296 | 31704 | 31704 |
| 27 | 28336 | 29296 | 30576 | 32856 | 32856 |
| 28 | 29296 | 30576 | 31704 | 34008 | 35160 |
| 29 | 30576 | 31704 | 32856 | 35160 | 35160 |
| 30 | 31704 | 32856 | 34008 | 36696 | 36696 |
| 31 | 32856 | 34008 | 35160 | 36696 | 37888 |
| 32 | 34008 | 35160 | 36696 | 37888 | 39232 |
| 33 | 35160 | 36696 | 37888 | 39232 | 40576 |
| 34 | 35160 | 37888 | 39232 | 40576 | 42368 |
| 35 | 36696 | 37888 | 40576 | 42368 | 43816 |
| 36 | 37888 | 39232 | 40576 | 43816 | 43816 |
| 37 | 39232 | 40576 | 42368 | 43816 | 45352 |
| 38 | 40576 | 42368 | 43816 | 45352 | 46888 |
| 39 | 40576 | 42368 | 45352 | 46888 | 48936 |
| 40 | 42368 | 43816 | 45352 | 48936 | 48936 |
| 41 | 43816 | 45352 | 46888 | 48936 | 51024 |
| 42 | 43816 | 46888 | 48936 | 51024 | 52752 |
| 43 | 45352 | 46888 | 48936 | 51024 | 52752 |
| 44 | 46888 | 48936 | 51024 | 52752 | 55056 |
| 45 | 46888 | 48936 | 51024 | 55056 | 55056 |
| 46 | 48936 | 51024 | 52752 | 55056 | 57336 |
| 47 | 48936 | 51024 | 55056 | 57336 | 57336 |
| 48 | 51024 | 52752 | 55056 | 57336 | 59256 |
| 49 | 51024 | 52752 | 57336 | 59256 | 61664 |
| 50 | 52752 | 55056 | 57336 | 59256 | 61664 |
| 51 | 52752 | 55056 | 59256 | 61664 | 63776 |
| 52 | 55056 | 57336 | 59256 | 61664 | 63776 |
| 53 | 55056 | 57336 | 61664 | 63776 | 66592 |
| 54 | 57336 | 59256 | 61664 | 63776 | 66592 |
| 55 | 57336 | 59256 | 63776 | 66592 | 68808 |
| 56 | 59256 | 61664 | 63776 | 66592 | 68808 |
| 57 | 59256 | 61664 | 66592 | 68808 | 71112 |
| 58 | 61664 | 63776 | 66592 | 68808 | 71112 |
| 59 | 61664 | 63776 | 68808 | 71112 | 73712 |
| 60 | 63776 | 66592 | 68808 | 71112 | 75376 |
| 61 | 63776 | 66592 | 71112 | 73712 | 76208 |
| 62 | 66592 | 68808 | 71112 | 73712 | 76208 |
| 63 | 66592 | 68808 | 73712 | 75376 | 78704 |
| 64 | 66592 | 71112 | 73712 | 76208 | 78704 |
| 65 | 68808 | 71112 | 75376 | 78704 | 81176 |
| 66 | 68808 | 73712 | 76208 | 78704 | 81176 |
| 67 | 71112 | 73712 | 76208 | 81176 | 81176 |
| 68 | 71112 | 75376 | 78704 | 81176 | 84760 |
| 69 | 73712 | 76208 | 78704 | 81176 | 84760 |
| 70 | 73712 | 76208 | 81176 | 84760 | 87936 |
| 71 | 75376 | 78704 | 81176 | 84760 | 87936 |
| 72 | 76208 | 78704 | 81176 | 84760 | 87936 |
| 73 | 76208 | 81176 | 84760 | 87936 | 90816 |
| 74 | 78704 | 81176 | 84760 | 87936 | 90816 |
| 75 | 78704 | 81176 | 84760 | 90816 | 93800 |
| 76 | 81176 | 84760 | 87936 | 90816 | 93800 |
| 77 | 81176 | 84760 | 87936 | 90816 | 93800 |
| 78 | 81176 | 84760 | 90816 | 93800 | 97896 |
| 79 | 84760 | 87936 | 90816 | 93800 | 97896 |
| 80 | 84760 | 87936 | 90816 | 97896 | 97896 |
| 81 | 84760 | 87936 | 93800 | 97896 | 101840 |
| 82 | 87936 | 90816 | 93800 | 97896 | 101840 |
| 83 | 87936 | 90816 | 93800 | 97896 | 101840 |
| 84 | 87936 | 93800 | 97896 | 101840 | 105528 |
| 85 | 90816 | 93800 | 97896 | 101840 | 105528 |
| 86 | 90816 | 93800 | 97896 | 101840 | 107832 |
| 87 | 90816 | 93800 | 101840 | 105528 | 107832 |
| 88 | 93800 | 97896 | 101840 | 105528 | 110136 |
| 89 | 93800 | 97896 | 101840 | 105528 | 110136 |
| 90 | 93800 | 97896 | 101840 | 107832 | 112608 |
| 91 | 93800 | 97896 | 105528 | 110136 | 112608 |
| 92 | 97896 | 101840 | 105528 | 110136 | 115040 |
| 93 | 97896 | 101840 | 107832 | 112608 | 115040 |
| 94 | 97896 | 101840 | 107832 | 112608 | 115040 |
| 95 | 101840 | 105528 | 110136 | 115040 | 117256 |
| 96 | 101840 | 105528 | 110136 | 115040 | 119816 |
| 97 | 101840 | 105528 | 112608 | 117256 | 119816 |
| 98 | 101840 | 107832 | 112608 | 117256 | 119816 |
| 99 | 105528 | 110136 | 115040 | 119816 | 124464 |
| 100 | 105528 | 110136 | 115040 | 119816 | 125808 |
| 101 | 105528 | 110136 | 117256 | 119816 | 125808 |
| 102 | 107832 | 110136 | 117256 | 124464 | 125808 |
| 103 | 107832 | 112608 | 117256 | 124464 | 128496 |
| 104 | 110136 | 115040 | 119816 | 124464 | 128496 |
| 105 | 110136 | 115040 | 119816 | 125808 | 128496 |
| 106 | 112608 | 115040 | 119816 | 125808 | 133208 |
| 107 | 112608 | 117256 | 124464 | 128496 | 133208 |
| 108 | 115040 | 119816 | 124464 | 128496 | 133208 |
| 109 | 115040 | 119816 | 125808 | 130392 | 133208 |
| 110 | 115040 | 119816 | 125808 | 130392 | 137792 |

NOTE: The entry corresponding to 6PRB, 110136 was modified in a later agreement

## RAN1#92:

Agreement:

The following tables are used to map L1 TBS to L2/L3/L4

| TBS_L1 | TBS_L2 |
| --- | --- |
| 107832 | 214176 |
| 110136 | 220296 |
| 112608 | 226416 |
| 115040 | 230104 |
| 117256 | 236160 |
| 119816 | 239656 |
| 124464 | 248272 |
| 125808 | 251640 |

| TBS_L1 | LBS_L3 |
| --- | --- |
| 105528 | 314888 |
| 107832 | 324336 |
| 110136 | 324336 |
| 112608 | 336576 |
| 115040 | 339112 |
| 117256 | 351224 |
| 119816 | 363336 |
| 124464 | 373296 |
| 125808 | 375448 |

| TBS_L1 | LBS_L4 |
| --- | --- |
| 105528 | 422232 |
| 107832 | 422232 |
| 110136 | 440616 |
| 112608 | 452832 |
| 115040 | 460232 |
| 117256 | 471192 |
| 119816 | 478400 |
| 124464 | 501792 |
| 125808 | 502624 |

Agreement:

The removed entries from 256QAM table are {5, 7, 9, 12, 14}.

Agreement:

Limit use of 1024QAM MCS table only to PDSCH transmission with CRC scrambled by C-RNTI or SPS-C-RNTI for all DCI Formats except DCI formats 1A and 1C.

Agreement:

The TBS table is corrected as follows:

| NRB | 105528 | 110136 | 115040 | 119816 | 125808 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1064 | 1096 | 1160 | 1192 | 1224 |
| 2 | 2088 | 2216 | 2280 | 2408 | 2472 |
| 3 | 3112 | 3240 | 3496 | 3624 | 3752 |
| 4 | 4264 | 4392 | 4584 | 4776 | 4968 |
| 5 | 5352 | 5544 | 5736 | 5992 | 6200 |
| 6 | 6456 | 67126456 | 6968 | 7224 | 7480 |

# List by topics

## 1024QAM

### Evaluation assumptions and conclusions

Agreement: Channel model; AWGN, TDL with delay spread of {10, 100}ns. Doppler; 5Hz. Bandwidth; 20 MHz. Tx EVM; Between 0-[3]%. Rx EVM; Between 0-[3]%. Number of tx/rx antennas; 2T2R, 2T4R, 2T8R, 8T8R (optional). Transmission modes; TM3 for open loop TM4 for closed loop TM9/10. Format of reported results; - Crossover SNR between 256QAM and 1024QAM - Throughput gain at [30]dB and [35]dB SNR. Modulation mapping; Gray mapping (described in R1-1705007). Link adaptation scheme; AMC (companies to provide details on the selected scheme). Channel estimation; Realistic. Antenna correlation (Tx and Rx); Uncorrelated.

Agreements: Observations based on the results submitted for RAN1#89: - With increased number of receiving antennas, the crossover SNR is decreased. - With increased TX/RX EVM, the crossover SNR is increased. - The gains provided by 1024QAM are higher in scenarios with LOS component. - The crossover SNR is lower for scenarios with LOS component. - For 2T2R under TDL-A/B channel, 1024QAM provides performance gain of 3%~10% at 40dB for TX EVM less than or equal to 2%. - For 2T4R under TDL-A/B channel, 1024QAM provides performance gain of 10%~22% at 35dB for TX EVM less than or equal to 2%. - For 2T8R under TDL-B/D channel, 1024QAM starts to provide performance gain at 24~28dB for TX EVM less than or equal to 2%. - For 2T2R under TDL-D/E channel with correlated LOS, 1024QAM provides performance gain of 0-11% at 30dB SNR for TX EVM less than or equal to 2% - For 2T4R under TDL-D/E channel with correlated LOS, 1024QAM provides performance gains of 11-16% at 30dB SNR for TX EVM less than or equal to 2% - For 2T2R under TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gain of 0-19% at 30dB SNR for Tx EVM less than or equal to 2% - For 2T4R under TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gain of 8-22% at 30dB SNR for Tx EVM less than or equal to 2% - For 2T2R and 2T4R TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gains of 16-22% gain at 35dB SNR - For 2T8R TDL-D channel, 1024QAM provides performance gains of 12.8%~21% at 35dB SNR for TX EVM less than or equal to 2% - For 2T2R TDL-A, 1024QAM doesn’t provide performance gains over 256QAM when 2 MIMO layer for Tx/Rx EVMs of {3,1.5}% - For 2T2R TDL-A, 1024QAM doesn’t provide performance gains over 256QAM when 2 MIMO layer for Tx/Rx EVMs of {3,3}% - For 2T2R TDL-A, 1024QAM, the peak spectral efficiency of 1024QAM has not been observed for {3,1.5}% Tx/Rx Conclusions: - Capture the observations in the TR - Capture in the TR the evaluation results from contributions for RAN1#88bis and #89. Editor to provide an updated TR before RAN1#90. - May exclude results that are not aligned with the agreed assumptions.

Agreements: - Followings are conclusions of TR - RAN1 has observed different degrees of performance improvement due to support of 1024QAM in some scenarios based on link level evaluations. - RAN1 has not conducted system level simulation and thus did not confirm the benefits of 1024QAM on system level. - RAN1 has concluded that 1024QAM is beneficial to achieve higher peak data rates than 256QAM and recommends to specify 1024QAM at least for some types of UEs (e.g. CPE)

### Modulation

Agreements: - Adopt the following modulation definition for 1024QAM at least for initial transmissions: x=16821-2bi16-1-2bi+28-1-2bi+44-1-2bi+62-1-2bi+8 + j16821-2bi+116-1-2bi+38-1-2bi+54-1-2bi+72-1-2bi+9 - FFS: whether same is used for re-transmission

Agreement: The same modulation (including constellation mapping) definition is used for initial transmission and retransmissions.

### TBS table

Agreement: 1024QAM supports peak data rates of at least 120Mbps per layer per 20 MHz CC

Agreement: All code blocks in any newly defined TBS have the same size and zero filler bits Agreement: The target peak data rate is 1 Gbps for a UE with 4 layers per component carrier and two component carriers. - Note: This target is only for determining the maximum TBS size and does not have any implications on the definition of UE categories. Agreement: The largest TBS size for a single layer and for two layers are chosen to be able to meet the target peak data rate. - FFS: Whether the maximum code rate of 0.931 has to be revisited Agreement: The largest TBS for a single layer is 125808 and for two layers is 251640. Agreement: Introduce two maximum entries for 1024QAM, with an RRC parameter selecting between

Agreements: - The new TBS entries for 100 PRBs, 1 layer are {105528, 110136, 115040, 119816/125808} - For each of the maximum TBS entries above, the complete TBS table is agreed as in the attached .xls (also copied below) NRB; 105528; 110136; 115040; 119816; 125808. 1; 1064; 1096; 1160; 1192; 1224. 2; 2088; 2216; 2280; 2408; 2472. 3; 3112; 3240; 3496; 3624; 3752. 4; 4264; 4392; 4584; 4776; 4968. 5; 5352; 5544; 5736; 5992; 6200. 6; 6456; 6712; 6968; 7224; 7480. 7; 7480; 7736; 7992; 8504; 8760. 8; 8504; 8760; 9144; 9528; 9912. 9; 9528; 9912; 10296; 10680; 11064. 10; 10680; 11064; 11448; 11832; 12384. 11; 11448; 12216; 12576; 12960; 13536. 12; 12576; 12960; 13536; 14112; 14688. 13; 13536; 14112; 14688; 15840; 15840. 14; 14688; 15264; 15840; 16992; 17568. 15; 15840; 16416; 16992; 18336; 18336. 16; 16992; 17568; 18336; 19080; 19848. 17; 17568; 18336; 19848; 20616; 21384. 18; 19080; 19848; 20616; 21384; 22152. 19; 19848; 20616; 22152; 22920; 23688. 20; 21384; 22152; 22920; 23688; 24496. 21; 22152; 22920; 24496; 25456; 26416. 22; 22920; 24496; 25456; 26416; 27376. 23; 24496; 25456; 26416; 27376; 28336. 24; 25456; 26416; 27376; 28336; 29296. 25; 26416; 27376; 28336; 30576; 30576. 26; 27376; 28336; 29296; 31704; 31704. 27; 28336; 29296; 30576; 32856; 32856. 28; 29296; 30576; 31704; 34008; 35160. 29; 30576; 31704; 32856; 35160; 35160. 30; 31704; 32856; 34008; 36696; 36696. 31; 32856; 34008; 35160; 36696; 37888. 32; 34008; 35160; 36696; 37888; 39232. 33; 35160; 36696; 37888; 39232; 40576. 34; 35160; 37888; 39232; 40576; 42368. 35; 36696; 37888; 40576; 42368; 43816. 36; 37888; 39232; 40576; 43816; 43816. 37; 39232; 40576; 42368; 43816; 45352. 38; 40576; 42368; 43816; 45352; 46888. 39; 40576; 42368; 45352; 46888; 48936. 40; 42368; 43816; 45352; 48936; 48936. 41; 43816; 45352; 46888; 48936; 51024. 42; 43816; 46888; 48936; 51024; 52752. 43; 45352; 46888; 48936; 51024; 52752. 44; 46888; 48936; 51024; 52752; 55056. 45; 46888; 48936; 51024; 55056; 55056. 46; 48936; 51024; 52752; 55056; 57336. 47; 48936; 51024; 55056; 57336; 57336. 48; 51024; 52752; 55056; 57336; 59256. 49; 51024; 52752; 57336; 59256; 61664. 50; 52752; 55056; 57336; 59256; 61664. 51; 52752; 55056; 59256; 61664; 63776. 52; 55056; 57336; 59256; 61664; 63776. 53; 55056; 57336; 61664; 63776; 66592. 54; 57336; 59256; 61664; 63776; 66592. 55; 57336; 59256; 63776; 66592; 68808. 56; 59256; 61664; 63776; 66592; 68808. 57; 59256; 61664; 66592; 68808; 71112. 58; 61664; 63776; 66592; 68808; 71112. 59; 61664; 63776; 68808; 71112; 73712. 60; 63776; 66592; 68808; 71112; 75376. 61; 63776; 66592; 71112; 73712; 76208. 62; 66592; 68808; 71112; 73712; 76208. 63; 66592; 68808; 73712; 75376; 78704. 64; 66592; 71112; 73712; 76208; 78704. 65; 68808; 71112; 75376; 78704; 81176. 66; 68808; 73712; 76208; 78704; 81176. 67; 71112; 73712; 76208; 81176; 81176. 68; 71112; 75376; 78704; 81176; 84760. 69; 73712; 76208; 78704; 81176; 84760. 70; 73712; 76208; 81176; 84760; 87936. 71; 75376; 78704; 81176; 84760; 87936. 72; 76208; 78704; 81176; 84760; 87936. 73; 76208; 81176; 84760; 87936; 90816. 74; 78704; 81176; 84760; 87936; 90816. 75; 78704; 81176; 84760; 90816; 93800. 76; 81176; 84760; 87936; 90816; 93800. 77; 81176; 84760; 87936; 90816; 93800. 78; 81176; 84760; 90816; 93800; 97896. 79; 84760; 87936; 90816; 93800; 97896. 80; 84760; 87936; 90816; 97896; 97896. 81; 84760; 87936; 93800; 97896; 101840. 82; 87936; 90816; 93800; 97896; 101840. 83; 87936; 90816; 93800; 97896; 101840. 84; 87936; 93800; 97896; 101840; 105528. 85; 90816; 93800; 97896; 101840; 105528. 86; 90816; 93800; 97896; 101840; 107832. 87; 90816; 93800; 101840; 105528; 107832. 88; 93800; 97896; 101840; 105528; 110136. 89; 93800; 97896; 101840; 105528; 110136. 90; 93800; 97896; 101840; 107832; 112608. 91; 93800; 97896; 105528; 110136; 112608. 92; 97896; 101840; 105528; 110136; 115040. 93; 97896; 101840; 107832; 112608; 115040. 94; 97896; 101840; 107832; 112608; 115040. 95; 101840; 105528; 110136; 115040; 117256. 96; 101840; 105528; 110136; 115040; 119816. 97; 101840; 105528; 112608; 117256; 119816. 98; 101840; 107832; 112608; 117256; 119816. 99; 105528; 110136; 115040; 119816; 124464. 100; 105528; 110136; 115040; 119816; 125808. 101; 105528; 110136; 117256; 119816; 125808. 102; 107832; 110136; 117256; 124464; 125808. 103; 107832; 112608; 117256; 124464; 128496. 104; 110136; 115040; 119816; 124464; 128496. 105; 110136; 115040; 119816; 125808; 128496. 106; 112608; 115040; 119816; 125808; 133208. 107; 112608; 117256; 124464; 128496; 133208. 108; 115040; 119816; 124464; 128496; 133208. 109; 115040; 119816; 125808; 130392; 133208. 110; 115040; 119816; 125808; 130392; 137792.

Agreement: The following tables are used to map L1 TBS to L2/L3/L4 TBS_L1; TBS_L2. 107832; 214176. 110136; 220296. 112608; 226416. 115040; 230104. 117256; 236160. 119816; 239656. 124464; 248272. 125808; 251640. TBS_L1; LBS_L3. 105528; 314888. 107832; 324336. 110136; 324336. 112608; 336576. 115040; 339112. 117256; 351224. 119816; 363336. 124464; 373296. 125808; 375448. TBS_L1; LBS_L4. 105528; 422232. 107832; 422232. 110136; 440616. 112608; 452832. 115040; 460232. 117256; 471192. 119816; 478400. 124464; 501792. 125808; 502624.

### MCS table

Agreement: - For introduction of 1024QAM MCS table: - Remove M entries from the 256QAM table while maintaining (close to) uniformly spaced SE, while keeping the lowest MCS - Add M new entries for 1024QAM, with (close to) uniformly spaced SE - Including 1 entry to support re-transmission with 1024 QAM

Agreement: Order CQI and MCS indices according to spectral efficiency.

Agreements: - 5 new MCS entries are introduced (4 + 1 for retransmission) - The removed set of entries from the MCS table are approximately every other entry starting from MCS 5 or 6

Agreement: The removed entries from 256QAM table are {5, 7, 9, 12, 14}. Agreement: Limit use of 1024QAM MCS table only to PDSCH transmission with CRC scrambled by C-RNTI or SPS-C-RNTI for all DCI Formats except DCI formats 1A and 1C.

### CQI table

Agreement: - Introduce at least 2 new entries in CQI table for 1024 QAM - For introduction of 1024QAM CQI table: - Remove N entries from the 256QAM table. - Add N entries for 1024QAM.

Agreements: - Two new entries are added in the CQI table, as follows: CQI index; modulation; code rate x 1024; efficiency. 14; 1024QAM; 853; 8.3321. 15; 1024QAM; 948; 9.2578. - The removed CQI entries are entries 5 and 7 from 256QAM table.

### Configuration

Agreements: - Joint RRC configuration of CQI/MCS table to support 1024QAM is supported for UE. - Per CC and per CSI subframe set if configured - FFS: Per codeword in addition

Agreement: One RRC configuration of CQI/MCS table is used for 1024QAM for both codewords.

### UE capability

Agreements: - UE capability for support of 1024QAM is reported per band/band combination.

### UE category

Agreements: - Introduce new DL UE categories based on a subset of LTE DL Categories 11~20 - FFS on which DL category/categories. - Note: other new UE DL category/categories are not precluded.

## DMRS overhead reduction

Agreement: - Define a new DMRS table or introduce new entries in existing DMRS table - DCI payload size is the same as legacy - Additional DMRS overhead reduction scheme for rank 3/4 transmission is FFS

Agreements: - New entries in DMRS table to support DMRS density reduction - At least including the following entries in DMRS table at least for two enabled CWs. - 3 layers, ports 7,8,11 (OCC=4) - 4 layers, ports 7,8,11,13 (OCC=4) - FFS: also for one enabled CW case - This applied to both TM9 and 10 - FFS: new DMRS table or modification based on legacy table - FFS: introducing n_scid for MU-MIMO - FFS: Additional DMRS overhead reduction scheme for rank 3/4 transmission

Agreements: - Introduce new entries, i.e., 3/4-layer(port 7, 8 and 11 for 3 layers, port 7,8,11 and 13 for 4 layers) OCC=4 for two enable CWs, to existing 4-bit DMRS table - FFS: Support OCC4 for rank 3 and 4 in one enabled CW case

Agreements: - When the OCC4 is used for rank 3 and 4 transmission, nSCID is fixed to zero - The OCC4 for rank 3 and 4 is not supported for one enabled CW case - No additional DMRS overhead reduction scheme for rank 3/4 transmission will be specified in R15 - Note: Applicable to PDSCH with C-RNTI One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Message; Value; Message. 0; 1 layer, port 7, nSCID=0 (OCC=2); 0; 2 layer, port 7-8, nSCID=0 (OCC=2). 1; 1 layer, port 7, nSCID=1 (OCC=2); 1; 2 layer, port 7-8, nSCID=1 (OCC=2). 2; 1 layer, port 8, nSCID=0 (OCC=2); 2; 2 layer, port 7-8, nSCID=0 (OCC=4). 3; 1 layer, port 8, nSCID=1 (OCC=2); 3; 2 layer, port 7-8, nSCID=1 (OCC=4). 4; 1 layer, port 7, nSCID=0 (OCC=4); 4; 2 layer, port 11,13, nSCID=0 (OCC=4). 5; 1 layer, port 7, nSCID=1 (OCC=4); 5; 2 layer, port 11,13, nSCID=1 (OCC=4). 6; 1 layer, port 8, nSCID=0 (OCC=4); 6; 3 layer, port 7-9. 7; 1 layer, port 8, nSCID=1 (OCC=4); 7; 4 layer, port 7-10. 8; 1 layer, port 11, nSCID=0 (OCC=4); 8; 5 layer, port 7-11. 9; 1 layer, port 11, nSCID=1 (OCC=4); 9; 6 layer, port 7-12. 10; 1 layer, port 13, nSCID=0 (OCC=4); 10; 7 layers, ports 7-13. 11; 1 layer, port 13, nSCID=1 (OCC=4); 11; 8 layers, ports 7-14. 12; 2 layers, ports 7-8; 12; 3 layers, ports 7, 8,11 (OCC=4). 13; 3 layers, ports 7-9; 13; 4 layers, ports 7, 8,11,13 (OCC=4). 14; 4 layers, ports 7-10; 14; Reserved. 15; Reserved; 15; Reserved.