3GPP TSG RAN WG1 Meeting #92bis 	R1-1803918

Sanya, PR China, 16th  March- 20th April 2018

Agenda Item: 7.1.5

Source: CableLabs

Title: Updated Coexistence Evaluation Assumptions

Document for: Discussion and Decision

# Introduction

RAN Plenary #77 approved the NR in Unlicensed Bands SID [1].

The approved objective of this new SID states:

“Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier”

While the Evaluation Methodology [2] covered most of the requirements required to develop simulations supporting the DL LAA LTE SID, it also comprised some parameters which could be subject to frther updates. As an example:

- The VoIP traffic requirements were optional while both Best Effort (FTP) and quasi real time traffic (semi-persistent allocation VoIP traffic) should have been mandatory since they stress the channel occupancy in different ways.

- The LAA LTE eNB total radiated power (EIRP) was set to 18 dBm (outdoor and indoor scenarios) while FCC part 15.407 allows transmission powers up to 30 dBm (5.15-5.25 and 5.725-5.85 GHz) or 24dBm (5.25-5.35 and 5.47-5.725 GHz)

- The total UE EIRP radiated power (across all carriers) was set to 18dBm while FCC allows transmission powers up to 24 dBm for the 5HGz bands.

This paper discusses and proposes some evaluation methodology assumptions referenced to the of the former DL LAA LTE SID [2].

# Evaluation Methodology Assumptions

## Access Assumptions

Table 1 summarizes the system BW per channel assumptions for 802.11ac and NR-U systems. [5] doesn’t comprise specifications for the NR 5 or 6 GHz  bands. The same reference specifies 40MHz as the smallest BW for the  for band n79 (4400-5000MHz), which is the closest one to the 5GHz band. However, the WiFi technology employed for the same band should match the min BW requirements chosen for NR-U.

| Parameter | 5GHz 802.11ac [2] | 5GHz NR Unlicensed | Comments |
| --- | --- | --- | --- |
| System BW per carrier | 20 MHz | 20 MHz (802.11ac/ax) 40 MHz (NR-U, band nX) | Updated from 20 MHz. For band n79 (38.1010-1), defined for 4400-5000MHz, min BW=40MHz |

Table 1. Summary of the NR-U updates concerning the BW per carrier referenced to TR38.889 (DL LAA LTE SID) and based on [5]

Proposal 1: The minimal NR-U channel bandwidth for the 5/6 GHz band is 20 or 40 MHz (FFS).

Proposal 2: The minimal WiFi channel BW for the 5/6 GHz matches the NR-U channel BW requirements. If the WiFi access technology has a different channel BW than NR-U, the simulations methodology will use a pro-rated EIRP referenced to the NR-U channel EIRP.

[2] used lower EIRP values for the eNB and UE than what actually FCC part 15.407 allows for this band. It is expected that the devices operating in this un-licensed band will make use of the max capability (EIRP) provided by FCC. Hence the simulations methodology should use the maximal values which are considered conservative.

| Parameter | 5GHz 802.11ac [2] | 5GHz NR Unlicensed | Comments |
| --- | --- | --- | --- |
| Total max BS (conducted) RF power | 18 dBm (across aggregated carriers) | 30 dBm (for both eNB and APs) | Updated per FCC part 15.407 5.15-5.25GHz: indoor operations - 5.15-5.25 GHz: Conducted RF out: 1W PSD: ≤17dBm/1MHz - 5.25-5.35&5.47-5.725GHz: Conducted RF out: 0.25W PSD: ≤11dBm/1MHz - 5.725-5.850 MHz Conducted power: 1W PSD≤30dBm/0.5MHz |
| Total max UE | 18dBm (across aggregated carriers) | 24 dBm (for both UE and STAs) | Updated per FCC part 15.407 - 5.15-5.25 GHz: Conducted RF out: 0.25W PSD: ≤11dBm/1MHz |

Table 2. Summary of the FCC EIRP requirements [6] for the 5 GHz spectra compared with similar requirements [2].

Proposal 3: The EIRP used by the eNB/gNB/AP is 30dBm.

Proposal 4: The EIRP used by the UE/STA is 24dBm.

3GPP issued channel model specifications for the NR spectra between 0.5 to 100GHz [3]. These specifications reuse and update the former ITU InH, UMa and UMi recommendations for NR applications.

| Parameter | 5GHz 802.11ac [2] | 5GHz NR Unlicensed [3] | Comments |
| --- | --- | --- | --- |
| Distance-dependent path loss | ITU InH | TR 38.901 section 7.2 | Updated accodingly with 3GPP NR channel model (38.901) specifications. |
| Shadowing | ITU InH TR36.814 | TR 38.901 Table 7.4.1-1 (InH-Office) | Updated accodingly with 3GPP NR channel model (38.901) specifications. |

Table 3. Comparative view of the indoor channel model related specs between [2] and [3].

| Parameter | 5GHz Unlicensed (DL LAA LTE SID) | 5GHz NR Unlicensed | Comments |
| --- | --- | --- | --- |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU Umi [referring to Table B.1.2.1-1 in TR36.814] Indoor UE-to-indoor UE: 3GPP TR 36.843 (D2D) (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for break point distance and LOS probability.) | TR38.901 section 7.4 for UMa scenario | Updated accodingly with 3GPP NR channel model (38.901) specifications. |
| Penetration | ITU UMi [referring to Table B.1.2.1-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance | TR 38.901 section 7.4.3.1, table 7.4.3-2, low-loss model | Updated accordingly with O2I requirements of 38.901 |
| Shadowing | ITU InH TR36.814 | TR 38.901 section 7.4.3.1, 7.4.3-2, low-loss model | Updated accordingly with O2I requirements of 38.901 |
| Fast fading channel | ITU UMa according to Table A.1-1 of 36.819 | UMa according with TR38.901, Tables .4.1-1, 7.4.2-1, 7.5-6 | Updated accordingly with TR38.901 UMa specifications |

Table 4. Comparative view of the outdoor channel model related specs between {2] and [3].

Proposal 5: The indoor and outdoor channel assumptions will use the distance dependent path loss, penetration, shadowing and fast fading channel assumptions used by [3].

### Traffic Assumptions

The eMBB traffic is DL centric, employing large packets with a relaxed type of scheduling supported by a Best Effort model. As opposed to this model, the VoIP traffic uses a semi-persistent allocation with tighter latency and jitter requirements being backed by a real time traffic model.

Since both these applications are commonly used, they should be employed simultaneously by an user and the traffic simulations should support both of them.

Proposal 6: The indoor and outdoor traffic models shall support simultaneously FTP and VoIP traffic.

The average inter-arrival rate ) backed by resource utilization (RU) and packet size or the accurate buffer occupation (BO) were not defined by [2]. While this was driven by certain practical considerations (variable PHY buffer due to the different MCS employed by the DL transmissions to different UE located to different distances referenced to the eNB), this also opened the door for larger variations in terms of traffic loading between different sets of simulations [2].

Observation: The traffic simulation methodology should limit the artefacts generrated by the random UE position inside the cell and the inter-arrival time, aiming for more compact Buffer Occupancy data results between different companies.

# Conclusions

The NR-U coexistence assumptions are different than the former DL LAA LTE [2], due to the evolution of the regualatory requirements, the NR characteristics and other factors.

Proposal 1: The minimal NR-U channel bandwidth for the 5/6 GHz band is 20 or 40 MHz (FFS).

Proposal 2: The minimal WiFi channel BW for the 5/6 GHz matches the NR-U channel BW requirements. If the WiFi access technology has a different channel BW than NR-U, the simulations methodology will use a pro-rated EIRP referenced to the NR-U channel EIRP.

Proposal 3: The EIRP used by the eNB/gNB/AP is 30dBm.

Proposal 4: The EIRP used by the UE/STA is 24dBm.

Proposal 5: The indoor and outdoor channel assumptions will use the distance dependent path loss, penetration, shadowing and fast fading channel assumptions used by [3].

Proposal 6: The indoor and outdoor traffic models shall support simultaneously FTP and VoIP traffic.

Observation: The traffic simulation methodology should limit the artefacts generrated by the random UE position inside the cell and the inter-arrival time, aiming for more compact Buffer Occupancy data results between different companies.

# References

- RP-172021, ‘Study on NR-based Access to Unlicensed Spectrum’, Qualcomm, RAN Plenary #77, September 2017

- 3GPPP TR 36.889 v13.0.0, “Study on Licensed-Assisted Access to Unlicensed Spectrum; (Rel 13)”, June 2015

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Rel 14)”, December 2017

- 3GPP TR 36.814, “Further advancement for E-UTRA physical layer aspects (release 9)”, v9.2.0 (2017-03)

- 3GPP TS38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range Standalone (Release 15)”, 2017-12

- FCC part 15.407